Lamprey Orifice Video Monitoring at McNary Dam
Summary report for the fall Chinook salmon run (08/09-10/31)
The adult fall Chinook salmon run at McNary Dam, as reported by USACE, was scheduled for the period from 08/09/10 to 10/31/10. Underwater video monitoring of open lamprey orifices was performed during the fall Chinook salmon run for 5 hours a day (7am to 12pm), five days a week (Sun-Thu) from 08/15/10 to 09/30/10. For data analyses, hourly counts from the fish counting station at the Oregon shore fish ladder of McNary Dam were obtained for all video monitoring periods.
Summary for the Fall Chinook Salmon Run:
· The total number of adult Chinook salmon passing McNary Dam between 08/09/10 and 09/30/10 amounted to 160237, with 66104 (41.3%) counted at the Washington side, and 94133 (58.7%) at the Oregon side fishway.

· Video monitoring of four open orifices in the Oregon fishway was performed between 08/09/10 and 09/30/10 to cover early and peak passage of fall Chinook salmon. Cameras 2 and 4 monitored the Washington side orifices of weirs 336 and 337, respectively, while cameras 3 and 5 monitored the Oregon side orifices of weirs 336 and 337, respectively. During video monitoring hours, 54.1% (50918) of the total fall Chinook salmon run at the OR side passed through.
· Orifice Passage Attempts:  No passage attempts at monitored orifices were recorded for fall Chinook salmon during the entire observation period.
· Orifice Presence: Fall Chinook salmon were not present in the field of view of cameras. 

Fall Chinook salmon were never observed in the vicinity of open lamprey orifices during the video monitoring period from 08/15 to 09/30. Monitoring covered more than 50% of the fall Chinook salmon run through the Oregon shore fishway of McNary Dam.
