LMN tailscreen basic information:
Lower Monumental requested exemption from NMFS screen criteria to allow lamprey and small fry to be passed back to the river. The location of the proposed change is the raceway tailscreen. The screen  material would change from wire mesh to perforated plate.  The width of the slot would more than triple, and length would be one-third greater (3/32” to 10/32”; 1.75 mm to 2.54 m) than with the existing wire mesh tailscreen.  After leaving the raceway, the drain would then take lamprey to the bypass outfall via a conduit.  

I have gathered some ballpark information for you on the conduit back to the outfall pipe. A 30” pipe leaves from the end of the raceway structure, passes thru a knife valve controlling outflow into a 24” pipe.  Flow entering the 30” pipe is ~ 16” deep & 8 fps.  Once past the valve into the 24” pipe, flow is ~ 17” deep and 10 fps.  The 24” pipe has a slope of ~ 3.2%.  By the point where this 24” conduit joins the outfall pipe, water is ~ 12” deep and flowing at ~ 16 fps.
