14AppD002

FPP Change Request Form
Change Request Number & Title: 14AppD002 NWW Updates
Date Submitted: January 10, 2014 
Project: Appendix D
Requester Name, Agency: Ken Fone, CENWW
Location of Change - FPP Project and Section:
Appendix D, Section 5 – NWW Project Operations for Lamprey.  
Proposed Changes:
See draft Appendix B below with edits to NWW sections.

Justification for Change:  
Updated with the most current information.

Comments from others:

1/17/14 NOAA Memo: Change Form:  14AppD002 - NWW Updates.  No objections.

1/22/14 FPOM: Lorz to confer with his lamprey people to review this section.

Record of Final Action:

1/22/14 FPOM: Pending CRITFC review.

5.  NWW PROJECTS

5.1.  Raceway Tailscreens.  
5.1.1. By 2012, lamprey-friendly raceway tailscreens were installed at all of the juvenile fish facilities that collected for transportation (Lower Granite, Little Goose, Lower Monumental and McNary dams) to allow collected lamprey to be returned to the river rather than transported.  The new tailscreen wire mesh diameter is 16.0 mm (0.063”) with an open width/height of 8.6 mm (0.337”), open diagonal dimension of 12.1 mm (0.477”) and overall screen open area of 71.0%.  
5.1.2. Lower Monumental tested a perforated plate tailscreen that can be cleaned with brushes without entangling lamprey.  The plate is 1/4” thick with 0.312”x1.0” slots that are spaced 1/4” apart oriented vertically in a side-staggered pattern.  With consensus of regional partners, the remaining mesh tailscreens at Lower Monumental were replaced with these perforated plate tailscreens in the 2012-13 winter maintenance season.
5.2.  McNary Dam.  
5.2.1.  Unit trash racks are raked during the winter maintenance period prior to January 15 to minimize the potential for lamprey entanglement in built-up debris when river flow increases.  
5.2.2. ESBSs are installed and operating prior to April 16 (two weeks later than other NWW projects) to allow juvenile lamprey passage directly through turbines without bypass collection. 
5.2.3. During lamprey passage season from June 15–September 30, the ladder entrance weirs at SFE and NFE are lowered to sill during nighttime hours (2100-0400) to reduce velocities and encourage lamprey entrance into the ladder.  This operation has been implemented since 2010.  During the 2013-14 winter maintenance period, a prototype structure designed for adult lamprey passage will be installed at SFE 2.  This structure will provide a deep-slot under the entrance weir gate with lower water velocities for lamprey.  Consequently, the SFE2 weir will not need to be lowered to sill at night with the prototype structure in place.
5.2.4. During the 2009-10 winter maintenance period, horizontal slots were cut at the bottom of the stem walls in the upper section of the Oregon shore ladder to allow adult lamprey attachment along a level pathway through the weir.  Plating was also attached on the diffuser gratings near the ladder walls to create a continuous path for lamprey attachment.
5.3. Other Improvements for Adult Lamprey.  
5.3.1. During the 2011-12 winter maintenance period at Ice Harbor and Lower Monumental dams, improvements were made to the upper fish ladder weirs for adult lamprey, including:
5.3.1.a. Horizontal slots were cut into weirs at the floor to allow adult lamprey attachment along a level pathway through the weir.  
5.3.1.b. Ramps were installed in the upper ladder weirs from the ladder floor to the bottom of elevated salmon orifices to assist lamprey in maintaining attachment as they maneuver through these areas.  
5.3.1.c. Plates were installed on diffuser grating adjacent to orifices in the Ice Harbor north ladder to provide attachment surfaces for lamprey in higher-velocity areas.  
5.3.1.d. Similar lamprey orifices and plating were installed in the Little Goose and Lower Granite ladders during the 2012-13 winter maintenance period.   Three of the weirs at Lower Granite, which did not have lamprey orifices cut due to time constraints, will have the orifices cut during the 2013-14 winter maintenance period.   
5.3.2. At all projects in 2011, picketed leads were raised and secured 1.5” off the ladder floor at the count stations to enable adult lamprey passage through a low-velocity passage route under the picketed leads around the adult fish count slot. 
5.3.3. Fallback adult lamprey collected off fish separators and other areas of the juvenile fish facilities at Lower Monumental, Little Goose and Lower Granite are released into the forebay rather than being bypassed back into the tailrace or transported downstream.
Page 1 of 3

