McNary

- Check trash and VBS differentials daily; use as indicator for need to clean.

- Monitor gatewells daily for buildup of debris, clear before 50% surface coverage occurs

- Orifice checks twice per day

- Keep separator, flumes, sample tanks clear of debris.

- Use bubblers in collection channel to avoid debris buildup

- Adjust cycle timing of ESBS cleaning brushes during periods of high debris

- Test dips with trash rake or pulling VBS when differential has increased or if descaling is prevalent; check units with greatest run time first

- Inspection of VBS when cleaned, with condition logged for use in mesh replacement prioritization

- Camera inspections of ESBS screens on a weekly basis

- Routine trash raking

- Use debris spill, facilitate with log bronc or rolling units if needed.

- Send ice blocks down bypass line once/day

Bobby Johnson