# Fish Passage Plan (FPP) Change Form

Change Form # & Title: 	21LGS006 – ASW Spill Rates

Date Submitted:		2 September 2021

Project:			Little Goose Dam

Requester Name, Agency:	Lisa Wright, Corps

Final Action:

FPP Section:

Chapter 8 LGS, section 2.3.2.7. ASW Operating Criteria.

Justification for Change:

FPOM approved adding Lower Granite RSW spill rates vs forebay elevation in Change Form 21LWG006 on 9/9/21 and requested adding the same information for the other Snake projects.

Proposed Change: (see following pages for edits to existing FPP in track changes)

Comments:

Record of Final Action:

- Proposed Change:

2.3.2.7. Adjustable Spillway Weir (ASW).

2.3.2.7.a. Little Goose has one adjustable spillway weir (ASW) in spillbay 1 that provides a surface route for fish passage. The ASW can be operated from the control room and the crest elevation can be adjusted lower or higher to pass more or less water, respectively, according to the flow criteria below. The ASW spill rate is a function of the crest elevation and forebay elevation – as the pool elevation over the crest increases, more water is spilled over the ASW:

| LGS Forebay Elevation (ft) | ASW Elevation 622 ft Spill Rate (kcfs) | ASW Elevation 618 ft Spill Rate (kcfs) |
| --- | --- | --- |
| 633 | 6.8 | 10.9 |
| 633.5 | 7.3 | 11.4 |
| 634 | 7.8 | 12.0 |
| 634.5 | 8.3 | 12.5 |
| 635 | 8.8 | 13.1 |
| 635.5 | 9.3 | 13.7 |
| 636 | 9.8 | 14.3 |
| 636.5 | 10.3 | 14.9 |
| 637 | 10.9 | 15.5 |
| 637.5 | 11.4 | 16.1 |
| 638 | 12.0 | 16.7 |

2.3.2.7.b. High Crest (ASW-Hi):

            - The ASW high crest is at elevation 622 feet msl and spills approximately 7–8 kcfs when the forebay elevation is in the MOP range. High crest spill patterns are in Table LGS-7 (Spring Spill) and Table LGS-8 (30% Spill).

            - Unless flow conditions defined below are met, ASW spill for fish passage will occur with the ASW at high crest (approximately 7-8 kcfs spill).

2.3.2.7.c. Low Crest (ASW-Lo):

            - The ASW low crest is at elevation 618 feet msl and spills approximately 11–12 kcfs when the forebay elevation is in the MOP range. Low crest spill patterns are in Table LGS-7 (Spring Spill) and Table LGS-9 (30%).

            - Change the ASW to low crest to pass more water during high flow (i.e., spring freshet) when both of the following flow criteria are met: 1) day average total project outflow above 85 kcfs, and 2) NWRFC inflow forecast above 85 kcfs for at least the next 3 days. When day average outflow drops below 85 kcfs and is forecasted to stay below 85 kcfs for at least the next three days, change back to high crest.