A new velocity meter was purchased at The Dalles to replace the old method of timing a float down the fishway channels and entrances areas. The resulting data had some evident differences. (Data collection and evaluation by Jim Day)

| Date | Measurement Location | Velocity meter (ft/sec) | Float velocity (ft/sec) | Difference |
| --- | --- | --- | --- | --- |
| 11/4/2021 | Transportation east | 2.46 |  |  |
| 11/4/2021 | Transportation west | 4.66 |  |  |
| 11/4/2021 | Collection east | 1.66 |  |  |
| 11/4/2021 | Collection west | 1.45 |  |  |
| 11/4/2021 | South entrance | 4.01 |  |  |
| 11/22/2021 | South entrance | 4.39 |  |  |
| 11/22/2021 | Collection west | 2.39 |  |  |
| 12/6/2021 | North entrance | 2.74 | 2.6 | 0.1 |
| 1/27/2022 | Transportation east | 4.17 | 3 | 1.8 |
| 1/27/2022 | ITS south | 4.32 | 3.1 | 1.2 |
| 1/28/2022 | Collection east | 2.02 | 3 | 1.0 |
| 1/28/2022 | Collection west | 3.19 | 3 | 0.2 |
| 1/28/2022 | South entrance | 3.89 | 3.9 | 0.0 |
| 1/28/2022 | ITS south | 4.17 | 3.9 | 0.3 |
| 1/28/2022 | Transportation east | 4.46 |  |  |
| 1/29/2022 | Transportation east | 4.43 | 2.9 | 1.5 |
| 1/29/2022 | South entrance | 4.20 | 3.9 | 0.3 |
| 1/29/2022 | South bridge east | 4.17 | 3.9 | 0.3 |
| 1/30/2022 | Transportation east | 4.31 | 2.8 | 1.5 |
| 1/30/2022 | South entrance | 4.79 | 4.2 | 0.6 |
| 1/30/2022 | South bridge east | 4.18 | 3.3 | 0.9 |
| 1/30/2022 | Collection east | 1.77 |  |  |

Conclusion;

-Using the float method down long sections of channels we usually underestimating the total velocity average due to surface tension and having the float hang up in various areas.

-Entrance area with short straight runs had similar results

-The meter is more likely to measure the highest velocity in the channel because it measures closer to mid channel. Areas along walls, floor and at surface will always be slower due to friction.

Questions;

-Where did 1.5-4.0 (optimum 2) come from? Reference to this criteria should be included in the FPP?

-If we have areas >4 fps, is that a problem for fish passage? They can follow the floor with less velocity.