# Fish Passage Plan (FPP) Change Form

Change Form # & Title:	22BON003 - Reduce frequency of velocity readings

Date Submitted: 		11-MAY-2022

Project: 			Bonneville Dam

Requester Name, Agency: 	Andrew Derugin, Tucker Gossett, Rebecca Cates, Jeanette Wendler -

USACE

Final Action:

FPP Section: BON section 2.4.2.7 Adult Fish Facilities / Adult Fish Passage Season

Justification for Change:

BON Fisheries measures water velocities daily during fishway inspections to ensure velocities are between FPP criteria of 1.5 – 4 feet per second. We would like to reduce the frequency of readings to once per week. This frequency reflects the protocols of other dams in the district. For example, The Dalles Dam – FPP section 2.4.2.6. only requires water velocity readings once per week during the adult fish passage season. BON Fisheries believes this request is justified because water velocities are rarely out of criteria (out of criteria velocity occurred zero times so far in 2022, twice in 2021, and once in 2020). If the velocity reading is out of criteria, we will increase the weekly readings to three times a week until proper channel conditions are met.

BON Fisheries is implementing the use of small portable flow meters to measure water velocities. The portable flow meters will replace the old stationary flow meter in Photo 1. Stationary flow meters are very large and in the center of the collection channel where fish must navigate around to pass. The stationary flow meters require constant positioning with changing channel elevations and collect vegetation/debris that comes down the fishway. This requires personnel to frequently raise the flow meter to clean or reposition to match elevations. The small, portable flow meters (Photo 2) will reduce time and effort spent to clean the equipment and reposition with changing elevations. They will also give the advantage of being deployed anywhere in the channel which will give a better overall representative reading of channel velocity. BON fisheries suggest removing all stationary flow meters from the fishways.

Photo 1. Old stationary flow meter        Photo 2. New mechanical flow meter

Proposed Change: [see below with edits to existing FPP in track changes]

- 2.4.2. 	Adult Fish Facilities – Adult Fish Passage Season (March 1 – November 30).

      - Maintain water velocity between 1.5 feet per second (fps) and 4 fps (2 fps preferred) for the full length of the powerhouse collection channel and the lower ends of the fish ladders that are below tailwater. Measure water velocities  once per week during fishway inspections to verify channels are operating between 1.5 and 4 fps. If the velocity reading is out of criteria, weekly readings will increase to three times a week until proper conditions are met.