OFFICIAL COORDINATION REQUEST

FOR NON-ROUTINE OPERATIONS AND MAINTENANCE

COORDINATION TITLE- 24TDA03 TDA MU22 Priority

COORDINATION DATE- 27 March 2024

PROJECT- The Dalles Dam

RESPONSE DATE- 11 April 2024 (FPOM)

Description of the Problem

Due to breaker issues MU22 operation is required to first on/last off. This will be required through August. This will affect the powerhouse unit priority per Fish Passage Plan requirement for loading units west to east. It will be more prevalent in the summer as flows decrease and less units are needed for operation. MU18 is required as 1 of 3 (MU1, MU8, MU18) priority units for providing surface flow into the sluiceway. If MU22 is considered the replacement for MU18 for this passage season, with its sluicegates open, the unit priority criteria can be maintained.

Type of outage required – MU18 sluicegates closed. MU22 sluicegates open through the passage season.

Impact on facility operation – Change in unit priority.

Dates of impacts/repairs – Apr 15 – Aug 30. Juvenile spill passage season.

Length of time for repairs – NA

Expected impacts on fish passage –

Upstream migrants (including Bull Trout).

No impacts expected. All 6 sluicegates will continue to be open for downstream adult steelhead passage. Sluiceway volume will be remaining the same.

Downstream migrants.

No impact expected. Studies have shown benefit to having an open sluicegate on the east end, but no preference to the specific unit. Its most important that the selected sluicegates have an operating unit under it. MU18 was selected due to ease of operation with push button sluicegates. All other gates on east end of powerhouse require crane for chain gate movement.

Lamprey – No impact expected.

Comments from agencies –.

Final Coordination Results –

After Action Update

Please email or call with questions or concerns. Thank you,

Bob Cordie/Eric Grosvenor

TDA Project Fisheries

541-506-8275

Patricia Madson

NWP Columbia River Coordination

Patricia.l.madson@usace.army.mil