CENWW-OD-EL







May 3, 2013
MEMORANDUM FOR THE RECORD  (13LWG06)
SUBJECT:  Dam Safety Spill Bay Test at Lower Granite
Due to dam safety considerations, Lower Granite Project is systematically taking out one spill bay at a time today, May 3, to investigate excess water passage through the spill bays, which may be the result of a failing expansion joint.  Beginning at 0830 hours, starting at bay 2 and moving toward bay 8, each bay will be closed for approximately 30 minutes.  Bay 1, with the removable spillway weir, will not be closed.  The 20 kcfs total spill required by the 2013 Fish Operations Plan will be maintained during the test by augmenting spill through other bays as needed.
This operation was scheduled by the project with the Reservoir Control Center late yesterday.  
