 
CENWP-OP-B                                                                                                      21 March 2000

MEMORANDUM FOR CENWP-OP, ATTN:  Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 11 of 2000, which covers the period 12 through 18 March.
1.  OPERATION SUMMARY.

1.1. Daily average river flow ranged from 193.7 to 214.2 kcfs.  Daily average tailwater elevation for the project ranged from 17.0’ to 18.0’ msl.  Daily average spill ranged from 94.4 to 119.5 Kcfs on 12 through 16 March this reporting week for the Spring Creek release.  River temperature ranged from 43 to 45° F.  Spill bays 1 and 18, which supply attraction flow for the Cascades Island ladder and Bradford Island ladder respectively, remain open 4”.  

1.2.  First powerhouse unit 10 remained out of service for stator rewind, unit 18 remained out of service for stator rewedge, and unit 5 remained out of service for turbine release pipe removal.  Unit 2 was out of service from 0648-1306 on 15 March for disconnect maintenance.  Units 7, 8, and 9 were out of service from 0706-1000 on 16 March to assist divers in recovering a bulkhead. 

1.3.  The first powerhouse ice and trash sluiceway remained open this reporting week.  The second powerhouse sluiceway remained open to elevation 68’ through 2000 on 16 March in accordance with the special operation for the Spring Creek release.
1.4.  Second powerhouse fish units remained in service.  The units are being placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.
2.  ACTIVITIES. 
2.1. AWS Closures.  FV1-1 was closed for cleaning from 1515-1535 on 13 March, and FV5-9 was closed for cleaning from 0705-0730 on 14 March.

2.2. Dewaterings and Fish Salvage.  There was no dewaterings or fish salvage this reporting week.
3.  RESEARCH. 
3.1. PH2 Bypass System Evaluation.  NMFS continued releasing PIT tagged juvenile chinook through the new bypass system as part of their post construction evaluation to determine if fish condition is adversely affected by system passage. 

3.2. PSMFC Smolt Monitoring.  NMFS continued 24-h/d PIT tag interrogation at PH2 JMF. 

3.3. WDFW Fish Counting.  WDFW continued 24-h/d video counting at Bradford Island and Washington shore. 

4.  FISHWAYS.  Project biologists inspected on 13, 14, and 15 March this reporting week.

4.1. Fish Passage Plan Observations. 

DATE
LOCATion
FPP Deviation
CAUSE
RESPONSE

3/15
PH1 collection channel velocity meter.
Meter registered 1.2 fps and should register 1.5-4.0 fps.
Unknown.
No action taken.

4.2. Fish Ladder Temperatures.  Readings are taken 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder below the count station at Washington shore.  Temperatures are supplied by The Fisheries Field Unit.
Date
Wash. Shore
Bradford Is.

3/12
6.0
5.9

3/13
6.1
6.0

3/14
6.2
6.1

3/15
6.3
6.2

3/16
6.4
6.4

3/17
6.4
6.3

3/18
6.5
6.4

                                                 Fish Ladder Temperatures ((C) 
4.3. Sensor Calibration.  Water level sensors were checked for calibration on 16 March this reporting week.   

4.4. Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

5.  OTHER.

5.1. Special Operation for Spring Creek Hatchery Release.  The special operation for the Spring Creek hatchery release, which was initiated at 2000 on 9 March, ended at 2000 on 16 March.  The operation called for 100 Kcfs spill at total flow of 200-220 Kcfs and 80 Kcfs spill at total flow of 180-200 Kcfs.  PH2 was operated as priority with two units operating at PH1 for station service.  The PH2 ice and trash sluiceway was opened to elevation 68’.

                                                                                                     DEBORAH L. CHENOWETH

                                 

                                                              Operations Project Manager

//S//

2
3

