 
CENWP-OP-B                                                                                                          5 April 2000

MEMORANDUM FOR CENWP-OP, ATTN:  Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 13 of 2000, which covers the period 26 March through 1 April.
1.  OPERATION SUMMARY.

1.1. Daily average river flow ranged from 166.1 to 211.1 kcfs.  Daily average tailwater elevation for the project ranged from 15.0’ to 17.3’ msl.  River temperature ranged from 46 to 47° F.  Spill bays 1 and 18, which supply attraction flow for the Cascades Island ladder and Bradford Island ladder respectively, remain open 4”.  

1.2.  First powerhouse unit 10 remained out of service for stator rewind, and unit 18 remained out of service for stator rewedge.  Units 5, 6, and 7 were out of service from 1300-1424, 0648-1554, and 1542-1654 on 27 March respectively, for divers to repair research antennas.  Unit 7 was also out of service from 0642-1048 on 28 March for divers to repair research antennas.  Unit 1 was out of service from 0706-1646 and 0806-0954 on 29 and 30 March respectively, for divers to repair research antennas.  Units 4, 5, and 6 were also out of service from 1000-1124 on 30 March for divers to repair research antennas. 

1.3.  The first powerhouse ice and trash sluiceway remained open this reporting week.   The second powerhouse sluiceway was opened from 1534-1603, 1142-1200, and 1014-1031 on 27, 29, and 30 March respectively, for debris removal. 
1.4.  Second powerhouse fish units remained in service this reporting week.  The units are being placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.
2.  ACTIVITIES. 
2.1. AWS Closures.  There were no AWS closures this reporting week.

2.2. Dewaterings and Fish Salvage.  There was no dewaterings or fish salvage this reporting week. 

2.3. STS Inspections.  Two units’ STS’s were inspected this reporting week.  All screens were in good condition unless otherwise noted below.  See Table 1.

Unit
Date
Run Hours
Comments

11
3/29
643
Replaced all screens

12
3/30
643


Table 1.  STS Inspection Data.


3.  RESEARCH. 
3.1. AFC&MF Adult PIT-tag Interrogation.  NMFS has completed enlarging the orifices of the six weirs in the AFC&MF return ladder from 18” to 26”.  They will be testing 18” and 26” detectors using PIT-tagged adult chinook and steelhead from the lab to get estimates of reading efficiencies for the two orifices.  They will be running several tests throughout the season with the first beginning in mid April with the 18” inserts. 

3.2. PH2 Bypass System Evaluation.  NMFS has completed releasing PIT tagged juvenile chinook and steelhead through the new bypass system.  They will resume releases in May as part of their post construction evaluation to determine if fish condition is adversely affected by system passage. 

3.3. PSMFC Smolt Monitoring.  NMFS continued 24-h/d PIT tag interrogation at PH2 JMF.  They are collecting fish for approximately 10 minutes per hour and sampling daily at 0800. 

3.4. WDFW Fish Counting.  WDFW continued 24-h/d video counting at Bradford Island and Washington shore and began 16-h/d visual counting on 1 April. 

4.  FISHWAYS.  Project biologists inspected on 27, 28, and 29 March this reporting week.

4.1. Fish Passage Plan Observations.  There were no Fish Passage Plan deviations this reporting week.

4.2. Fish Ladder Temperatures.  Readings are taken 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder below the count station at Washington shore.  Temperatures are supplied by The Fisheries Field Unit.
Date
Wash. Shore
Bradford Is.

3/26
7.3
7.3

3/27
7.5
7.4

3/28
7.4
7.4

3/29
7.5
7.5

3/30
7.7
7.6

3/31
7.8
7.8

4/1
8.1
8.1

                                                 Fish Ladder Temperatures ((C) 
4.3. Sensor Calibration.  Water level sensors were checked for calibration on 30 March this reporting week.   

4.4. Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.


5.  OTHER NOTEABLE ITEMS.

5.1. DSM2 solenoid valves are scheduled to be replaced on 10 through 21 April.  This is due to continued failures of additional valves in the DSM.  As a result, the channel will have to be operated in manual control during this time and orifices will have to be closed one at a time for 2-3 hours each.

                                                                                                     DEBORAH L. CHENOWETH

                                 

                                                              Operations Project Manager

//S//

2
2

