 
CENWP-OP-B                                                                                               02 October 2000

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 39 of 2000, which covers the period 24 September through 30 September.
1.  OPERATION SUMMARY.

1.1 Average daily river flow ranged from 100.0 to 124.3 kcfs.  Average daily tailwater elevation for the project ranged from 10.0’ to 11.5’ msl.  Average daily forebay elevation for the project ranged from 73.5’ to 74.3’ msl.  River temperature ranged between 64 and 65° F.  Spill bays 1 and 18, which supply attraction flow for Cascades Island ladder and Bradford Island ladder respectively, remain open 4”.

1.2 Unit 3 remained out of service for turbine rehab.  Unit 4 remained out of service due to high bearing temperatures.  Unit 6 remained out of service for 100-day inspection.  Unit 10 went out of service for transformer bank maintenance 0430 until 0824 25 September.   Unit 9 went out of service for transformer bank maintenance 0824 25 September until 1648 26 September.  Unit 13 went out of service for it’s four year overhaul 0824 25 September.
1.3 Second powerhouse fish units were operated normally all week.  The units are being placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.
2.  ACTIVITIES. 
2.1 AWS Closures.  No AWS closures.

2.2 Dewatering and Fish Salvage.  Salvaged two sturgeon, four channel catfish, and one juvenile fall chinook from Unit 13 draft tube.

2.3 STS Inspections.  Nine STSs were inspected this reporting week.  

Unit
Date
Run Hours
Comments

1
9/28
672
Video inspection

11
9/28
589
Video inspection

12
9/28
292
Video inspection

13
9/28
296
Pulled for U13 overhaul

14
9/28
450
Video inspection

15
9/27
801
Video inspection

16
9/27
697
Video inspection

17
9/27
681
Video inspection.  Rip in A slot mesh, repaired with a patch.

18
9/27
822
Video inspection

3.   RESEARCH. 
3.1 AFC&MF Adult PIT-tag Interrogation.  The 26” orifice inserts and shields were installed in the return ladder of the AFC&MF.  NMFS is testing and PIT-tagging adults from the lab to get estimates of reading efficiencies for the 26” orifice inserts.

3.2 AFC&MF Adult Sampling and Tagging.  The University of Idaho continues radio tagging and tracking adult Chinook for their adult telemetry research.  They are currently sampling 7 days per week.  CRITFC and ODFW are sampling adult Chinook and steelhead approximately 2 days per week for their age composition research.  

3.3 Fisheries Field Unit Video Monitoring.  The Fisheries Field Unit is assisting NMFS adult PIT-tag interrogation work this year during their testing at the AFC&MF.  They are monitoring submerged orifice and overflow weir passage in the return ladder of the AFC&MF and assisting NMFS in verifying passage of streamer-marked PIT-tag fish during their testing.  They are also periodically monitoring submerged orifice and overflow weir passage at Cascades Island and Washington Shore ladders this year.

3.4 Normandeau/Mid-Columbia Consulting/Battelle PH2 Sluicechute Survival Testing.  Researchers continue releasing balloon tagged fish through the second powerhouse ice and trash sluiceway for survival testing.  This work is in support of corner collector research.  

3.5 NMFS Lamprey Tagging and Radio Tracking.  NMFS continues trapping and tagging lamprey in the AFC&MF.  They are releasing the radio tagged fish downstream and tracking the fish back through the project in cooperation with the University of Idaho.

3.6 PSMFC DSM1 and JMF Smolt Monitoring.  PSMFC finished 24-h/d PIT tag interrogation at DSM1.  They continue PIT tag interrogation and 24 h/d sampling at PH2 JMF.

3.7 University of Idaho Lamprey Test Flume.  The University of Idaho continues testing lamprey-swimming capabilities under various flow conditions in the lamprey test flume at the AFC&MF.     

3.8 WDFW Fish Counting.  WDFW continues 16-h/d visual and 8-h/d video counting at Bradford Island and Washington shore.   

3.9 WDFW/ODFW Fall Chinook counting.  WDFW and ODFW are still counting Fall Chinook at the Bradford Island and Washington shore count stations during the day.

3.10 Yakima Nation.  Tribal members are fishing for northern pikeminnow from the powerhouses during the afternoon and evening hours.

4.  FISHWAYS. 
4.1 Project biologists inspected on 25, 26, 27, 28, and 29 September this reporting week. 

4.2 Fish Passage Plan Observations.

DATE
LOCATion
FPP Deviation
CAUSE
RESPONSE

All week
Cascades Island diffuser FG6-18.
Diffuser is stuck partially open and should be closed.
Unknown.
Will repair during winter maintenance period.

All week.
PH1 and PH2 collection channel electronic velocity meter.
Meter was not working and should register 1.5-4.0 fps per the FPP.
Unknown.
Investigating alternative method of measure.  



4.3 Fish Ladder Temperatures.  Readings are taken 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are supplied by Fisheries Field Unit.

Date
Wash. Shore
Bradford Is.

9/24
17.7
17.6

9/25
17.6
17.4

9/26
17.6
17.4

9/27
17.7
17.5

9/28
17.9
17.7

9/29
17.9
17.8

9/30
18.0
17.8

                                                 Fish Ladder Temperatures ((C) 
4.3 Sensor Calibration.  Water level sensors were checked for calibration on 28 September this reporting week.  

4.4 Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

                                                                                                     DEBORAH L. CHENOWETH

                                 

                                                              Operations Project Manager

//S//

1
3

