 
CENWP-OP-B                                                                                               11 December, 2000

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 49 of 2000, which covers the period 03 through 09 December.
1.  OPERATION SUMMARY.

1.1 Average daily river flow ranged from 133.9 to 162.6 kcfs.  Average daily tailwater elevation for the project ranged from 11.7’ to 13.9’ msl.  Average daily forebay elevation for the project ranged from 73.5’ to 74.6’ msl.  River temperature ranged between 45 and 48° F.  Spill bay 1, which supplies attraction flow for Cascades Island ladder, remained open 4”.  Spill bay 18, which supplies attraction flow for Bradford Island ladder, went on seal at 0801 04 December.

1.2 Unit 3 remained out of service for turbine rehab.  Unit 2 returned to service at 0748 07 December.  Unit 13 returned to service at 1236 05 December.  Unit 11 was out of service from 0500 04 December until 1612 05 December for orifice ring replacement.  Unit 14 was out of service from 1030 until 1100 04 December for relay replacement.  Unit 13 was out of service from 1936 06 December until 0806 07 December due to the oil pumps cycling.  Unit 11 was out of service from 1000 until 1500 07 December for a breaker inspection.  Unit 2 was out of service from 1054 until 1242 08 December for a thrust bearing oil change.
1.3 Second powerhouse fish units operated normally this reporting week.  The units are being placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.
2.  ACTIVITIES. 
2.1 AWS Closures.  FV1-1, FV3-7, FV4-3, and FV4-4 were closed for the Bradford Island ladder dewatering at 1145 03 December.  FV6-9 and FV5-9 were closed for cleaning from 1248 until 1312 09 December.

2.2 STS Inspections.  No STS’s were inspected this reporting week.

2.3 Dewatering and Fish Salvage.
Bradford Island junction pool; 10 wild steelhead, 11 hatchery steelhead, 2 wild coho, 1 hatchery coho, 1 wild chinook, 5 hatchery chinook, 11 rainbow trout, and assorted residents and shad were recovered 05 December. Please note that the junction pool fish numbers included juveniles though no distinction is made in the counts.  

Bradford Island A-Branch; 1 dead, tagged hatchery steelhead, 1 juvenile steelhead, 1 sucker, 2 juvenile wild chinook, and about 5 whitefish and peamouth were recovered 05/06 December.  

Bradford Island B-Branch; 1 dead lamprey, 7 lamprey, 1 juvenile chinook, 4 sculpin, and 3 whitefish were recovered 07/08 December.

3.   RESEARCH. 
3.1 AFC&MF Adult PIT-tag Interrogation.  The 26” orifice inserts and shields remain installed in the return ladder of the AFC&MF.  

3.2 AFC&MF Adult Sampling and Tagging. WDFW is sampling adult steelhead approximately 7 days per week, throughout the winter, for age composition research.  

3.3 Fisheries Field Unit Video Monitoring.  The Fisheries Field Unit is assisting NMFS adult PIT-tag interrogation work this year during their testing at the AFC&MF.  They are monitoring submerged orifice and overflow weir passage in the return ladder of the AFC&MF and assisting NMFS in verifying passage of streamer-marked PIT-tag fish during their testing.  They are also periodically monitoring submerged orifice and overflow weir passage at Cascades Island and Washington Shore ladders this year.

3.4 NMFS Lamprey Tagging and Radio Tracking.  NMFS completed trapping and tagging lamprey in the AFC&MF.  They are still tracking the fish back through the project in cooperation with the University of Idaho.

3.5 WDFW Fish Counting.  WDFW continues 24-h/d video counting at Washington shore.   

4.  FISHWAYS. 
4.1 Project biologists inspected on 04, 06, and 07 December this reporting week.

4.2 Fish Passage Plan Observations.

DATE
LOCATion
FPP Deviation
CAUSE
RESPONSE

All week
Cascades Island diffuser FG6-18.
Diffuser is stuck partially open and should be closed.
Unknown.
Will repair during winter maintenance period.

All week.
PH2 collection channel electronic velocity meter.
Meter was not working and should register 1.5-4.0 fps per the FPP.
Unknown.
Investigating alternative method of measure.  



All week
Cascade Island bird wires.
One wire is down.
Rangers cut wire to remove entangled gull.
Will repair during winter maintenance period.

4.3  Fish Ladder Temperatures.  Readings are taken 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are supplied by Fisheries Field Unit.

Date
Wash. Shore
Bradford Is.

12/3
7.3
7.2

12/4
7.1
7.0

12./5
7.0


12/6
6.8


12/7
6.7


12/8
6.6


12/9
6.7


                                                 Fish Ladder Temperatures ((C) 
4.4 Sensor Calibration.  Water level sensors were checked for calibration on 06 December this reporting week. 

4.5 Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

5. MISCELLANEOUS

5.1 Because of mechanical reasons, DSM2 screen cleaners are run once per shift (or more often depending on debris loads) to reduce wear and tear on the equipment. 

5.2 Bradford Island went to orifice flow at 1145 03 December in anticipation of the winter outage scheduled for 05 December.  

5.3 Units 11 and 12 are scheduled to be on standby from 06 December until 10 December to facilitate the drilling in the second powerhouse tailrace.

                                                                                                     DEBORAH L. CHENOWETH

                                 

                                                              Operations Project Manager

//S//

1
2

