 
CENWP-OP-B                                                                                                                                   16 April 2001

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 15 of 2001, which covers the period 08 through 14 April.
1.  OPERATION SUMMARY.

1.1 Daily river flow averaged 98.2 to 137.2kcfs.  Daily spill averaged 0 kcfs.  Daily project tailwater elevation averaged 9.3’ to 12.8 msl.  Daily project forebay elevation averaged 75.4’ to 75.8’ msl.  River temperature averaged 49 to 50° F.  Spill bays 1 and 18, which supply attraction flow for Cascades Island and Bradford Island ladders respectively, remain open 4” during daylight hours.  They are closed for water conservation between sunset and one hour before sunrise per BPA request.  

1.2 Unit 3 remained out of service for turbine rehab.   Unit 15 remained out of service for gate slot modifications.   It is expected to come back in service next week.

1.3 Powerhouse Two fish units remain in service.  They provide attraction flow for the Washington Shore fish ladder.  The units are being placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

2.  ACTIVITIES. 
2.1 AWS Closures.   FV4-3 and FV4-4 at B-branch and FV5-3 and FV5-4 are all operating in manual until the computer system is reinstalled. 

2.2 STS/VBS Inspections.  The following is a list of units whose screens were video inspected this reporting week and the run time between inspections.

Unit
Date
Run Hours
Comments

01
4/10
469


09
4/10
502


10
4/9
709


Table 1.  STS/VBS Inspection Data.


2.3 Dewatering and Fish Salvage.   No dewatering or fish salvage activities this week.

 
3.   RESEARCH. 

3.1 WDFW Fish Counting.  WDFW continues 16-hour visual counts and 8-hour video counts at Bradford Island and Washington Shore ladders.

3.2 NMFS Saltwater rearing. The Saltwater rearing facility is under construction.  Fish collection is scheduled to begin in early May.

3.3 OCFWRU Barging condition study.  Expected to begin at the end of April.

3.4 ODFW Pikeminnow electrofishing.  Capture and tagging program for pikeminnow, smallmouth bass, and walleye in the tailrace and forebay.  Expected to begin next week. 

3.5 PSMFC Adult PIT.  Collection and tagging of fish began April 10th. 

3.6 PSMFC Smolt Monitoring Program.  Collection continues 24h/d at the monitoring facility.  Monitoring at DSM1 began 9 April.

3.7 WES Hydroacoustic Monitoring.  Transducers are installed at Units 1-5 and Units 7-10.  Monitoring is expected to begin 23 April.

3.8 UI Barging condition study. They will begin sampling juvenile fish from transport barges on 22 April.

3.9 USGS Juvenile Radiotracking.  Radio tagging began 1 April.  Tracking should begin 15 April.

3.10 USGS Adult EMG tracking.  Underwater and aerial antennas are installed around Bradford Island and on the pier noses of the spillway.  Tracking may begin next week.  

3.11 UI Adult Fallback. UI Adult Radiotracking/Tagging.  PCC entrance gates at Powerhouse One are operated according to UI request.  This will continue until 31 October.   Tagging of Chinook began at the AFC&MF 2 April and will continue seven days a week until the end of the season. 

3.12 CRITFC/ODFW.  Tagging Spring Chinook on Tuesdays and Thursdays, with Friday as an extra day if needed.

3.13 WDFW.  Tagging Spring Chinook with jaw tags in conjunction with their tooth-net studies.

4.  FISHWAYS. 
4.1 Project biologists inspected on 09, 10, and 12 April this reporting week.

4.2 Fish Passage Plan Observations.

DATE
LOCATion
FPP Deviation
CAUSE
RESPONSE

All week.
PH1and PH2 collection channel electronic velocity meter.
Meter was not working and should register 1.5-4.0 fps.
Unknown.
New velocity meters to be installed during mid-season dives.

4/9
FG3-26
Should have been open.
Controls are in manual at B-Branch.
Operators adjusted valves and gates with the changing tailwater.

4.3  Fish Ladder Temperatures.  Temperatures are measured 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.

Date
Wash. Shore
Bradford Is.

4/8
8.4
8.3

4/9
8.4
8.4

4/10
8.4
8.4

4/11
8.4
8.4

4/12
8.4
8.4

4/13
8.4
8.4

4/14
8.4
8.3

4.4 Sensor Calibration.  Water level sensors were checked on 10 April for calibration by biologists this reporting week.  

4.5 Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

5. MISCELLANEOUS

5.1 The high outfall hydrocannons for the Second Powerhouse monitoring facility remain in service.
5.2 As of 22 March, DSM2 orifice 11A-S, F2A-N, and F2B-N are kept open to keep the channel elevation within criteria while Unit 15 is out of service.
5.3 DSM2 Add-in water remains in operation.  
5.4 All week long there were reports of seals or sea lions in the tailraces of both powerhouses.  The Project did not take any action towards removing the natural predators from the river system.
5.5 From 0816 09 April until 1605 12 April the monitoring facility operated in bypass mode to facilitate follow-on work in the sample flumes.
5.6 PH1 ITS was closed from 1720 until 1835 09 April while USGS worked in the BRZ.
5.7 DSM2 screen cleaners were off during the days of 11-13 April for modifications and testing.
5.8 UI discovered one dead clipped Chinook at the AFF on 11 April.  It had jumped between two layers of netting surrounding the collection pool.  Unfortunately it had expired by the time anyone found it.  The top of the netting has since been sealed.
5.9 At 1605 on 13 April, the lights at the viewing windows were turned off after reports of large numbers of fish holding in the ladders.

                                                                                                                GENE L. LADOUCEUR

                                 

                                                             Operations Project Manager

//S//

1
3

