 
CENWP-OP-B                                                                                                                                   23 April 2001

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 16 of 2001, which covers the period 15 through 21 April.
1.  OPERATION SUMMARY.

1.1 Daily river flow averaged 98.2 to 137.0kcfs.  Daily spill averaged 0 kcfs.  Daily project tailwater elevation averaged 9.3’ to 12.3 msl.  Daily project forebay elevation averaged 74.9’ to 75.6’ msl.  River temperature averaged 49 to 51° F.  Spill bays 1 and 18, which supply attraction flow for Cascades Island and Bradford Island ladders respectively, remain open 4” during daylight hours.  They are closed for water conservation between sunset and one hour before sunrise per BPA request.  

1.2 Unit 3 remained out of service for turbine rehab.   Unit 15 returned to standby at 2142 on 19 April.  It is being operated last on first off.

1.3 Powerhouse Two fish units remain in service.  They provide attraction flow for the Washington Shore fish ladder.  The units are being placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

2.  ACTIVITIES. 
2.1 AWS Closures.   FV4-3 and FV4-4 at B-branch and FV5-3 and FV5-4 are all operating in manual until the computer system is reinstalled. 

2.2 STS/VBS Inspections.  STSs were installed in Unit 15 on 19 April.

2.3 Dewatering and Fish Salvage.   Project biologists salvaged three channel catfish and five sculpin from Unit 15 tail logs on 18 April.

 
3.   RESEARCH. 

3.1 WDFW Fish Counting.  WDFW continues 16-hour visual counts and 8-hour video counts at Bradford Island and Washington Shore ladders.

3.2 NMFS Saltwater rearing. The Saltwater rearing facility is under construction.  Fish collection is scheduled to begin in early May.

3.3 OCFWRU Barging condition study.  Expected to begin 24 April.

3.4 ODFW Pikeminnow electrofishing.  Capture and tagging program for pikeminnow, smallmouth bass, and walleye in the tailrace and forebay took place this week.   

3.5 PSMFC Adult PIT.  Chinook are collected and tagged on Wednesdays.

3.6 PSMFC Smolt Monitoring Program.  Collection continues 24h/d at the monitoring facility.  Monitoring at DSM1 continues.

3.7 WES Hydroacoustic Monitoring.  Transducers are installed at Units 1-5 and Units 7-10.  Monitoring is expected to begin 23 April.

3.8 UI Barging condition study. They will begin sampling juvenile fish from transport barges on 22 April.

3.9 USGS Juvenile Radiotracking.  Radio tagging began 1 April. 

3.10 USGS Adult EMG tracking.  Underwater and aerial antennas are installed around Bradford Island and on the pier noses of the spillway.  Fish release and tracking began 18 April.  

3.11 UI Adult Fallback. UI Adult Radiotracking/Tagging.  PCC entrance gates at Powerhouse One are operated according to UI request.  This will continue until 31 October.   Tagging of Chinook began at the AFC&MF 2 April and will continue seven days a week until the end of the season. 

3.12 CRITFC/ODFW.  Tagging Spring Chinook and steelhead on Tuesdays and Thursdays, with Friday as an extra day if needed.

3.13 WDFW.  Tagging Spring Chinook with jaw tags in conjunction with their tooth-net studies.

4.  FISHWAYS. 
4.1 Project biologists inspected on 16, 17, and 20 April this reporting week.

4.2 Fish Passage Plan Observations.

DATE
LOCATion
FPP Deviation
CAUSE
RESPONSE

All week.
PH1and PH2 collection channel electronic velocity meter.
Meter was not working and should register 1.5-4.0 fps.
Unknown.
New velocity meters to be installed during mid-season dives.

One day this week.
Calibration check.
Not done
Short staffed
No action taken

4.3  Fish Ladder Temperatures.  Temperatures are measured 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.

Date
Wash. Shore
Bradford Is.

4/15
8.3
8.3

4/16
8.5
8.5

4/17
8.8
8.8

4/18
9.1
9.1

4/19
9.3
9.3

4/20
9.4
9.4

4/21
9.5
9.6

4.4 Sensor Calibration.  Water level sensors were not checked for calibration by biologists this reporting week.  

4.5 Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

5. MISCELLANEOUS

5.1 The high outfall hydrocannons for the Second Powerhouse monitoring facility remain in service.
5.2 Unit 15 returned to service on 20 April.  DSM2 orifices F2A-N and F2B-N were closed with their lights unplugged.  11A-S remains in auto.
5.3 DSM2 Add-in water remains in operation.  On 19 April 7 dead fish were removed from the perf plate.  On 20 April 6 dead fish were removed.
5.4 All week long there were reports of seals or sea lions in the tailraces of both powerhouses.  The Project did not take any action towards removing the natural predators from the river system.
5.5 PH1 ITS was closed from 0112 until 0211 18 April while ODFW worked in the BRZ.
5.6 DSM2 screen cleaners were being run once per shift while repairs were made to the PLC system 15-17 April.  NMFS requested the sweeps be run through the weekend once per shift with the time and debris load noted for noise testing with the Adult PIT tag detectors.
                                                                                                                GENE L. LADOUCEUR

                                 

                                                             Operations Project Manager

//S//

2
2

