 
CENWP-OP-B                                                                                                                                   14 April 2001

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 18 of 2001, which covers the period 29 April through 05 May.
1.  OPERATION SUMMARY.

1.1 Daily river flow averaged 134.9 to 161.9kcfs.  Daily spill averaged 0 kcfs.  Daily project tailwater elevation averaged 12.6’ to 14.5’ msl.  Daily project forebay elevation averaged 75.3’ to 75.5’ msl.  River temperature averaged 54 to 55° F.  

1.2 Spill bays 1 and 18, which supply attraction flow for Cascades Island and Bradford Island ladders respectively, remain open 4” during daylight hours.  They are closed for water conservation between sunset and one hour before sunrise per BPA request.  

1.3 Unit 3 remained out of service for turbine rehab.  Unit 15 will be used only for FGE testing from now until 15 July.

1.4 Powerhouse Two fish units remain in service.  They provide attraction flow for the Washington Shore fish ladder.  The units are being placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

2.  ACTIVITIES. 
2.1 AWS Closures.   FV4-3 and FV4-4 at B-branch and FV5-3 and FV5-4 are all operating in manual until the new computer system is installed.   

2.2 STS/VBS Inspections.  Video inspection results of STS/VBS screens include the following units.

Unit
Date
Run Hours
Comments

11
5/2
699


12
5/2
644


13
5/3
617


14
5/3
639


17
5/4
643


18
5/4
649


2.3 Dewatering and Fish Salvage.   No fish salvage this week.

 
3.   RESEARCH. 

3.1 WDFW Fish Counting.  WDFW continues 16-hour visual counts and 8-hour video counts at Bradford Island and Washington Shore ladders.

3.2 NMFS Saltwater rearing. The Saltwater rearing facility plans to start operations by 11 May.

3.3 OCFWRU Barging condition study.  Began 24 April.

3.4 ODFW Pikeminnow electrofishing.  Done for this year.

3.5 PSMFC Adult PIT.  Chinook are collected and tagged on Wednesdays.

3.6 PSMFC Smolt Monitoring Program.  Collection continues 24h/d at the monitoring facility.  Monitoring at DSM1 continues.

3.7 WES Hydroacoustic Monitoring.  Transducers are installed at Units 1-5 and Units 7-10.  Monitoring began  20 April.

3.8 UI Barging condition study. They began sampling juvenile fish from transport barges on 22 April.

3.9 USGS Juvenile Radiotracking.  Radio tagging began 1 April.  First release from Hood River bridge is expected to occur on 1 May.

3.10 USGS Adult EMG tracking.  Underwater and aerial antennas are installed around Bradford Island and on the pier noses of the spillway.  Fish release and tracking began 18 April.  

3.11 UI Adult Fallback. UI Adult Radiotracking/Tagging.  PCC entrance gates at Powerhouse One are operated according to UI request.  This will continue until 31 October.  Tagging of Chinook began at the AFC&MF 2 April and will continue seven days a week until the end of September. 

3.12 CRITFC/ODFW.  Tagging Spring Chinook and steelhead on Tuesdays and Thursdays, with Friday as an extra day if needed.

3.13 WDFW.  Tagging Spring Chinook with jaw tags in conjunction with their tooth-net studies.

4.  FISHWAYS. 
4.1 Project biologists inspected on 30 April, 1,2, 3 and 4 May this reporting week.

4.2 Fish Passage Plan Observations.

DATE
LOCATion
FPP Deviation
CAUSE
RESPONSE

All week.
PH1and PH2 collection channel electronic velocity meter.
Meter was not working and should register 1.5-4.0 fps.
Unknown.
New velocity meters to be installed during mid-season dives.

4/30
Spill bays 1 and 18
Closed during the daylight hours.
Dive at Bradford Island landfill.
COE District coordinated this deviation with NMFS.

5/3
11A-S DSM2
One steelhead smolt found on walkway.
Water jet dislodged grating at 11A-S.
Orifice closed until repairs are made.

4.3  Fish Ladder Temperatures.  Temperatures are measured 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.

Date
Wash. Shore
Bradford Is.

4/29
11.2
11.1

4/30
11.2
11.1

5/1
10.8
10.8

5/2
10.8
10.7

5/3
11.3
11.3

5/4
11.7
11.6

5/5
in next week’s report
in next week’s report

4.4 Sensor Calibration.  Water level sensors were checked for calibration on 30 April this reporting week.  

4.5 Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

5. MISCELLANEOUS

5.1 DSM2 Add-in water remains in operation.  Four dead smolts were removed from the perf plate 30 April, nine were removed 2 May, and six were removed 4 May.
5.2 One steelhead smolt was found on the grating at 11A-S on 3 May.  The jet of water from the orifice dislodged the grating.  11A-S will remain closed until the grating is replaced.
5.3 University of Idaho reported the death of an adult spring chinook (clipped) at the AFF on 4 May.  U of I biologists had radiotagged the fish and left it to recover from anesthesia.  The fish never recovered, dying after nearly an hour of resuscitative efforts.
5.4 The high outfall hydrocannons for the Second Powerhouse monitoring facility remain in service.
5.5 Though Unit 15 is only being used for FGE/OPE testing, the orifices are still open.  
5.6 Throughout the week there were reports of seals or sea lions in the tailraces of both powerhouses.  The Project did not take any action towards removing the natural predators from the river system.
                                                                                                                GENE L. LADOUCEUR

                                 

                                                             Operations Project Manager

//S//

2
3

