 
CENWP-OP-B                                                                                                                          25 November, 2001

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 47 of 2001, which covers the period 18 through 24 November.
1.  OPERATION SUMMARY.

1.1 Daily river flow averaged 104.4 to 117.2kcfs.  Daily project tailwater elevation averaged 9.5’ to 11.7’ msl.  Daily project forebay elevation averaged 74.4’ to 75.2’ msl.  River temperature averaged 51 to 53° F.
1.2 Spill Bay 18 was closed at 1330 on 31 October for spill deflector work.  Bay 1 was opened 8” to provide additional adult attraction flow.  This translates into a daily spill average of 2.5kcfs.  
1.3 Units 3 and 5 remain out of service for turbine rehab.  Unit 8 went out of service at 0818 on 19 November for a five-year overhaul.  Unit 16 was forced out of service at 0736 on 21 November due to a governor leak.  The unit went only to tailwater. 
1.4 Second Powerhouse fish units remain in service.  They provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly, if drawdown exceeds 1.0’.
2.  ACTIVITIES. 
2.1 AWS Closures.   AWS valves were closed for trashrack cleaning as listed below.  

19 November- FV3-7 from 1036 until 1100





 FV3-9 from 1036 until 1100

21 November- FV1-1 from 1331 until 1345

 FV6-9 from 1500 until 1515

2.2 STS/VBS Inspections.  No STSs were video inspected this reporting week.

2.3 STS removal. PH1 STS remain OOS per NMFS request.

2.4 Dewatering and Fish Salvage.  Unit 8 was dewatered on 20 November, no fish were found.

3.  RESEARCH.

3.1 WDFW Fish Counting.  24-hour video counts ongoing at BI and WA Shore ladders.

3.2 NMFS Saltwater rearing. The Saltwater rearing facility is in operation.  Sampling May- November.

3.3 PSMFC Adult PIT.  Planning for construction at the Bradford Island and Cascade Island ladders continues.

3.4 WDFW Winter Steelhead tagging.  Tagging began 05 November.

4.  FISHWAYS. 
4.1 Project biologists inspected 18-24 November this reporting week.

4.2 Fish Passage Plan Observations.

DATE
LOCATion
FPP Deviation
CAUSE
RESPONSE
TR/WAR

All week
PH2 collection channel electronic velocity meter.
The velocity meter is not yet working.
Not fully installed.
Working on final installation.


All week
FG3-28
Out of service
Manually closed because of displaced diffuser valve cover.
New covers will be installed during winter maintenance.
20013904

All week
FG3-27
Not in correct position for TW elev.
FG3-27 is open to replace FG3-28.
See above.


All week
Spill bay 18
On seal
Spillway deflector installation.
Spill bay 1 opened 8”.


All week
Main Dam Avian wires
Wires removed.
Spillway deflector installation.
Grid to be replaced in Mid-April.


11/18-24
Weir 37
Reading >1.1’
 See misc. 5.3
See misc. 5.3


All week
PH1 PCC velocity
Velocity meter registers <1.5 fps.
Water backs up in the channel.  
Monitoring situation.


11/19, 11/22
Bradford Island fishway staff gauges
Reading <1.0’
Unknown
Operators informed


4.3 Fish Ladder Temperatures.  Temperatures are measured 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.

Date
Wash. Shore
Bradford Is.

11/18
10.9
10.7

11/19
10.7
10.6

11/20
10.5
10.3

11/21
10.5
10.3

11/22
10.3
10.2

11/23
9.9
9.4

11/24
9.9
9.8

4.4 Sensor Calibration.  Water level sensors were checked for calibration on 24 November. 

4.5 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week. 

5. MISCELLANEOUS

5.1 Because of low tailwater elevations and the physical limits of the weir gates, entrance openings for PH1 Collection Channel are less than 8.0’.   Gate 64 has been reprogrammed to maintain at least an 8’ opening.  

5.2 The B-Branch and Cascade Island computers were installed.  Until sensors have been calibrated, entrance gates, diffusers, valves FV4-3, FV4-4, FV5-3, FV5-4 are operated in manual.

5.3 Weir 37 float control doesn’t adequately control water level over the weir.  A sonic transducer will be installed this winter, which should correct the problem.  

5.4 To aid spawning Chum below Bonneville, a tailwater restriction of 11.0’ to 11.2’ went into effect at 0600 on 20 November.  The restriction will continue until further notice.

5.5 The lower portion of Washington Shore ladder will be dewatered 3-7 December because of Adult PIT tag antenna problems.  All other ladder dewaterings have been postponed by one week.  The AFF will be dewatered the week of 25 November.   The Washington Shore ladder will be dewatered again in February.  

 DEBORAH L. CHENOWETH

                                 

                                                             Operations Project Manager

//S//

1
2

