
CENWP-OP-B                                                                                                                          31 December, 2001

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 52 of 2001, which covers the period 23 through 29 December.
1.  OPERATION SUMMARY.

1.1 Daily river flow averaged 116.1 to 131.1kcfs.  Daily project tailwater elevation averaged 11.5’ to 13.2’ msl.  Daily project forebay elevation averaged 72.8’ to 73.7’ msl.  River temperature averaged 44 to 48° F.
1.2 Spill Bay 18 and 1 are closed for spill deflector work.  
1.3 Units 3 and 5 remain out of service for turbine rehab.  
1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly, if drawdown exceeds 1.0’.  F1 remains out of service for winter maintenance.  F2 went to standby from as little as 3 hours to as many as 9 hours throughout the week to facilitate trashrack raking.  (see Activities 2.1)    The ice and trash chute was used to augment entrance attraction flow at PH2 until 27 December.  As coordinated with NMFS, F2 is supplying all attraction flow at this time. (see Misc. 5.6)
2.  ACTIVITIES. 
2.1 AWS Closures. Bradford Island AWS valves closed for winter maintenance.

23 December- FV5-9, FV6-9 and Weir 37 were closed for cleaning and;

F2 and Unit 18 went to standby for trashrack raking from 1250 until 1800.

26 December- FV5-9 and FV6-9 closed for cleaning from 0750 until 0805.

26 December- F2 trashrack raking from 0710 until 1630.

27 December- F2 trashrack raking from 1400 until 1700.

27 December- FV6-9 closed for cleaning from 1510 until 1530.

28 December- F2 and Unit 18 trashrack raking from 0724 until 1018.

28 December- FV6-9 and Washington Shore Exit cleaned from 0900 until 1000

29 December- FV5-9 closed for cleaning from 0830 until 0900.

2.2 STS/VBS Inspections.  All STSs have been removed for winter maintenance.

2.3 Dewatering and Fish Salvage.  No dewatering or fish salvage activity this reporting week.

3.  RESEARCH.

3.1 WDFW Fish Counting.  24-hour video count ongoing at the WA Shore ladder.

3.2 NMFS Saltwater rearing.  Shut down for the winter.  Clean up activities continue.

3.3 PSMFC Adult PIT.  Installation underway at Bradford Island ladder.

3.4  WDFW Winter Steelhead tagging.  Trapping continues three or four days a week.

3.5 NMFS Lamprey.  Tracking continues.  Plans for a trap in FV3-9 AWS channel underway.  More lamprey plates to be installed in the Washington Shore ladder in February.

4.  FISHWAYS. 
4.1 Project biologists inspected 26 through 28 December this reporting week.

4.2 Fish Passage Plan Observations.

DATE
LOCATion
FPP Deviation
CAUSE
RESPONSE
TR/WAR

All week
PH2 electronic velocity meter.
Meter not yet working
Need new cable.
Will be installed in February.


12/27, 12/28
Weir 37
Reading >1.0’
Faulty sonic transducer.
 See misc. 5.2.


All week
DSM2 channel
Channel elevation <64.2’
Low forebay level.
Add-in valve open 100%.


12/26, 12/28
Weir 38
Reading > or <1.0’
FV6-9 not controlling properly.
Trashracks raked and AWS valves reset to auto.


4.3 Fish Ladder Temperatures.  Temperatures are measured 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are not recorded during winter maintenance season.

4.4 Sensor Calibration.  Calibration was not checked as there were no sensors in service this reporting week.

4.5 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week. 

5. MISCELLANEOUS

5.1 The B-Branch and Cascade Island computers were installed.  Until sensors have been calibrated, entrance gates, diffusers, valves FV4-3, FV4-4, FV5-3, FV5-4 are operated in manual.

5.2 Weir 37 float control doesn’t adequately control water level over the weir.  A sonic transducer will be installed this winter, which should correct the problem.  

5.3 To aid spawning Chum below Bonneville, a tailwater restriction of 11.4’ to 11.6’ went into effect at 1054 on 28 November.  The restriction will continue until further notice.

5.4 Because of low forebay and DSM channel elevations, the DSM2 Add-in water was open in auto from 1000 until 2000 on 24 December, from 0630 on 26 December until 2000 on 28 December, and at 0415 on 29 December. The perf plate becomes clogged with debris on a daily basis, debris and impinged fish are brushed off and deposited into the channel.  Fisheries personnel have removed 1 dead smolt on 26 December, 13 dead juvenile shad on 27 December and 6 dead juvenile shad on 28 December.

5.5 PH2 ice and trash chute was flushed at 1020 on 27 December.  Twenty-two adult salmonids were flushed out in good condition.  As coordinated with NMFS, the ITC will not be used for AWS water until after the January FPOM meeting, where the issue of debris and fish collection/delay will be discussed.  Until those issues are resolved, F2 is being operated to maintain a 1.5’ differential.  In an effort to reduce the debris load during daylight hours, the fish unit is put on standby from 1800 until 0500 to float trash. 

 DEBORAH L. CHENOWETH

                                 

                                                             Operations Project Manager

//S//

1
2

