
CENWP-OP-B                                                                                                                           25 February, 2002

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 08 of 2002, which covers the period 17 through 23 February 2002.
1.  OPERATION SUMMARY.

1.1 Daily river flow averaged 119.1 to 149.3kcfs.  Daily project tailwater elevation averaged 11.6’ to 13.8’ msl.  Daily project forebay elevation averaged 74.3’ to 74.8’ msl.  River temperature averaged 44 to 45° F.  Daily turbidity averaged 5.5 to  >7’ on the secchi disk.

1.2 Spill Bays 18 and 1 are closed for spill deflector work.  

1.3 Unit 3 remains out of service for governor rehab.  Unit 5 remains out of service for turbine rehab.  Unit 17 remains out of service for VBS slot modification work.  

Units out of service for trash rack raking: 

19 February- Unit 1 (0724- 1406) and Unit 2 (1406- 1642)

20 February- Unit 2 (0654-l 0848), Unit 4 (1206- 1348) and Unit 6 (1348- 1548)

21 February- Unit 10 (0736- 1300), Unit 9 (1300- 1500), Unit 8 (1500- 1524) and Unit 7 (1524- 1636)

Units out of service for  research transducer installation:

22 February- Unit 1 (0718- 1142) and Unit 2 (1142- 1554)

23 February – Unit 4 (0636- 0936), Unit 6 (0936- 1212), and Unit 7 (1212- 1606) 

1.4 Second Powerhouse Fish Units 1 and 2 remain out of service for winter maintenance.
1.5 Gatewell Drawdown was measured on 20 February.  All units were within criteria.
2.  ACTIVITIES. 
2.1 AWS Closures. 

All Cascade Island and Washington Shore AWS valves are closed for winter maintenance.

18 February- FV1-1 was closed for cleaning from 0641 until 0705.

18 February- FV3-9 was closed for cleaning from 0712 until 0730.

2.2 STS/VBS Inspections.  All STSs have been removed for winter maintenance.

Dewatering and Fish Salvage.   Two lamprey were removed from the dewatered Washington Shore fishway on 21 February.  The fish had apparently come out from behind the count station crowder.  A shallow, muddy pool remains in this dark area after dewatering.  It is difficult to find all the fish.  Both were in good condition and were released to the forebay.

3.  RESEARCH.

3.1 WDFW Fish Counting.  24-hour video count ongoing at the Bradford Island ladder.

3.2 PSMFC Adult PIT.  Installation underway at Cascade Island ladder.  Non in-water work ongoing at Bradford Island. 

3.3 PSMFC/NMFS Juvenile PIT.  The flat plate detector was returned to DSM1 on 20 February. 

3.4 WDFW Winter Steelhead tagging.  Done until Washington Shore comes back in service.

3.5 NMFS Lamprey.  Tracking continues. More lamprey plates to be installed in the Washington Shore ladder.

4.  FISHWAYS. 
4.1 Project biologists inspected 19-21 February this reporting week.

4.2 Fish Passage Plan Observations.

DATE
LOCATion
FPP Deviation
CAUSE
RESPONSE
TR/WAR

All week
PH1 PCC velocity
Velocity <1.5fps on the electronic meter.
Unknown



All week
Calibration checking
Project Biologists did not check calibration of the fishways.
Shorthanded for the week.



4.3 Fish Ladder Temperatures.  Temperatures are measured 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperature will be recorded beginning 01 March.

4.4 Sensor Calibration.  Calibration was not checked this reporting week.

4.5 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week. 

5. MISCELLANEOUS

5.1 The B-Branch and Cascade Island computers were installed, though we still wait on brackets for the transducers.  Until brackets are installed, FV4-3, FV4-4, FV5-3, FV5-4 are operated in manual.

5.2 To aid spawning Chum below Bonneville, a minimum tailwater restriction of 11.4’ went into effect at 1054 on 28 November.  The restriction will continue until further notice.

5.3 PH1 ITS was closed from 1145 until 1530 on 17 February to aid in the USCG inspection of the forebay and tailrace.

5.4 Divers inspected Cascades Island fishway entrance and Washington Shore collection channel on 19 February.  Diffuser valve cover FG6-15 was blown off.  The ladder will be completely dewatered next week to repair the cover.  Washington Shore collection channel diffuser gratings were intact.  Divers carefully inspected floor gratings in the triangle area.  All gratings were intact.  The triangle area pickets will be left out this year (as coordinated with NMFS).

5.5 A beaver was found in Washington Shore ladder on 19 February.  It seemed unable to find its way back to the river.  A Ranger repeatedly tossed a rescue throw bag behind the beaver forcing it to swim back to the tailrace.  

 DEBORAH L. CHENOWETH

                                 

                                                             Operations Project Manager

//S//

1
2

