
CENWP-OP-B                                                                                                                                  24 March, 2002

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 12 of 2002, which covers the period 17 through 23 March 2002.
1.  OPERATION SUMMARY.

1.1 Daily river flow averaged 121.0 to 144.7kcfs.  Daily project tailwater elevation averaged 11.7’ to 13.1’ msl.  Daily project forebay elevation averaged 75.3’ to 75.7’ msl.  River temperature averaged 45 to 46° F.  Daily turbidity averaged 5.0 to 6.8’ on the secchi disk.

1.2 Spill Bays 18 and 1 remain closed for hoist work.  They are scheduled to be back in service by 25 March. Daily spill averaged 0.0kcfs.  
1.3 Unit 5 remains out of service for turbine rehab.  Unit 17 remains out of service for VBS slot modification work.  Unit 7 went out of service at 0400 on 18 march for governor rehab and exciter installation.  Unit 10 returned to service at 1326 on 21 March.
1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly, if drawdown exceeds 1.0’.   Both fish units were out of service from 0654 until 1212 on 20 March for the North Monolith dive.
1.5 Gatewell drawdown was measured on 20 March.  All units were within criteria.
2.  ACTIVITIES. 
2.1 AWS Closures.  No AWS closures.

2.2  STS/VBS Inspections.  No STS inspections this reporting week.

2.3  Dewatering and Fish Salvage.  No dewatering or fish salvage activities for this reporting week.

3.  RESEARCH.

3.1 WDFW Fish Counting.  24-hour video count ongoing at Bradford Island and Washington Shore ladders.

3.2 PSMFC Adult PIT.  Waiting for the electronics office.

3.3 PSMFC/NMFS Juvenile PIT.  Monitoring continues. 

3.4 WDFW Winter Steelhead tagging.  Trapping and tagging of steelhead continues.

3.5 NMFS Lamprey.  Tracking continues. 

3.6 PSMFC JMF.  Smolt monitoring continues.

3.7 NMFS Saltwater rearing.  Set up underway, start up is expected for April.

4.  FISHWAYS. 
4.1 Project biologists inspected 18-23 March this reporting week.

4.2 Fish Passage Plan Observations.

DATE
LOCATion
FPP Deviation
CAUSE
RESPONSE
TR/WAR

3/18-19, 3/21-22
PH1 PCC velocity
Velocity <1.5fps on the electronic meter.
Unknown- the rest of the ladder is in criteria.
No action at this time.


3/17-21, 3/23
PH2 PCC velocity meter.
U-sonic failure.
Transducer is installed, now need to be calibrated.



3/17

Biologists did not inspect 7 days/ week.
Personnel shortage.
Job advertised.


3/18-22
Weir 37
Reading > or < 1.0’
A dampener needs to be installed in the stillwell.
Electricians will fix during 20 March dive.


3/20
Washington shore fishway
Most everything
Dive in PCC
Back in criteria by 1200


3/21
Weir 38
Reading > or < 1.0’
Unknown
Operators looking into it.


4.3 Fish Ladder Temperatures.  Temperatures are measured 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.   Temperatures are in Celsius.

Date
Wash. Shore
Bradford Is.

3/17
5.4
5.5

3/18
5.4
5.5

3/19
5.4
5.4

3/20
5.6
5.6

3/21
5.5
5.5

3/22
5.3
5.3

3/23
5.6
5.6

4.4 Sensor Calibration.  Calibration was checked on 21 March this reporting week.

4.5 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week. 

5. MISCELLANEOUS

5.1 To aid spawning Chum below Bonneville, a minimum tailwater restriction of 11.4’ went into effect at 1054 on 28 November.  The restriction will continue until further notice.

5.2 FV4-3, FV4-4, FV5-3, and FV5-4 sensor brackets are finished and waiting to be installed.  Until they are installed, the valves are operated in manual.

5.3 On 20 March, divers installed the velocity meter at PH2.  

5.4 The monitoring facility went to bypass from 0700 on 21 March until 1630 on 22 March.  The wedge-wire at the PDS was re-oriented.  For the first minute of being watered back up, it sounded as if the replacement was a success, then the humming started.  PSMFC will monitor the noise over the weekend and a plan of action will be decided early next week.  

 DEBORAH L. CHENOWETH

                                 

                                                             Operations Project Manager

//S//

1
2

