
CENWP-OP-B                                                                                                                                  05 May, 2002

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 18 of 2002, which covers the period 28 April through 04 May 2002.
1.  OPERATION SUMMARY.

1.1 Daily river flow averaged 199.3 to 251.2kcfs.  Daily project tailwater elevation averaged 17.2’ to 19.5’ msl.  Daily project forebay elevation averaged 75.3’ to 76.0’ msl.  River temperature averaged 48 to 49° F.  Daily turbidity measured 3.0 to 4.5’ on the secchi disk.

1.2 Spill Bays 1and 18, which supply attraction flow for Cascades Island and Bradford Island ladders respectively, are operational and included in the spill pattern.  Spill averaged 90.2 to 139.0kcfs for the week.  

1.3 Unit 0 was out of service from 0400 until 1230 on 29 April for maintenance work.  Unit 1 returned to service at 1400 on 26 April.  Unit 5 remains out of service for turbine rehab.  Unit 8 remains out of service for digital governor and exciter work.  Unit 14 was out of service from 0206 on 28 April until 1512 on 29 April due to a breaker problem.  Unit 17 was out of service for FGE testing from about 1700 until 2200 on 30 April.

1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.  

1.5 Gatewell drawdowns were measured on 01 May.

2.  ACTIVITIES. 
2.1 AWS Closures.  There were no AWS closures this reporting week.

2.2 STS/VBS Inspections.  There were no STS/VBS inspections this reporting week.

2.3 Dewatering and Fish Salvage.  The JMF went to bypass from 0700 to 1415 on 29 April.   The dewatering facilitated removal of PVC sleeves that were too wide.

3.  RESEARCH.

3.1 CRITFC Age and Length Composition.  Collected 180 Spring Chinook this reporting week.

3.2 NMFS FGE.  Unit 17 FGE study is underway.  

3.3 NMFS Lamprey.  Tracking continues.   Collection begins in June.

3.4 NMFS Saltwater Rearing.  200 yearling Chinook are collected from barges every other day, equaling 1109 fish to date.  Half are introduced to the seawater for long term rearing; the other half will be transported to Newport for the Vibrio challenge on 7 May.  PIT tagged Rapid River Hatchery fish have been detected.  Those fish that have passed through only Bonneville’s JBS or Bonneville and 2-5 other JBS’s will rear in seawater, all others will be released back to the river. 

3.5 Normandeau Balloon-tag at Spillway.  Resheduled for 20 May – mid June and 1-15 August.

3.6 OCFWRU Transportation Study.  Study of post-transportation migration and survival of juveniles to run 29 April through 14 July.

3.7 ODFW Summer Steelhead sampling.  Sampling 2 days/week 8 hours/day.  Will sample 1% of the run.

3.8 ODFW Pikeminnow Electroshocking.  Part of northern pikeminnow management program.  Capturing and collecting biological data from pikeminnow, smallmouth bass, and walleye in Bonneville’s BRZ.  Electroshocking 1-5 April and 22-26 April.

3.9 Pelfrey Sturgeon Hatchery.  Scheduled to begin capturing broodstock in late May.

3.10 PSMFC Adult PIT.  Detectors are installed in all ladders.  Some of the detectors have developed leaks and will be monitored throughout the season.  Repairs are scheduled for 2002-03 winter maintenance season.

3.11 PSMFC JMF.  Debris loads were light this week.  The following fish were found stranded on the separator bars: 2 smolts and 2 kelts (released alive).  The PVC sleeves were on for two days in this report period, during which time 609 smolt were lost.  For the other 5 days without the bars, only 274 smolt were lost.

3.12 PSMFC/NMFS Juvenile PIT.  Monitoring and sampling continue.

3.13 U of I Transportation Study.  Evaluation of physiological condition of river run v. barged Snake River juveniles to occur May-June.  May add separation–by-code study June-August.

3.14 U of I Adult Radio tracking.  Trapping began 1 April and will continue 7 days/week until October.

3.15 USGS Juvenile Radio tracking.  Study in progress.

3.16 USGS MGR.  Equipment installation continues.  

3.17 USGS EMG studies.  Tissue samples will be taken from 200 Chinook starting 1 April through June.

3.18 USGS Lamprey.  Collection begins on 6 May.

3.19 USGS Sort by Code.  Length and weight are recorded and fish are released back to the river.

3.20 WDFW Fish Counting.  16-hr visual and 8-hr video counts at Bradford Island and Washington Shore.

3.21 WDFW Jaw tagging.  Tagging 50-75 fish per week.

3.22 WDFW Winter Steelhead Tagging.  Trapping and tagging of steelhead continues once a week.

3.23 WES Spillway Deflector Testing.  WES will be monitoring water quality equipment to determine effectiveness of new flip lips.  Study began with initiation of spill on 10 April.

3.24 WES/Battelle/Mevatech Juvenile Hydroacoustics.  Study in progress.  

4.  FISHWAYS 
4.1 Project biologists inspected 29-30 April and 4 May this reporting week.

4.2 Fish Passage Plan Observations.

	DATE
	LOCATion
	FPP Deviation
	CAUSE
	RESPONSE
	TR/WAR

	All week
	PH1 PCC velocity
	Velocity <1.5fps on the electronic meter.
	Unknown- the rest of the ladder is in criteria.
	No action at this time.
	

	4/28, 5/1-3
	
	Biologists did not inspect 7 days/ week.
	Personnel shortage.
	Job advertised.
	

	4/29
	PDS
	To bypass
	Mods to separator bars.
	See misc. 5.3
	

	5/4
	B-Branch
	Diffusers not in proper position for TW elevation.
	Computer system not yet fully functioning.
	Operators informed, adjusted diffusers.
	#6474


4.3 Fish Ladder Temperatures.  Temperatures are measured 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.   The temperature is recorded every 15 seconds, averaged for each hour then averaged for the entire day.  Temperatures are in Fahrenheit.  

	Date
	Wash. Shore
	Bradford Is.

	4/21
	50.0
	49.8

	4/22
	50.0
	50.0

	4/23
	49.8
	49.8

	4/24
	49.6
	49.6

	4/25
	49.8
	49.8

	4/26
	49.6
	49.6

	4/27
	49.8
	49.8

	4/28
	50.0
	50.2

	4/29
	50.4
	50.2

	4/30
	50.5
	50.5

	5/1
	50.9
	50.9

	5/2
	51.6
	50.9

	5/3
	50.9
	50.7

	5/4
	50.7
	50.9


4.4 Sensor Calibration.  Calibration was checked on 04 May this reporting week.  Everything is within criteria.

4.5 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week. 

5. MISCELLANEOUS

5.1 FV4-3, FV4-4, FV5-3, and FV5-4 are operated in manual.  Calibration is mostly completed.  

5.2 River temperature is the temperature of the water in the scroll case of Unit 2.  This temperature is recorded every 6 hours (the FPP requires a reading three times a day).  

5.3 The PDS was in bypass from 0730 until 1430 on 29 April for separator bar modifications.  In an effort to reduce possible stranding of kelts and other adults, PVC sleeves were installed to reduce the gap between bars earlier this year.  This mod had the desired result of fewer kelts being stranded but it also resulted in too many juvenile salmonids sliding over the top of the bars instead of falling through to the hopper and on to PIT tag interrogation and monitoring.  BON agreed to remove the PVC sleeves and PSMFC agreed to monitor closely for stranded kelts and other adults.  

5.4 Turbulent flows from spillbays 1 and 18 enter the side entrance gates of Cascade Island and Bradford Island B-branch ladders, causing poor entrance conditions.  District hydraulic engineers are investigating the problem.  Riggers installed a bulkhead at Cascade Island on 03 May.

5.5 Sea lions and harbor seals have been observed in the tailrace of both powerhouses and the spillway.  No action is being taken by the project to harass, harm, or otherwise annoy these animals.  FFU is monitoring and recording their activity.

5.6 On 24 April, Hatchery Well H2 failed.  The problem has been turned over to District engineers.  ODFW may need to supplement existing well water supplies with water from Tanner Creek.

 DEBORAH L. CHENOWETH

                                 

                                                             Operations Project Manager

//S//

1
3

