
CENWP-OP-B                                                                                                                                  01 July, 2002

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 26 of 2002, which covers the period 23 through 29 June 2002.

1.  OPERATION SUMMARY.

1.1 Daily river flow averaged 288.5 to 356.4kcfs.  Daily project tailwater elevation averaged 21.9’ to 25.7’ msl.  Daily project forebay elevation averaged 75.5’ to 75.9’ msl.  River temperature averaged 59 to 61° F.  Daily turbidity measured 3.5’ to 5.0’ on the secchi disk.
1.2 Spill Bays 1 and 18, which supply attraction flow for Cascades Island and Bradford Island ladders respectively, are operational and included in the spill pattern.  Spill averaged 93.1 to 167.0kcfs.  

1.3 Unit Operation
1.3.1 Unit 0 (station service) was out of service from 0506 on 25 June until 1612 on 27 June to facilitate cutting the slip rings.

1.3.2 Unit 1 remains out of service for turbine rehab.

1.3.3 Unit 5 remains out of service for turbine rehab.  

1.3.4 Unit 6 remains out of service for governor modifications.

1.3.5 Unit 8 was out of service from 0930 until 1112 on 27 June for replacement of an exciter fan.

1.3.6 Unit 9 was out of service from 0648 until 0842 on 25 June for shaft packing adjustments and from 1306 until 1618 on 26 June for exciter part replacement.

1.3.7 Unit 17 was out of service for FGE testing from about 1600 until 2100 from 24-29 June.  VBSs were cleaned during swing shift on FGE test nights.

1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.  F1 was out of service from 0818 until 1206 on 23 June due to turbine bearing water contamination.
1.5 Gatewell drawdowns were measured every day for Unit 17 and on 26 June for the rest of the units.

2.  ACTIVITIES. 
2.1 AWS Closures.  The following AWS valves were closed for cleaning this reporting week.

24 June-
FV3-7 from 1515 until 1631

26 June-
Weir 37 from 1515 until 1530

28 June-
FV3-9 from 1610 until 1630, FV1-1from 1630 until 1640, FV5-9 from 1810 until 1830

2.2 STS/VBS Inspections.  No STS/VBSs were inspected this reporting week.

2.3 Dewatering and Fish Salvage.  DSM1 was dewatered on 25 June to facilitate repairs to the small trash sweep.  No fish were recovered.

3.  RESEARCH.

3.1 CRITFC Age and Length Composition.  Collected 100 Chinook and 150 Sockeye this reporting week.

3.2 NMFS FGE.  Unit 17 FGE study is ongoing.  Summer tests are expected to be complete by 12 July.

3.3 NMFS Lamprey.  Fish are being collected for NMFS, UI, and the Warm Springs tribe.  The lamprey flume was installed in the Bradford Island AWS channel.  Testing began 26 June.

3.4 NMFS Saltwater Rearing.  Collection and rearing continue. 

3.5 Normandeau Balloon-tag at Spillway.  Low flow study commences in August.

3.6 OCFWRU Transportation Study.  Study of post-transportation migration and survival of juveniles to run 29 April through 14 July.

3.7 ODFW Summer Steelhead sampling.  Sampling 2 days/week 8 hours/day.  Will sample 1% of the run.

3.8 PSMFC Adult PIT.  Detectors are installed in all ladders.  Some of the detectors have developed leaks and will be monitored throughout the season.  Repairs are scheduled for 2002-03 winter maintenance season.

3.9 PSMFC JMF.  No kelts were found stranded on the separator bars.  Sliding over the bars were 71 Steelhead, 21 salmon and 1 unknown salmonid reported as fallbacks and 117 smolts.  Sampling was interrupted on 29 June for DSM2 screen cleaner #2 repair (See misc. 5.4).

3.10 PSMFC/NMFS Juvenile PIT.  200 sub yearling Chinook were examined for signs of GBT.  Total descaling for all fish sampled at PH1 was 4.9%, down from 6.7% last week.  The Flat Plate Pit Tag detection system in powerhouse 1 was out of service from 2040 on 24 June through 1435 on 25 June to repair the small dewatering screen’s trash sweep (See misc. 5.5).

3.11 U of I Adult Radio tracking.  Trapping began 1 April and will continue 7 days/week until October.

3.12 U of I Transportation Study.  Evaluation of physiological condition of river run v. barged Snake River juveniles to occur June-September.

3.13 USGS Juvenile Radio tracking.  Study in progress.

3.14 WDFW Fish Counting.  16-hr visual and 8-hr video counts at Bradford Island and Washington Shore.

3.15 WES/Battelle/Mevatech Juvenile Hydroacoustics.  Study in progress.  

3.16 WES Spillway Deflector Testing.  WES monitoring water quality equipment to determine effectiveness of new flip lips.  Study began with initiation of spill on 10 April.

4.  FISHWAYS 
4.1 Project biologists inspected 25-28 June this reporting week.

4.2 Fish Passage Plan Observations.

	DATE
	LOCATion
	FPP Deviation
	CAUSE
	RESPONSE
	TR/WAR

	All week
	PH1 PCC velocity
	Velocity <1.5fps on the electronic meter.
	Unknown
	No action at this time.
	

	6/23, 6/29
	
	Biologists did not inspect 7 days/ week.
	Personnel shortage.
	Training a new employee.
	

	6/25, 6/27
	Weir 37
	Reading <1.3’
	Screen clogged
	Screen cleaned.
	


4.3 Fish Ladder Temperatures.  Temperatures are measured 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Rankine this week.  Rankine is an absolute temperature scale named after Scottish engineer and physicist William John Macquorn Rankine.  For conversion information, please see last week’s report.

	Date
	Wash. Shore
	Bradford Is.

	6/23
	521.55
	521.55

	6/24
	521.73
	521.73

	6/25
	521.73
	521.91

	6/26
	522.27
	522.27

	6/27
	522.45
	522.45

	6/28
	522.45
	522.45

	6/29
	522.45
	522.45


4.4 Sensor Calibration.  Calibration was checked on 26 June this reporting week.  

4.5 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week. 

5. MISCELLANEOUS

5.1 B-branch and Cascade Island are operated in manual mode until stillwells can be installed for the tailwater sensors.  Installation will wait until spill has concluded.

5.2 Turbulent flows from spillbays 1 and 18 enter the side entrance gates of Cascade Island and Bradford Island B-branch ladders, causing poor entrance conditions.  District hydraulic engineers are investigating the problem.  On 3 May, the structural crew installed a bulkhead in the side entrance at Cascade Island.  Entrance conditions improved.  CG funds will be used to build a bulkhead for Bradford Island B-branch.

5.3 There has been some difficulty in maintaining an opening for fish at PH1 weir gate 1.  Electricians are exploring the possibility of reprogramming the gate such that it will open if there is less than a 6’ opening and not close if less than 8’.

5.4 DSM2 screen cleaner #2 was out of service from 1058 until 1159 on 24 June due to a broken chain.  At 1940 on 28 June, screen cleaner #2 again broke a brush arm chain.  All three screen cleaners were placed in local control.  Mechanics came in on 29 June to do repairs.  From 0720 until 0820 the ERG was tripped and the Add-in water closed while mechanics tried to find the brush arm chain, believed to be in the channel.  By 1410 the screen cleaner was repaired and all systems were back in auto.

5.5 The DSM1 small trash sweep chain jumped a gear tooth at 2040 on 24 June.  The DSM was turned south from 0720 until 1432 on 25 June to facilitate repairs.

5.6 On the evening of 29 June, JMF personnel reported a problem with the three way rotating gate.  The gate was sticking in the sample position resulting in several thousand fish being diverted into the PIT tag holding tanks.  A work request was submitted to the mechanics in the morning of 01 July.

 DEBORAH L. CHENOWETH

                                 

                                                             Operations Project Manager

//S//

1
3

