
CENWP-OP-B                                                                                                                                  05 August 2002

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 31 of 2002, which covers the period 28 July through 03 August 2002.

1.  OPERATION SUMMARY.

1.1 Daily river flow averaged 162.0 to 198.1kcfs.  Daily project tailwater elevation averaged 13.9’ to 16.0’ msl.  Daily project forebay elevation averaged 74.2’ to 75.6’ msl.  River temperature averaged 67° F.  Daily turbidity measured 3.0’ to 7.0’ on the secchi disk.

1.2 Spill Bays 1 and 18, which supply attraction flow for Cascades Island and Bradford Island ladders respectively, are operational and included in the spill pattern.  Spill averaged 95.1 to 138.6kcfs.  

1.3 Unit Operation

1.3.1 Unit 1 remains out of service for turbine rehab.

1.3.2 Unit 5 remains out of service for turbine rehab.  

1.3.3 Unit 6 was out of service from 0430 on 29 July until 1624 on 31 July and from 1630 until 1648 on 01 August for Bank 5/6 switching.

1.3.4 Unit 7 remains out of service

1.3.5 Unit 8 was out of service from 0430 on 29 July until 1648 on 01 August for bank switching and for oil cleanup.

1.3.6 Unit 13 returned to service at 0924 on 30 July.

1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.  

1.5 Gatewell drawdowns were measured every day for Unit 17 and on 31 July for the rest of the units.

2.  ACTIVITIES. 
2.1 AWS Closures.  No AWS valves were closed.

2.2 STS/VBS Inspections.  No STS/VBSs were inspected this reporting week.

2.3 Dewatering and Fish Salvage.  DSM1 was turned south on 29 July after removal of priority unit screens.  No fish were recovered.

3.  RESEARCH.

3.1 CRITFC Age and Length Composition.  Collected 30 Chinook this reporting week.

3.2 NMFS Lamprey.  The lamprey flume is in operation at the Bradford Island AWS channel.  Collection and tracking continue.

3.3 NMFS Saltwater Rearing.  Mortality is increasing.  Specific data available from Lyle Gilbreath. 

3.4 Normandeau Balloon-tag at Spillway.  Low flow study commences 18 August.

3.5 ODFW Summer Steelhead sampling.  Sampling 2 days/week 8 hours/day.  Will sample 1% of the run.

3.6 PSMFC Adult PIT.  Detectors are installed in all ladders.  Some of the detectors have developed leaks and will be monitored throughout the season.  Repairs are scheduled for 2002-03 winter maintenance season.

3.7 PSMFC JMF.  Recorded as going over the bars were six steelhead, one salmon and one smolt.  

3.8 PSMFC/NMFS Juvenile PIT.  200 sub yearling Chinook were examined for GBT this week.  

3.9 U of I Adult Radio tracking.  Trapping began 1 April and will continue 7 days/week until October.

3.10 U of I Transportation Study.  Evaluation of physiological condition of river run v. barged Snake River juveniles to occur June-September.

3.11 USGS Juvenile Radio tracking.  Study in progress.

3.12 WDFW Fish Counting.  16-hr visual and 8-hr video counts at Bradford Island and Washington Shore.

3.13 WES/Battelle/Mevatech Juvenile Hydroacoustics.  Study in progress.  

3.14 WES Spillway Deflector Testing.  WES monitoring water quality equipment to determine effectiveness of new flip lips.  

3.15 Yakama Dam Angling.  YIN began angling for pikeminnows at Bonneville on 1 July.  

4.  FISHWAYS 
4.1 Project biologists inspected 28-31 July and 01, 03 August this reporting week.  NMFS inspected on 30 July.

4.2 Fish Passage Plan Observations.

	DATE
	LOCATion
	FPP Deviation
	CAUSE
	RESPONSE
	TR/WAR

	7/29-30, 8/1
	PH1 PCC velocity
	Velocity <1.5fps on the electronic meter.
	Unknown
	No action at this time.
	

	All week
	Main dam
	Two avian wires broken.
	Vandalism
	Will be repaired this winter maintenance season.
	

	8/02
	
	Biologists did not inspect 7 days/ week.
	Personnel shortage.
	Training a new employee.
	

	7/28-29
	PH2 PCC velocity
	Velocity <1.5fps on the electronic meter.
	Unknown
	TR submitted
	#8497

	8/3
	DSM2
	Screen cleaner #2 broken
	
	Mechanic called in.
	


4.3 Fish Ladder Temperatures.  Measurements are taken 3 weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Celsius.

	Date
	Wash. Shore
	Bradford Is.

	7/28
	20.2
	18.7

	7/29
	20.3
	18.5

	7/30
	20.7
	18.4

	7/31
	20.7
	17.8

	8/1
	20.7
	17.5

	8/2
	20.6
	17.0

	8/3
	20.4
	16.4


4.4 Sensor Calibration.  Calibration was not checked this week due to wind conditions.

4.5 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week. 

5. MISCELLANEOUS

5.1 B-branch and Cascade Island are in manual mode until tailwater stillwells can be installed

5.2 Turbulent flows from spillbay 18 enter the side entrance gate of Bradford Island B-branch ladder, causing poor entrance conditions.  CG funds will be used to build a bulkhead for B-branch.

5.3 DSM1 went south at 0940 on 29 July because screens were pulled on priority units last week in accordance with NMFS request.

5.4 Due to signs of infectious hematopoietic necrosis (IHN), the Bonneville Hatchery euthanized 197,000 summer steelhead fingerlings and about 2000 trout with MS222.  Euthanized fish will be frozen and sent to a landfill.  IHN is common in the Columbia River and attacks the blood forming tissues of the kidney.

 DEBORAH L. CHENOWETH

                                 

                                                             Operations Project Manager

//S//

1
2

