CENWP-OP-B                                                                                                                                  09 December 2002

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 49 of 2002, which covers the period 01 through 07 December 2002.  

1.  OPERATION SUMMARY.

1.1 Daily river flow averaged 121.4 to 126.4 kcfs.  Daily project tailwater elevation averaged 11.5’ to 11.7’ msl.  Daily project forebay elevation averaged 73.7’ to 75.0’ msl.  River temperature averaged 45 to 46° F.  Daily turbidity measured 4.8’ to 7.0’ on the secchi disk.

1.2 Spill Bays 1 and 18, which supply attraction flow for Cascades Island and Bradford Island ladders respectively, are now closed 24 hours.  Spill averaged 0.0 to 1.4 kcfs, the 1.4 came from the bays 1 and 18 being opened 6” from 0705 until 1640 on 02 December.

1.3 Unit Operation

1.3.1 Unit 1 remains out of service for turbine rehab.

1.3.2 Unit 2 was out of service from 0530 until 1348 on 04 December for new power supply installation.

1.3.3 Unit 7 returned to service at 1054 on 02 December.

1.3.4 Unit 8 returned to service at 1548 on 03 December.

1.3.5 Unit 9 returned to service at 1506 on 03 December.

1.3.6 Unit 10 returned to service at 1736 on 30 November. 

1.3.7 Units 11 and 12 were out of service for B2 corner collector dive operations from about 0600 until 1830 on 02-04 December, from 1136 until 1548 on 06 December, from 0706 until 1930 on 06 December and from 0700 until 2330 on 07 December.

1.3.8 Unit 15 was out of service from 0500 until 1130 on 05 December for DSM2 fyke net frame removal.

1.3.9 Unit 16 remains out of service for an overhaul.

1.3.10 Units 17 and 18 were out of service from 0500 until 1130 on 05 December for DSM2 fyke net frame removal.

1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.  Fish Unit 1 remains out of service for overhaul and stator rewind.

1.5 Gatewell drawdowns were measured on 04 December.

2.  ACTIVITIES. 
2.1 AWS Closures.  The following AWS valves were closed for cleaning.

01 December- FV6-9 closed at 1100 for Washington shore dewatering activities.

03 December- FV3-9 from 0835 until 0852

 FV3-7 from 0852 until 1027

 FV5-9 from 1008 until 1125

 FV1-1 from 1244 until 1425


      04 December- FV5-9 from 0816 until 0826







 FV3-9 from 0909 until 0923

      05 December- FV3-9 from 0724 until 0740





 FV1-1 from 0742 until 0815

      07 December- FV1-1 from 0752 until 0800

 FV3-9 from 0814 until 0830

2.2 STS/VBS Inspections.  No STS/VBSs were inspected this reporting week. 

2.3 Dewatering and Fish Salvage.  USGS and CoE biologists removed one crawfish, five sculpin, 40 live adult steelhead, one dead adult steelhead, and 20 resident fish from the UMT on 03 December.  200-300 lamprey, 12 sculpin and one steelhead was removed from the Washington Shore ladder on 03 December.  All fish were released upstream of the navlock.

3.  RESEARCH.

3.1 NMFS Lamprey.  Tracking continues.

3.2 NMFS Saltwater Rearing.  Rearing continues.  Specific data available from Lyle Gilbreath. 

3.3 WDFW Fish Counting.  24-hr video counts at Bradford Island and Cascade Island.

4.  FISHWAYS 
4.1 Project biologists inspected 02, 04-06 December this week.

4.2 Fish Passage Plan Observations.

	DATE
	LOCATion
	FPP Deviation
	CAUSE
	RESPONSE
	TR/WAR

	All week
	Main dam, 

PH1, PH2
	Avian wires broken or removed
	Vandalism and BCC work. 
	Will be repaired this winter.
	

	12/02, 12/04, 12/06
	PH1 PCC velocity
	Velocity <1.5 fps on electronic meter.
	Unknown
	No action taken.
	

	12/04
	A&B Branch
	Low water
	Unknown
	Operator to check, trashracks to be cleaned.
	

	
	
	Calibration not checked.
	
	
	


4.3 Fish Ladder Temperatures.  Temperatures are not recorded in the winter.

4.4 Sensor Calibration.  Calibration was not checked this reporting week.

4.5 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week. 

5. MISCELLANEOUS

5.1 B-branch and Cascade Island are in manual mode until tailwater stillwells can be installed this winter.

5.2 During low tailwater, the PH1 weir gates are unable to maintain greater than 8’ openings.

5.3 As of 13 November, and until further notice, the Project has a hard constraint tailwater elevation no less than 11.3’ msl during all hours and a soft constraint of 11.3’-11.7’msl between 0700-1900 hours.  This is for the protection of the chum redds around Ives Island.

5.4 Washington Shore went out of service at 1100 on 01 December.  Dewatering activities occurred on 03 December.

5.5 Cascade Island exit was opened at 0826 on 02 December.  Fish counting video was set up on 03 December.

5.6 Unit 8A ESBS was repaired and installed on 03 December.

5.7 On 6 December, PH1 avian wires were removed by USDA-WS to facilitate upcoming fyke net frame removal.

5.8 DSM1 was calibrated on 07 December.

 JAMES R. MAHAR

                                 

                                                           Operations Manager

//S//

1
2

