CENWP-OP-B                                                                                                                                     02 March 2003

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 09 of 2003, which covers the period 23 February through 01 March 2003.

1.  OPERATION SUMMARY.

1.1 Daily river flow averaged 119.9 to 134.0 kcfs.  Daily project tailwater elevation averaged 11.7’ to 12.4’ msl.  Daily project forebay elevation averaged 75.3’ to 75.9’ msl.  River temperature averaged 40 to 42° F.  Daily turbidity measured 4.0’ to 6.2’ on the secchi disk.

1.2 Spill Bays 1 and 18, which supply attraction flow for Cascade Island and Bradford Island fishways respectively, are open .5’ during daylight hours in accordance to the FPP.  Spill averaged .9 to 2.0 kcfs.

1.3 Unit Operation

1.3.1 Unit 0 was out of service from 0942 until 1324 to facilitate testing of Unit 1.

1.3.2 Unit 1 remains out of service for turbine rehab.

1.3.3 Unit 2 was out of service from 0736 on 24 February until 1418 on 25 February for servo shaft work.  Unit 2 was out of service from 1042 until 1252 on 26 February for STS installation.

1.3.4 Unit 7 was out of service from 1254 until 1542 on 26 February for STS installation.

1.3.5 Unit 8 was out of service from 0654 until 1136 on 27 February for ESBS installation.

1.3.6 Unit 9 was out of service from 0730 until 1042 on 26 February for STS installation.

1.3.7 Unit 10 was out of service from 1248 until 1442 on 27 February for STS installation.

1.3.8 Unit 11 returned to service at 1100 on 25 February.  Unit 11 was out of service from 0736 until 1236 on 28 February for dive operations.

1.3.9 Unit 12 was out of service from 0530 on 24 February until 1100 on 25 February and from 0736 until 1236 on 28 February for dive operations.

1.3.10 Unit 16 was out of service from 1048 until 1336 on 25 February for STS installation.

1.3.11 Unit 17 was out of service on 25 February from 0854 until 1048 for STS installation and from 1336 until 1424 for transducer installation.

1.3.12 Unit 18 was out of service from 0742 until 0854 on 25 February for STS installation.

1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  During fish passage season, the units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.  Fish Unit 1 remains out of service for overhaul and stator rewind.

1.5 Gatewell drawdowns are measured on Wednesdays.

2.  ACTIVITIES. 
2.1 AWS Closures.  No AWS closures to report this week.

2.2 STS/VBS Inspections.  STS/VBSs were installed this reporting week. 

	Unit
	Date
	Run Hours
	Comments

	2
	2/26
	
	installation

	3
	3/01
	
	Installation

	4
	3/01
	
	Installation

	5
	3/01
	
	Installation

	6
	3/01
	
	Installation

	7
	2/26
	
	Installation

	8
	2/27
	
	Installation

	9
	2/26
	
	installation

	10
	2/27
	
	Installation

	11
	2/28
	
	Installation

	12
	2/28
	
	Installation

	13
	2/28
	
	Installation

	14
	2/28
	
	Installation

	15
	2/28
	
	Installation

	16
	2/24
	
	Installation

	17
	2/25
	
	Installation

	18
	2/25
	
	Installation


Table 1.  STS Inspection Dates.

2.3 Dewatering and Fish Salvage.  No dewatering or salvage activities this week.

3.  RESEARCH.

3.1 University of Idaho/WDFW jaw tagging.  Steelhead and spring chinook will be tagged for studies involving the tooth tangle net fishery on the Columbia.

3.2 NMFS Lamprey.  Tracking continues.

3.3 WDFW Fish Counting.  16-hr video counts at Washington Shore.

3.4 WDFW steelhead.  Trapping and tagging of winter steelhead continues.

4.  FISHWAYS 
4.1 Project biologists inspected 25-27 February this week.

4.2 Fish Passage Plan Observations.

	DATE
	LOCATion
	FPP Deviation
	CAUSE
	RESPONSE
	TR/WAR

	All week
	PH1, PH2
	Wires removed.
	Contractor work.
	Will replace wires 4-6 March.  
	

	All week
	Main dam
	Two wires broken
	Vandalism
	See above
	

	All week
	PH2 velocity meter
	No velocity measure.
	Meter is missing
	see misc. 5.1
	


4.3 Fish Ladder Temperatures.  Temperatures are not recorded in the winter.

4.4 Sensor Calibration.  Calibration could not be checked as there were no calibrated sensors in operation.

4.5 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week. 

5. MISCELLANEOUS

5.1 Divers discovered the PH2 velocity meter was missing from the collection channel.  A new meter will be installed during the mid season dives this summer.

5.2 Cascade Island and B-Branch controls are in manual mode awaiting tailwater sensor calibration.  Electricians are awaiting parts. 

5.3 On 24 January, the Project tailwater hard constraint decreased to 11.0’ msl for all hours.  The soft constraint went to 11.2’ msl between 0700-1900 hours.  This is for the protection of the chum redds around Ives Island.  On 17 February, BPA agreed to a hard tailwater constraint of 11.3’ msl and a soft constraint of 11.5’ msl when inflows to Bonneville are >120Kcfs.

5.4 Bradford Island returned to service at 1645 on 26 February.

5.5 A control failure necessitated switching FV3-9 to manual from 1704 on 26 February until 1358 on 27 February.

5.6 Powerhouse 2 became the priority powerhouse on 01 March.

 JAMES R. MAHAR

                                 

                                                           Operations Manager

//S//

1
3

