CENWP-OP-B                                                                        

                                                      27 April 2003

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 17 of 2003, which covers the period 20 through 26 April 2003.
1.  OPERATION SUMMARY.

1.1 Daily river flow averaged 207.6 to 278.4 kcfs.  Daily project tailwater elevation averaged 17.3’ to 20.3’ msl.  Daily project forebay elevation averaged 73.9’ to 76.1’ msl.  River temperature averaged 48 to 50° F.  Daily turbidity measured 3.2’ to 5.0’ on the secchi disk.

1.2 Spill Bays 1 and 18, which supply attraction flow for Cascade Island and Bradford Island fishways respectively, are included in the spill pattern.  Spill averaged 101.5 to 142.4 kcfs.

1.3 Unit Operation

1.3.1 Unit 1 remains out of service for turbine rehab.

1.3.2 Unit 9 was out of service from 1054 until 1530 on 22 April to facilitate draining and filling the thrust bearings.

1.3.3 Unit 14 was out of service from 1230 on 23 April until 1336 on 26 April to facilitate removal of the power supply for Unit 15.

1.3.4 Unit 15 returned to service at 0742 on 24 April.  Unit 15 went out of service at 0718 on 26 April for governor work.  

1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  During fish passage season, the units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.  

1.5 Gatewell drawdowns are measured on Wednesdays.

2.  ACTIVITIES. 
2.1 AWS Closures.  No AWS valves were closed for cleaning this reporting week.

2.2 STS/VBS Inspections.  No STS/VBS inspections were conducted this reporting week.

2.3 Dewatering and Fish Salvage.  No dewatering or fish salvage activities this reporting week.

3.  RESEARCH.

3.1 CRITFC.  Sampling of chinook, sockeye, and coho continues until 31 October.

3.2 NMFS Lamprey.  Tracking continues once a month.  Tagging to begin the second week in May.

3.3 ODFW Pikeminnow Tagging.  Activities completed week of 14 April.  

3.4 ODFW Steelhead.  Trapping 1% of the summer steelhead continues two days a week.

3.5 PSMFC at DSM1.  Continuous PIT tag interrogation continues.  GBT and monitoring continue.

3.6 PSMFC at the SMF.  NO kelts were observed stuck on the separator bars.  Monitoring personnel observed 127 steelhead, 48 salmon, four unknown salmonids and 81 smolts slide over the separator bars.

3.7 Spring Creek NFH.  Scheduled to release 3.5 million fall chinook on 08 May.

3.8 University of Idaho.  Adult tracking tagging and tracking continue.

3.9 WDFW jaw tagging.  Tagging steelhead and spring chinook for tooth tangle net studies.

3.10  USGS Juvenile Radiotracking.  USGS still installing equipment around the project.

3.11 WDFW Fish Counting.  16-hr visual counts at Washington Shore and Bradford Island.

3.12 WES/Battelle/BAE systems Juvenile hydroacoustic monitoring.  Installing equipment.

4.  FISHWAYS 
4.1 Project biologists inspected 20-23, 25-26 April this week.  NOAA Fisheries inspected on 22 April.

4.2 Fish Passage Plan Observations.

	DATE
	LOCATion
	FPP Deviation
	CAUSE
	RESPONSE
	TR/WAR

	All week
	PH1 PCC
	Velocity <1.5fps
	Meter in slowest part of channel
	No response at this time.
	

	All week
	PH2 PCC
	No velocity measure.
	Meter is missing
	See misc. 5.1
	

	4/21
	A-Branch
	Staff gauge reading >1.0’
	Unknown
	No action taken.
	

	4/24
	Bonneville
	Biologists did not inspect 7 days/week.
	Personnel shortage
	
	

	All week
	Bonneville
	Biologists did not check calibration.
	Personnel shortage
	
	


4.3 Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Kelvin.  

	Date
	Wash. Shore
	Bradford Is.

	4/20
	283.6
	283.7

	4/21
	283.7
	283.7

	4/22
	283.8
	283.8

	4/23
	284.0
	284.0

	4/24
	283.9
	283.9

	4/25
	283.8
	283.8

	4/26
	283.7
	283.7


4.4 Sensor Calibration.  Calibration was not checked this week.

4.5  Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week. 

5. MISCELLANEOUS

5.1 Divers discovered the PH2 velocity meter was missing from the collection channel.  A new meter will be installed during the mid season dives this summer.

5.2 On 24 January, the Project tailwater hard constraint decreased to 11.0’ msl for all hours.  The soft constraint went to 11.2’ msl between 0700-1900 hours.  This is for the protection of the chum redds around Ives Island.  On 17 February, BPA agreed to a hard tailwater constraint of 11.3’ msl and a soft constraint of 11.5’ msl when inflows to Bonneville are >120Kcfs.

5.3 A request for a contract for a tug to remove debris from PH2 forebay has been made.  

5.4 On 22 and 24 April, researchers reported chinook mortalities at the AFF.  Fish seem to be attracted to the NW corner of the holding pool.  They jump into the netting until they create a hole and jump through.  Project Biologists are working on a permanent fix to the problem.

5.5 On 21 April the AFF bridge crane failed.  The crane has been out of service all week.  Researchers are able to trap fish still.  Project maintenance crews expect to have the crane back in service by the end of next week.

5.6 Sea lions and harbor seals have been observed in the tailrace of both powerhouses and the spillway.  No action is being taken by the project to harass, harm, or otherwise annoy these animals.  FFU is monitoring and recording their activity.

 JAMES R. MAHAR

                                 

                        

                        Operations Manager

//S//

1
2

