CENWP-OP-B                                                                                                                                           05 May 2003

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 18 of 2003, which covers the period 27 April through 03 May 2003.

1.  OPERATION SUMMARY.

1.1 Daily river flow averaged 227.9 to 251.6 kcfs.  Daily project tailwater elevation averaged 17.2’ to 20.4’ msl.  Daily project forebay elevation averaged 73.1’ to 75.3’ msl.  River temperature averaged 49 to 50° F.  Daily turbidity measured 4.0’ to 5.3’ on the secchi disk.

1.2 Spill Bays 1 and 18, which supply attraction flow for Cascade Island and Bradford Island fishways respectively, are included in the spill pattern.  Spill averaged 87.1 to 146.7 kcfs.

1.3 Unit Operation

1.3.1 Unit 1 remains out of service for turbine rehab.

1.3.2 Units 5-8 were out of service from 0900 until 1542 on 28 April to facilitate BPA repairs on A-6 breaker.

1.3.3 Unit 11 was out of service from 0830 until 1054 on 30 April for STS repairs.

1.3.4 Unit 13 was out of service from 0718 until 0736 on 29 April due to a voltage control problem.

1.3.5 Unit 15 remains out of service for governor work.

1.3.6 Unit 16 was out of service from 1418 on 27 April until 1712 on 28 April due to an STS failure in gate slot “C”.

1.3.7 Unit 17 was out of service from 0712 until 1536 on 01 May to facilitate STS removal and equipment installation for the gap loss study.

1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  During fish passage season, the units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.  

1.5 Gatewell drawdowns are measured on Wednesdays.

2.  ACTIVITIES. 
2.1 AWS Closures.  No AWS valves were closed for cleaning this reporting week.

2.2 STS/VBS Inspections.  The following STS/VBS inspections were conducted this reporting week.

	Unit
	Date
	Run Hours
	Comments

	11
	4/30
	839
	Repaired rip in A slot

	12
	4/29
	755
	

	13
	4/29
	762
	

	14
	4/29
	623
	

	15
	4/15
	544
	

	16
	4/29
	747
	

	17
	4/29
	835
	Flow vanes added to STSs

	18
	4/30
	839
	


Table 1.  STS Inspection Dates.

2.3 Dewatering and Fish Salvage.  No dewatering or fish salvage activities this reporting week.

3.  RESEARCH.

3.1 CRITFC.  Sampling of chinook, sockeye, and coho continues until 31 October.

3.2 NMFS Lamprey.  Tracking continues once a month.  Tagging to begin the second week in May.

3.3 NMFS Flat plate DSM1.  NMFS released test fish over the flat plate in DSM1 on 01 May.

3.4 PSMFC at DSM1.  Continuous PIT tag interrogation continues.  GBT and monitoring continue.

3.5 PSMFC at the SMF.  Three kelts were observed stuck on the separator bars and released alive.  Monitoring personnel observed 103 steelhead, 29 salmon, four unknown salmonids and 263 smolts slide over the separator bars.

3.6 Spring Creek NFH.  Scheduled to release 3.5 million fall chinook on 08 May.

3.7 University of Idaho.  Adult tracking tagging and tracking continue.  Two mortalities were reported this week.  Please see Misc. 5.6.
3.8 WDFW jaw tagging.  Tagging steelhead and spring chinook for tooth tangle net studies.

3.9 USGS Juvenile Radiotracking.  USGS did not received funding this fiscal year.  

3.10 WDFW Fish Counting.  16-hr visual counts at Washington Shore and Bradford Island.

3.11 WES/Battelle/BAE systems Juvenile hydroacoustic monitoring.  Began monitoring gap loss with acoustic camera.  

3.12 U of I Transportation Study.  Evaluation of physiological condition of river run vs. barged Snake River juveniles to occur May-June.  

4.  FISHWAYS 
4.1 Project biologists inspected 27-30 April and 01,03 May this week.  

4.2 Fish Passage Plan Observations.

	DATE
	LOCATion
	FPP Deviation
	CAUSE
	RESPONSE
	TR/WAR

	All week
	PH1 PCC
	Velocity <1.5fps
	Meter in slowest part of channel
	No response at this time.
	

	All week
	PH2 PCC
	No velocity measure.
	Meter is missing
	See misc. 5.1
	

	5/02
	Bonneville
	Biologists did not inspect 7 days/week.
	Personnel shortage
	
	

	All week
	Bonneville
	Biologists did not check calibration.
	Personnel shortage
	
	


4.3 Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Kelvin.  http://students.washington.edu/kyle/temptable.jpg 

	Date
	Wash. Shore
	Bradford Is.

	4/27
	283.8
	283.9

	4/28
	283.9
	283.9

	4/29
	284.0
	284.0

	4/30
	284.2
	284.2

	5/1
	284.4
	284.5

	5/2
	284.6
	284.6

	5/3
	284.6
	284.5


4.4 Sensor Calibration.  Calibration was not checked this week.

4.5  Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week. 

5. MISCELLANEOUS

5.1 Divers discovered the PH2 velocity meter was missing from the collection channel.  A new meter will be installed during the mid season dives this summer.

5.2 On 24 January, the Project tailwater hard constraint decreased to 11.0’ msl for all hours.  The soft constraint went to 11.2’ msl between 0700-1900 hours.  This is for the protection of the chum redds around Ives Island.  On 17 February, BPA agreed to a hard tailwater constraint of 11.3’ msl and a soft constraint of 11.5’ msl when inflows to Bonneville are >120Kcfs.

5.3 A request for a contract for a tug to remove debris from PH2 forebay has been made.  

5.4 The AFF crane returned to service at 0905 on 28 April.

5.5 DSM1 was out of service from 1052 until 1115 on 30 April for flat plate repairs.  NMFS wanted to install a strip of neoprene to close a gap between the flat plate and the small dewatering screen.  They were also hoping to reduce vibration and interference with the PIT tag detections.

5.6 University of Idaho reported two tagging mortalities on 01 May.  Please contact Chris Peery or Steve Lee for more information.

5.7 Sea lions and harbor seals have been observed in the tailrace of both powerhouses and the spillway.  No action is being taken by the project to harass, harm, or otherwise annoy these animals.  FFU is monitoring and recording their activity.

 JAMES R. MAHAR

                                 

                        

                        Operations Manager

//S//

1
3

