CENWP-OP-B                                                                                                                                           01 June 2003

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 22 of 2003, which covers the period 25 through 31 May 2003.

1.  OPERATION SUMMARY.

1.1 Daily river flow averaged 251.3 to 350.5 kcfs.  Daily project tailwater elevation averaged 19.4’ to 25.0’ msl.  Daily project forebay elevation averaged 74.4’ to 75.8’ msl.  River temperature averaged 54 to 55° F.  Daily turbidity measured 2.2’ to 5.0’ on the secchi disk.

1.2 Spill Bays 1 and 18, which supply attraction flow for Cascade Island and Bradford Island fishways respectively, are included in the spill pattern.  Spill averaged 94.9 to 140.6 kcfs.  See misc. 5.3 and 5.4.
1.3 Unit Operation

1.3.1 Unit 1 remains out of service for turbine rehab.

1.3.2 Unit 14 remains out of service.

1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.  

1.5 Gatewell drawdowns are measured on Wednesdays.

2.  ACTIVITIES. 
2.1 AWS Closures.  The following AWS valves were closed for cleaning this reporting week.

2.1.1 FV3-9 from 1500 until 1505 on 29 May.

2.2 STS/VBS Inspections.  The following STS/VBS inspections were conducted this reporting week.

	Unit
	Date
	Run Hours
	Comments

	1
	
	
	Out of service

	2
	5/29
	518
	

	3
	5/29
	372
	

	4
	5/29
	181
	Unable to video C slot.  Will pull at 720 hours

	5
	5/28
	291
	

	6
	5/28
	148
	

	7
	5/28
	239
	

	8
	2/24
	
	Do not video ESBSs

	9
	5/28
	645
	

	10
	5/28
	741
	


Table 1.  STS Inspection Dates.

2.3 Dewatering and Fish Salvage.  No dewatering or fish salvage activities this reporting week.

3.  RESEARCH.

3.1 CRITFC.  Sampling of chinook, sockeye, and coho continues until 31 October.

3.2 NMFS Lamprey.  Tracking continues once a month.  Trapping and tagging began 26 May.

3.3 OCFWRU.  Fish collection continues. 

3.4 ODFW Steelhead.  Trapping continues. 

3.5 PSMFC at DSM1.  Continuous PIT tag interrogation continues.  GBT and monitoring continue.

3.6 PSMFC at the SMF.  Monitoring personnel removed on live kelt and two dead kelts from the separator bars.  148 steelhead, 14 salmon, six unknown salmonids and 861 smolts slid over the separator bars.

3.7 University of Idaho.  Adult tracking tagging and tracking continue.  

3.8 U of I Transportation Study.  Evaluation of river run vs. barged Snake River juveniles to occur May-June.  

3.9 University of Washington.  Northern pikeminnow tagging and tracking from May through July.  

3.10 USGS Lamprey.  Collection to begin in June.

3.11 WDFW jaw tagging.  Tagging steelhead and spring chinook for tooth tangle net studies.

3.12 WDFW Fish Counting.  16-hr visual counts at Washington Shore and Bradford Island.

3.13 WES/Battelle/BAE systems Juvenile hydroacoustic monitoring.  Gap loss monitoring continues. 

4.  FISHWAYS 
4.1 Project biologists inspected 25 through 31 May this week.  

4.2 Fish Passage Plan Observations.

	DATE
	LOCATion
	FPP Deviation
	CAUSE
	RESPONSE
	TR/WAR

	All week
	PH1 PCC
	Velocity <1.5fps
	Meter in slowest part of channel
	No response at this time.
	

	All week
	PH2 PCC
	No velocity measure.
	Meter is missing
	See misc. 5.1
	


4.3 Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Kelvin.  http://students.washington.edu/kyle/temptable.jpg 

	Date
	Wash. Shore
	Bradford Is.

	5/25
	286.6
	286.6

	5/26
	286.7
	286.7

	5/27
	286.6
	286.6

	5/28
	287.0
	287.0

	5/29
	287.3
	287.3

	5/30
	287.5
	287.4

	5/31
	287.5
	287.5


4.4 Sensor Calibration.  Calibration was checked on 31 May this reporting week.

4.5  Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week. 

5. MISCELLANEOUS

5.1 Divers discovered the PH2 velocity meter was missing from the collection channel.  A new meter will be installed during the mid season dives this summer.

5.2 27 May was the last day sea lions were observed in the tailrace. 

5.3 Spillbay 4 was out of service from 0415 on 27 May until 1100 on 29 May due to a failure to control properly.  Bays 3 and 5 were used to compensate for flow.  

5.4 Spillbay 5 was found in the full open position at 0400 on 29 May.  Cause is unknown at this time.  Operators returned the gate to the appropriate position and will monitor the spillway every two hours.

5.5 The SMF PDS screen cleaner returned to service on 28 May.  It cycles four times an hour.

5.6 A forebay restriction of 74.0-75.5’ went into effect at 1645 on 29 May and ended at 1800 on 31 May.  This restriction was to assist the tribal gillnet fishery above Bonneville.

5.7 At 1500 on 30 May, the Project is restricted to a maximum tailwater of 27’.  This is to prevent overtopping the bulkhead in the B2 Corner Collector.  The bulkhead went in about a month ago and is located at Station 24 +56 (near the start of the monolith).  

JAMES R. MAHAR, P.E.

Operations Manager

//S//

1
2

