CENWP-OP-B                                                                                                                                           29 June 2003

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 26 of 2003, which covers the period 22 through 28 June 2003.

1.  OPERATION SUMMARY.

1.1 Daily river flow averaged 193.5 to 249.7 kcfs.  Daily project tailwater elevation averaged 15.3’ to 18.9’ msl.  Daily project forebay elevation averaged 73.8’ to 75.0’ msl.  River temperature averaged 60 to 62° F.  Daily turbidity measured 4.0’ to 6.0’ on the secchi disk.

1.2 Spill Bays 1 and 18, which supply attraction flow for Cascade Island and Bradford Island fishways respectively, are included in the spill pattern.  Spill averaged 93.2 to 151.9 kcfs.  

1.3 Unit Operation

1.3.1 Unit 1 remains out of service for turbine rehab.

1.3.2 Unit 11 was out of service from 0754 until 1012 on 26 June for trashrack raking.

1.3.3 Unit 15 returned to service at 0900 on 25 June.

1.3.4 Unit 17 was out of service from 0654 until 1642 on 23 June while screens were switched with unit 15.  Unit 17 was out of service from 0706 until 0754 on 26 June for pressure transducer calibration (part of the gap loss study).

1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.   

1.5 Gatewell drawdowns are measured on Wednesdays.

2.  ACTIVITIES. 
2.1 AWS Closures.  No AWS valves were closed for cleaning this reporting week.

2.2 STS/VBS Inspections.  The following STS/VBS inspections were conducted this reporting week.

	Unit
	Date
	Run Hours
	Comments

	12
	6/26
	449
	

	13
	6/26
	443
	

	14
	6/26
	406
	

	15
	6/23
	410
	Cleaned VBS on 23 June

	16
	6/26
	444
	

	17
	6/23
	671
	Inspected during gap loss study


Table 1.  STS Inspection Dates.

2.2.1 STSs were pulled from units 3-6 on 24 June, in accordance with section 2.4.1.2.o of the FPP.

2.3 Dewatering and Fish Salvage.  No dewatering or fish salvage activities this reporting week.

3.  RESEARCH.

3.1 CRITFC.  Sampling of chinook, sockeye, and coho continues until 31 October.

3.2 NMFS Lamprey.  Trapping, tagging and tracking continue.

3.3 OCFWRU.  Fish collection continues. 

3.4 ODFW Steelhead.  Trapping continues. 

3.5 PSMFC at DSM1.  Continuous PIT tag interrogation continues.  GBT concluded.

3.6 PSMFC at the SMF.  SMF personnel observed 11 steelhead, five salmon and 12 smolts slide over the separator bars.  GBT exams will occur at the SMF for the remainder of the season.

3.7 University of Idaho.  Adult tracking tagging and tracking continue.  

3.8 U of I Transportation Study.  Evaluation of river run vs. barged Snake River juveniles to occur May-June.  

3.9 University of Washington.  Northern pikeminnow tagging and tracking from May through July.  

3.10 Warm Springs Tribes.  Collected 71 lamprey from NMFS on 26 June.

3.11 WDFW Fish Counting.  16-hr visual counts at Washington Shore and Bradford Island.

3.12 WES/Battelle/BAE systems Juvenile hydroacoustic monitoring.  Gap loss monitoring continues.

4.  FISHWAYS 
4.1 Project biologists inspected 22 through 28 June this week.  NMFS inspected on 25 June.

4.2 Fish Passage Plan Observations.

	DATE
	LOCATion
	FPP Deviation
	CAUSE
	RESPONSE
	TR/WAR

	All week
	PH1 PCC
	Velocity <1.5fps
	Meter in slowest part of channel
	No response at this time.
	

	All week
	PH2 PCC
	No velocity measure.
	Meter is missing
	See misc. 5.1
	

	6/22
	Weir 37 
	Reading >or <1.3’ +/- .1’
	Change in FB elev.
	
	

	6/22
	Weir 38
	Reading >or <1.3’ +/- .1’
	Change in FB elev.
	
	

	6/22
	UMT weir
	Reading >or <1.3’ +/- .1’
	Change in FB elev.
	
	

	6/22, 6/25
	FG3-25
	Not in correct position for TW elev.
	Unknown
	Operators opened valve.
	


4.3 Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Kelvin.  http://students.washington.edu/kyle/temptable.jpg 

	Date
	Wash. Shore
	Bradford Is.
	AFF exit

	6/22
	289.7
	289.7
	289.7

	6/23
	289.7
	289.4
	289.6

	6/24
	289.9
	289.7
	289.9

	6/25
	290.0
	290.1
	290.0

	6/26
	290.4
	290.5
	290.3

	6/27
	290.7
	290.8
	290.6

	6/28
	291.0
	291.1
	290.9


4.4 Sensor Calibration.  Calibration was checked on 22 and 26 June this reporting week.

4.5  Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week. 

5. MISCELLANEOUS

5.1 Divers will replace the missing velocity meter at PH2 during our mid-season dive, scheduled for 21 July.

5.2 As of 1500 on 30 May, the Project is restricted to a maximum tailwater of 27’.  This is to prevent overtopping the bulkhead in the B2 Corner Collector, which is located at Station 24 +56.  

5.3 As of 28 June, the Project will have a daytime spill of 75 kcfs.

5.4 The ERG at DSM2 was tripped open from 0915 until 0930 on 26 June.  The sensor is believed to have tripped the ERG when channel elevations rose due to Unit 11 orifices being opened too close together.  Care will be taken to avoid this situation in the future.

5.5 DSM2 Screen Cleaner #1 broke a chain at 1230 on 27 June.

5.6 Orifice 18A-N has a bad proximity switch.  The orifice was put to local/ open at 1250 on 27 June.

JAMES R. MAHAR, P.E.

Operations Manager

//S//

1
2

