CENWP-OP-B                                                                                                                        28 September 2003

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 39 of 2003, which covers the period 21 through 27 September 2003.

1.  OPERATION SUMMARY.

1.1 Daily river flow averaged 84.3 to 127.3 kcfs.  Daily project tailwater elevation averaged 9.3’ to 11.5’ msl.  Daily project forebay elevation averaged 75.7’ to 76.0’ msl.  River temperature averaged 64° F.  Daily turbidity measured 7.0’ to 7.5’ on the secchi disk.

1.2 Spill Bays 1 and 18, which supply attraction flow for Cascade Island and Bradford Island fishways respectively, are open .5’ from 0530 until 1930 each day.  Spill averaged 2.4 to 2.5 kcfs.  

1.3 Unit Operation

1.3.1 Unit 1 remains out of service for turbine rehab.

1.3.2 Units 2 through 6 remain out of service due to crane rail replacement.  
1.3.3 Unit 7 remains out of service for its five year overhaul.

1.3.4 Unit 8 remains out of service due to crane rail replacement. 

1.3.5 Unit 14 remains out of service for its four year overhaul.

1.3.6 Unit 17 was out of service from 1212 until 1322 on 25 September due to voltage regulator problems.

1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.   F1 was out of service from 0806 until 1630 on 22 September to facilitate electrical mods.  F2 was out of service from 0400 until 1454 on 23 September to facilitate electrical mods.  Fishways remained in criteria.

1.5 Gatewell drawdowns are measured on Wednesdays.

2.  ACTIVITIES. 
2.1 AWS Closures.  No AWS valves were closed this reporting week.

2.2 STS/VBS Inspections.  No STS/VBS inspections were conducted this reporting week.

2.3 Dewatering and Fish Salvage.  The following dewatering or fish salvage activities occurred this week.  

24 September- biologists removed one lamprey from the scroll case of Unit 7.  

3.  RESEARCH.

3.1 CRITFC.   Sampling of chinook, sockeye, and coho continues until 31 October.  

3.2 NMFS Lamprey.  Trapping, tagging and tracking continue.

3.3 ODFW Steelhead.  Trapping continues one to two days a week.

3.4 PSMFC at the SMF.  One steelhead and three salmon slid over the separator bars.

3.5 University of Idaho.  Adult tracking tagging and tracking continues.

3.6 WDFW Pink DNA.  WDFW is taking DNA samples from pinks collected in the AFF this week.

3.7 WDFW Fish Counting.  16-hr visual counts at Washington Shore and Bradford Island.

3.8 WDFW Tule counter.  Visual counts continue at Washington Shore and Bradford Island.

4.  FISHWAYS 
4.1 Project biologists inspected 21 through 27 September this reporting week.  

4.2 Fish Passage Plan Observations.

	DATE
	LOCATion
	FPP Deviation
	CAUSE
	RESPONSE
	TR#

	All week.
	PH1 PCC
	Velocity <1.5fps
	Meter in slowest part of channel
	No response at this time.
	

	All week
	PH2 PCC
	No velocity measure.
	Logic board was short circuited
	Sending in to mfr for repair
	

	All week
	PH1 ITS
	7A closed.
	Crane rail work
	8B opened.  See const. 5.2.1
	

	9/23-9/27
	FG3-3, FG3-4
	Diffuser out of service
	Mechanical binding.
	FG3-4 to compensate.  Repairs scheduled this winter.
	


4.3 Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Celsius this week.  http://students.washington.edu/kyle/temptable.jpg   (295.37K = 22.22C= 72F)

	Date
	WA Shore
	Bradford Is.
	AFF exit

	9/21
	19.05
	19.05
	19.05

	9/22
	18.95
	18.95
	18.95

	9/23
	19.15
	19.15
	19.15

	9/24
	19.25
	19.35
	19.25

	9/25
	19.15
	19.15
	19.15

	9/26
	19.25
	19.25
	19.35

	9/27
	19.25
	19.25
	19.35


4.4 Sensor Calibration.  Calibration was checked on 25 September this reporting week.

4.5  Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week. 

5. CONSTRUCTION

5.1 PH 2 Corner Collector-  Nothing to report this week.
5.2 PH 1 Crane Rail Replacement- started on 01 August and will continue until 16 December. 

5.2.1 As a safety precaution, chain gate 7A has been closed (8A to compensate) and the end gate will be closed during the hours of 0530 until 1900 for the contractors working over the ITS.  The end gate will be opened each evening to flush fish.  

5.2.2 PH1 priority is Unit 10 then Unit 9 with STSs.  The ITS will be opened during unit operation and allowed to run for not less than four hours after the units have been shutdown (to flush fish that may have been attracted into PH1 forebay).

5.2.3 PH1 ITS, Unit 9 and Unit 10 operations summary

21 September- Units 9 and 10 in service as the priority units.  See misc. 6.2

22 September- Units 9 and 10 in service as the priority units.  See misc. 6.2
23 September- Units 9 and 10 in service for power generation.

24 September- Units 9 and 10 in service until 0307.  ITS closed from 0540 until 1710.

25 September- ITS closed from 0510 until 1715.

26 September- ITS closed from 0535 until 1306.  Units 9 and 10 in service at 1306.

27 September- Units 9 and 10 in service until 0258.  ITS closed from 0530 until 1000.  Units 9 and 10 in service at 0911.

5.3 Adult PIT tag detection (vertical slots).  The project management plan is out for review.
5.4 High Flow PIT tag detectors for B2CC.   Prototype placement coordination on going.
6. MISCELLANEOUS

6.1 At 1340 on 22 September, RCC issued a teletype directing Bonneville to return to PH2 priority.  
6.2 DSM1 was in manual from 0815 until 1040 on 25 September.  This was due to the crane rail contractors cutting the PH1 forebay signal to the PLC.
6.3 The Bonneville Winter Maintenance schedule is available.  Please contact Bonneville Fisheries for a copy.
JAMES R. MAHAR, P.E.

Operations Manager

//S//

1
2

