CENWP-OP-B                                                                                                                        06 October 2003

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 40 of 2003, which covers the period 28 September through 04 October 2003.

1.  OPERATION SUMMARY.

1.1 Daily river flow averaged 83.1 to 121.8 kcfs.  Daily project tailwater elevation averaged 8.1’ to 10.9’ msl.  Daily project forebay elevation averaged 72.4’ to 75.4’ msl.  River temperature averaged 64 to 65° F.  Daily turbidity measured 5.5’ to 7.0’ on the secchi disk.

1.2 Spill Bays 1 and 18, which supply attraction flow for Cascade Island and Bradford Island fishways respectively, are open .5’ from 0530 until 1930 each day.  Spill averaged 2.3 to 2.5 kcfs.  

1.3 Unit Operation

1.3.1 Unit 1 remains out of service for turbine rehab.

1.3.2 Units 2 through 6 remain out of service due to crane rail replacement.  
1.3.3 Unit 7 remains out of service for its five year overhaul.

1.3.4 Unit 8 remains out of service due to crane rail replacement. 

1.3.5 Unit 14 remains out of service for its four year overhaul.

1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.   

1.5 Gatewell drawdowns are measured on Wednesdays.

2.  ACTIVITIES. 
2.1 AWS Closures.  The following AWS valves were closed for cleaning this reporting week.

FV3-9- closed from 0810 until 0840 on 29 September.

FV5-9- closed from 0735 until 0753 on 29 September.

FV6-9- closed from 0753 until 0810 on 29 September.

2.2 STS/VBS Inspections.  No STS/VBS inspections were conducted this reporting week.

2.3 Dewatering and Fish Salvage.  No dewatering or fish salvage activities occurred this week.  

3.  RESEARCH.

3.1 CRITFC.   Sampling of chinook, sockeye, and coho continues until 31 October.  

3.2 ODFW Steelhead.  Trapping continues one to two days a week.

3.3 PSMFC at the SMF.  Three steelhead and 27 salmon fallbacks slid over the separator bars.

3.4 University of Idaho.  Adult tracking tagging and tracking continues.

3.5 WDFW Pink DNA.  WDFW is taking DNA samples from pinks collected in the AFF this week.

3.6 WDFW Fish Counting.  16-hr visual counts at Washington Shore and Bradford Island.

3.7 WDFW Tule counter.  Visual counts continue at Washington Shore and Bradford Island.

4.  FISHWAYS 
4.1 Project biologists inspected 28 September through 04 October this reporting week.  

4.2 Fish Passage Plan Observations.

	DATE
	LOCATion
	FPP Deviation
	CAUSE
	RESPONSE
	TR#

	All week.
	PH1 PCC
	Velocity <1.5fps
	Meter in slowest part of channel
	No response at this time.
	

	All week
	PH2 PCC
	No velocity measure.
	Logic board was short circuited
	Sending in to mfr for repair
	

	All week
	PH1 ITS
	7A closed.
	Crane rail work
	8B opened.  See const. 5.2.1
	

	All week
	FG3-3, FG3-4
	Diffuser out of service
	Mechanical binding.
	FG3-4 to compensate.  Repairs scheduled this winter.
	

	9/28
	FG3-26
	Diffuser not in correct position for TW elev.
	Unknown
	Operators closed the diffuser.
	


4.3 Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Celsius this week.  http://students.washington.edu/kyle/temptable.jpg   (295.37K = 22.22C= 72F)

	Date
	WA Shore
	Bradford Is.
	AFF exit

	9/28
	19.25
	19.35
	19.35

	9/29
	19.25
	19.15
	19.25

	9/30
	19.15
	19.05
	19.05

	10/01
	18.95
	18.95
	18.85

	10/02
	18.85
	18.85
	18.85

	10/03
	18.85
	18.85
	18.75

	10/04
	18.85
	18.75
	18.75


4.4 Sensor Calibration.  Calibration was checked on 03 October this reporting week.

4.5  Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week. 

5. CONSTRUCTION

5.1 PH 2 Corner Collector-  Coordination to install the gate and test the chute on 16 October in progress.
5.2 PH 1 Crane Rail Replacement- started on 01 August and will continue until 16 December. 

5.2.1 As a safety precaution, chain gate 7A has been closed (8A to compensate) and the end gate will be closed during the hours of 0530 until 1900 for the contractors working over the ITS.  The end gate will be opened each evening to flush fish.  

5.2.2 PH1 priority is Unit 10 then Unit 9 with STSs installed.  The ITS will be opened during unit operation and allowed to run for not less than four hours after the units have been shutdown (to flush fish that may have been attracted into PH1 forebay).

5.2.3 PH1 ITS, Unit 9 and Unit 10 operations summary

28 September- Units 9 and 10 in service from 0911 on 27 September until 0400 on 28 September.
29 September- ITS closed from 0518 until 1730.

30 September- ITS closed from 0518 until 0907.  Units 9 and 10 in service from 0907 until 1603.

1 October- ITS closed from 0520 until 1730.

2 October- Unit 10 in service at 0800.

3 October- ITS closed from 0530 until 2010.  Unit 10 to standby at 0536.

5.3 Adult PIT tag detection (vertical slots).  The project management plan is out for review.
5.4 High Flow PIT tag detectors for B2CC.   50% P&S out.  Comments due by 20 October.
5.5 Main dam excavation work.  Coordination meeting with the contractors to be scheduled.
5.6 PH2 FGE/VBS mods.  BCOE comments due by 16 October.  
6. MISCELLANEOUS

6.1 F2 manual orifices were open from 0840 on 29 September until 2130 on 04 October to maintain DSM channel elevations.
6.2 Unit 12 had a 1% violation starting at 0901 on 29 September.  The violation was due to troubleshooting the governor lock up and power failure.
6.3 Per RCC, Bonneville had a hard forebay constraint of 71.5 – 72.5’ msl from 0630 until 1700 on 29 and 30 September.  This was to facilitate cleaning of the spillway by the structural crew.
6.4 The Bonneville Winter Maintenance schedule is available.  Please contact Bonneville Fisheries for a copy.
JAMES R. MAHAR, P.E.

Operations Manager

//S//

1
2

