CENWP-OP-B                                                                                                                        05 January 2004

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 01 of 2004, which covers the period 28 December 2003 through 03 January 2004.

1.  OPERATION SUMMARY.

1.1 Daily river flow averaged 122.5 to 170.6 kcfs.  Daily project tailwater elevation averaged 12.5’ to 15.4’ msl.  Daily project forebay elevation averaged 72.3’ to 74.9’ msl.  River temperature averaged 39 to 41° F.  Daily turbidity measured 7.0 on the secchi disk.  

1.2 Spill Bay 18, which supplies attraction flow for the Bradford Island fishway, is open .5’ from 0630 until 1600 each day.  Spill averaged .7 to .8 kcfs.  Spillbay 1 will remain closed until Cascades Island returns to service.

1.3 Unit operation

1.3.1 Unit 1 remains out of service for turbine rehab.

1.3.2 Unit 12 had a 1% violation due to trashrack raking on 30 December.

1.3.3 Unit 16 had a 1% violation due to asbestos sampling from 0830 until 0900 on 30 December.

1.3.4 Unit 17 had a 1% violation due to trashrack raking from 1355 until 1553 on 29 December.

1.3.5 Unit 18 had a 1% violation due to trashrack raking from 1317 until 1355 on 29 December.

1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.   

1.4.1 F1 remains out of service for testing and winter maintenance.

1.4.2 F2 remains out of service for F1 testing and winter maintenance.

1.5 Gatewell drawdowns are measured on Wednesdays.

2.  ACTIVITIES. 
2.1 AWS Closures. No AWS valves were closed for cleaning this reporting week.

2.2 STS/VBS Inspections.  All STSs are removed until fish passage season.

2.3 Dewatering and Fish Salvage.  No dewatering or salvage activities this reporting week.

2.3.1 Upcoming events

05-09 Jan- Washington Shore and Cascades Island return to service.

07-09 Jan- Unit 17 dewater for VBS mods

12 Jan- two mile pipe survey

3.  RESEARCH.

3.1 NOAA Fisheries.  Demonstration of measuring and weighing board at the AFF on/around 12 January.

3.2 WDFW Fish Counting.  16-hr video counts at Bradford Island.

3.3 WES/Battelle/BAE Systems.  Collecting preliminary hydroacoustic data on MU 10. 

4.  FISHWAYS 
4.1 Project biologists inspected 29-31 December this reporting week.

4.2 Fish Passage Plan Observations.

	DATE
	LOCATion
	FPP Deviation
	CAUSE
	RESPONSE
	TR#

	All week
	PH1 PCC
	Velocity <1.5fps
	Meter in slowest part of channel
	No response at this time.
	

	All week
	FG3-3, FG3-4
	FG3-3 OOS
	Mechanical binding.
	FG3-4 to compensate.  
	


4.3 Fish Ladder Temperatures.  Temperatures are not recorded during winter maintenance season.

4.4 Sensor Calibration.  Calibration was checked on 29 December this week.

4.5  Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week. 

5. CONSTRUCTION

5.1 PH 2 Corner Collector- Gate test is scheduled on 8-9 January.  

5.2 PH 2 Corner Collector Hi-Q PIT tag detector- Prototype still planned for installation in 2004.

5.3 PH2 Corner Collector Avian Wires- Frame work completed for avian wire installation scheduled in February over the plunge pool.   

5.4 PH 1 Crane Rail Replacement- Crane rail installed, clean up work ongoing.   

5.5 PH1 PSC Removal.  Scheduled for 05 January 04.

6. MISCELLANEOUS

6.1 Project Fisheries is working with Bonneville Rangers in submitting a grant proposal for the Trout Unlimited Embrace A Stream grant.  We are looking at weed removal, re-vegetation, and in stream restoration at Bass Lake.

6.2 Per a teletype sent on 18 December, Project tailwater will have a hard constraint of 11.8’ and a soft constraint of 11.8’ to 12.1’ between the hours of 0700 until 1700.  This constraint is for chum spawning habitat located below the dam.

JAMES R. MAHAR, P.E.

Operations Manager

//S//

1
2

