CENWP-OP-B                                                                                                                        12 January 2004

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 02 of 2004, which covers the period 04 through 10 January 2004.

1.  OPERATION SUMMARY.

1.1 Daily river flow averaged 112.5 to 182.6 kcfs.  Daily project tailwater elevation averaged 12.3’ to 15.9’ msl.  Daily project forebay elevation averaged 71.9’ to 73.9’ msl.  River temperature averaged 35 to 39° F.  Daily turbidity measured 4.5 to 7.0 on the secchi disk.  

1.2 Spill Bay 18, which supplies attraction flow for the Bradford Island fishway, is open .5’ from 0630 until 1600 each day.  Spill averaged .5 to .8 kcfs.  Spillbay 1 will remain closed until Cascades Island returns to service.

1.3 Unit operation

1.3.1 Unit 1 remains out of service for turbine rehab.

1.3.2 Unit 4 was out of service from 0436 until 1200 on 05 January due to high bearing temperatures.

1.3.3 Unit 11 remains out of service due to an exciter failure on 17 October. 

1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.   

1.4.1 F1 remains out of service for testing and winter maintenance.

1.4.2 F2 remains out of service for F1 testing and winter maintenance.

1.5 Gatewell drawdowns are measured on Wednesdays.

2.  ACTIVITIES. 
2.1 AWS Closures. The following AWS valves were closed for cleaning this reporting week.

FV1-1—from 1240 until 1330 on 05 January.

FV3-7—from 1345 until 1400 on 05 January.

FV3-9—from 0747 until 0835 on 05 January.

2.2 STS/VBS Inspections.  All STSs are removed until fish passage season.

2.3 Dewatering and Fish Salvage.  No dewatering or salvage activities this reporting week.

2.3.1 Upcoming events

13-15 Jan- Washington Shore and Cascades Island return to service.

14-17 Jan- Unit 17 dewater for VBS mods

3.  RESEARCH.

3.1 NOAA Fisheries.  Demonstration of measuring and weighing board at the AFF on/around 12 January.

3.2 WDFW Fish Counting.  16-hr video counts at Bradford Island.

4.  FISHWAYS 
4.1 Project biologists inspected 05 January this reporting week.

4.2 Fish Passage Plan Observations.

	DATE
	LOCATion
	FPP Deviation
	CAUSE
	RESPONSE
	TR#

	All week
	PH1 PCC
	Velocity <1.5fps
	Meter in slowest part of channel
	No response at this time.
	

	All week
	FG3-3, FG3-4
	FG3-3 OOS
	Mechanical binding.
	FG3-4 to compensate.  
	

	All week
	BI fishway
	Water over weirs
	Trashracks need cleaned
	Riggers cleaned trashracks.
	

	All week
	Bonneville Dam
	Less than three inspections/ week by Project Bios
	Project closed due to snow and ice.
	No response
	


4.3 Fish Ladder Temperatures.  Temperatures are not recorded during winter maintenance season.

4.4 Sensor Calibration.  Calibration was not checked due to Project closure most of the week.

4.5  Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week. 

5. CONSTRUCTION

5.1 PH 2 Corner Collector- Gate test is rescheduled for later in the winter maintenance season.

5.2 PH 2 Corner Collector Hi-Q PIT tag detector- Prototype still planned for installation in 2004.

5.3 PH2 Corner Collector Avian Wires- Frame work completed for avian wire installation scheduled in February over the plunge pool.   

5.4 PH1 PSC Removal.  Scheduled for January 04.

6. MISCELLANEOUS

6.1 Project Fisheries is working with Bonneville Rangers in submitting a grant proposal for the Trout Unlimited Embrace A Stream grant.  We are looking at weed removal, re-vegetation, and in stream restoration at Bass Lake.

6.2 Per a teletype sent on 18 December, Project tailwater will have a hard constraint of 11.8’ and a soft constraint of 11.8’ to 12.1’ between the hours of 0700 until 1700.  This constraint is for chum spawning habitat located below the dam.

6.3 FG3-5 is in manual and closed.  There is a problem with the limit switch.

6.4 Bonneville Lock and Dam was closed due to snow and ice from 06 through 09 January.

JAMES R. MAHAR, P.E.

Operations Manager

//S//

1
2

