CENWP-OP-B                                                                                                                        03 February 2004

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 05 of 2004, which covers the period 25 through 31 January 2004.

1.  OPERATION SUMMARY.

1.1 Daily river flow averaged 112.9 to 186.5 kcfs.  Daily project tailwater elevation averaged 12.8’ to 18.1’ msl.  Daily project forebay elevation averaged 74.5’ to 74.9’ msl.  River temperature averaged 35 to 38° F.  Daily turbidity measured 5.0 to 7.0 on the secchi disk.  

1.2 Spill Bays 1 and 18 will remain closed until more fish start passing the Project or spill season starts.  Spill averaged 0 kcfs.  

1.3 Unit operation

1.3.1 Unit 1 remains out of service for turbine rehab.

1.3.2 Unit 2 was out of service from 0453 until 1525 on 26 January for PSC removal.

1.3.3 Unit 3 was out of service from 0453 until 1824 on 26 January and 0526 until 1652 on 27 January for PSC removal.

1.3.4 Unit 4 was out of service from 0453 until 1824 on 26 January, 0526 until 1652 on 27 January, 0600 until 1708 on 28 January, and 0600 until 1648 on 29 January for PSC removal.

1.3.5 Unit 5 was out of service from 0453 until 1824 on 26 January, 0526 until 1652 on 27 January, 0600 until 1708 on 28 January, 0600 until 1648 on 29 January, and 0600 until 1614 on 30 January.

1.3.6 Unit 6 was out of service from 0600 until 1708 on 28 January, 0600 until 1648 on 29 January, and 0600 until 1614 on 30 January.

1.3.7 Unit 11 remains out of service. 

1.3.8 Unit 17 remains out of service for VBS modifications.

1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.  

1.4.1 F2 remains out of service for winter maintenance.

1.5 Gatewell drawdowns are measured on Wednesdays.

2.  ACTIVITIES. 
2.1 AWS Closures. The following AWS valves were closed this reporting week.

FV1-1—remains closed.

FV3-7— remains closed.

FV3-9— remains closed.

FV4-3/FV4-4— remains closed.

2.2 STS/VBS Inspections.  All STSs are removed until fish passage season.

2.3 Dewatering and Fish Salvage.  No dewatering or salvage activities this reporting week. 

3.  RESEARCH.

3.1 Battelle FGE testing.  Transducers to be installed at PH2 in early February.

3.2 WDFW Fish Counting.  16-hr video counts at Washington Shore.

4.  FISHWAYS 
4.1 Project biologists inspected 26-29 January this reporting week.

4.2 Fish Passage Plan Observations.

	DATE
	LOCATion
	FPP Deviation
	CAUSE
	RESPONSE
	TR#

	All week
	PH2 PCC
	Velocity <1.5fps
	Unknown
	Electricians to look at.
	


4.3 Fish Ladder Temperatures.  Temperatures are not recorded during winter maintenance season.

4.4 Sensor Calibration.  Calibration was checked on 29 January.

4.5  Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week. 

4.6 Fishway Status.  

4.6.1 Both DSMs remain out of service.

4.6.2 Bradford Island fishway remains dewatered.  B-Branch returned to tailwater at 1545 on 31 January.

4.6.3 The south end of PH1 PCC will be dewatered on 09 February.  This is for hydraulic engineers working on the HELCRABS model.

5 CONSTRUCTION

5.1 PH 2 Corner Collector- Gate test is rescheduled for 06 February.  Hydraulic tests to be conducted 09-11 Feb

5.2 PH 2 Corner Collector Hi-Q PIT tag detector- Prototype still planned for installation in 2004.

5.3 PH2 Corner Collector Avian Wires- Avian wire installation scheduled for 25 February over the plunge pool.   

5.4 PH1 PSC Removal.  Removal ongoing.

5.5 Unit 17 VBS mods.  Modifications underway.

5.6 B2 Follow-on.  The contractors are still working on the airburst system in DSM2.  Their schedule has been extended by 3.5 days.  A pipe survey is for 03-05 February.

6 MISCELLANEOUS

6.1 Project Fisheries is working with Bonneville Rangers in submitting a grant proposal for the Trout Unlimited Embrace A Stream grant.  Grant recipients will be notified in early Spring.

6.2 Per a teletype sent on 18 December, Project tailwater will have a hard constraint of 11.8’ and a soft constraint of 11.8’ to 12.1’ between the hours of 0700 until 1700.  This constraint is for chum spawning habitat located below the dam.  

6.3 Per a teletype sent on 26 January, the Project can exceed the forebay constraint of 76.5’ msl beginning at 2000 on 29 January.

JAMES R. MAHAR, P.E.

Operations Manager

//S//

1
2

