CENWP-OP-B                                                                                                                                     26 April 2004

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 17 of 2004, which covers the period 18 through 24 April 2004.

1.  OPERATION SUMMARY.

1.1 Daily river flow averaged 177.7 to 236.8 kcfs.  Daily project tailwater elevation averaged 14.6’ to 17.9’ msl.  Daily project forebay elevation averaged 74.5’ to 76.1’ msl.  Daily river temperature averaged 50 to 51° F.  Daily turbidity measured 4.3 to 6.0 on the secchi disk.  

1.2 Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B Branch respectively, are part of the FPP spill pattern.  Spill averaged 74.9 to 94.1 kcfs.   See misc. 6.3
1.3 Unit operation.

1.3.1 Unit 1 remains out of service for turbine rehab.

1.3.2 Unit 2 was out of service from 1117 until 1800 on 22 April to replace HUB air compressor.

1.3.3 Unit 4 was out of service from 1004 until 1710 on 19 April due to scintillation rack installation.

1.3.4 Unit 11 was out of service from 1246 until 1612 on 21 April for transducer repairs.

1.3.5 Unit 15 went to manual 3D cam adjustment at 0933 on 05 April.  The cam was automatically adjusting and draining power which led to a governor lock up and complete loss of control.  New modules are on order.  Repairs should be made by the end of the month.  

1.3.6 Unit 18 was out of service from 0704 until 1057 on 21 April for transducer repairs.  

1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.  

1.5 Gatewell drawdowns are measured on Wednesdays.  Drawdowns are measured daily on Unit 17.

2.  ACTIVITIES. 
2.1 AWS Closures.  No AWS valves were closed this reporting week.

2.2 STS/VBS Inspections.  No STSs were inspected this reporting week.

2.3 Dewatering and Fish Salvage.  No dewatering or salvage activities occurred this reporting week. 

3.  RESEARCH.

3.1 Battelle/BAE Systems FGE testing.  Hydroacoustic installation complete.  Data collection began 15 April.     

3.2 Battelle/BAE Systems Gap Loss Testing.  Scheduled to begin 06 May. 

3.3 CRITFC.  Sampling twice a week.

3.4 ERDC TDG.  Sensors remain installed in the tailrace.

3.5 ODFW Pikeminnow Tagging.  Electrofishing in Bonneville’s forebay and tailrace.

3.6 NOAA FISH lamprey.  Lamprey ladder installation in April.  

3.7 PSMFC SMF.  Three kelts (two dead, one alive) were removed from the separator bars and 22 steelhead fell back over the separator bars this week.  Unusually high fish mortalities reported.  Please see misc. 6.2
3.8 PSMFC PIT tag.  Testing an antenna for the serpentine sections of the fishways.

3.9 USGS FPE testing.  Installation of Radio telemetry equipment complete.  First tagged fish release scheduled for 29 April.  See misc. 6.4
3.10 University of Idaho.  Sampling daily in the AFF.

3.11 Wildlife Services/Avian Abatement.  Birds were hazed this reporting week with rockets, cracker shells, and 15mm pyrotechnics.  None were lethally taken.

3.12 WDFW Fish Counting.  16-hr visual counts at Washington Shore and Bradford Island.

4.  FISHWAYS 
4.1 Project biologists inspected 18 through 24 April this reporting week.  

4.2 Fish Passage Plan Observations.

	DATE
	LOCATion
	FPP Deviation
	CAUSE
	RESPONSE
	TR#

	All week
	PH1 PCC
	Velocity <1.5 fps
	Meter in slowest part of the channel.
	This is a normal condition.
	

	All week
	FG3-4
	Incorrect position for TW
	Bent shaft
	No action taken.
	

	All week
	PH 2 PCC
	Velocity < 1.5 fps
	Unknown
	
	

	4/23
	Weirs 37/38
	Weirs reading < > 1.0’
	Unknown
	No action taken.  Only off by about .1’
	

	4/19-24
	Spill bays 1 and 18
	Improper spill gate opening
	Gate hoist malfunction and safety
	See misc. 6.3 
	


4.3 Fish Ladder Temperatures.  .  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Fahrenheit.

	Date
	Wash. Shore
	Bradford Is.

	4/18
	52.1
	52.4

	4/19
	52.3
	52.5

	4/20
	52.3
	52.6

	4/21
	52.8
	52.9

	4/22
	53.3
	53.3

	4/23
	53.3
	53.5

	4/24
	53.2
	53.4


4.4 Sensor Calibration.  Calibration was checked on 24 April this reporting week.

4.5  Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week. 

4.6 Fishway Status.  

4.6.1 DSM1 remains out of service.

4.6.2 On 18 April, FG3-5 went to manual.

4.6.3 On 23 April, the CI fish ladder went to manual due to computer problems.  The ladder remained in criteria.

5 CONSTRUCTION

5.1 PH 2 Corner Collector Hi-Q PIT tag detector- Prototype antenna construction started on Cascades Island.  If feasible, the prototype antenna will then be constructed into the existing channel during the in-water work period in FY05.
5.2 SMF hydrocannons.  The hydro cannons at the SMF are not operational at this time.  The Project is working on repairs.  

5.3 Adult PIT tag exit detectors.  Current schedule is set for installation in both ladders for Winter 04/05.

5.4 Head Gate Delivery.  Off loading of headgates at PH1 is scheduled for 07 June 2004.

5.5 PH 1 Gantry Crane Delivery and Erection.  Scheduled for 9 August through 4 December.    

6 MISCELLANEOUS AND OTHER NOTEWORTHY ITEMS

6.1 Sea lions and harbor seals have been observed in the tailrace of both powerhouses and the spillway.  No action is being taken by the project to harass, harm, or otherwise annoy these animals.  

6.2 PSMFC continues to report usual numbers of juvenile morts in their samples, but the numbers appear to be on the decline.  Project Biologists thoroughly inspected the DSM 2 gatewells, orifices and channel for blockages or signs of dead fish.  In recent years, we have experienced a sudden increase in mortalities, usually later in the spring, but have yet to figure out where they are coming from.  The following numbers were reported by PSMFC.

18 April- one unclipped yearling chinook, two clipped yearling chinook, six unclipped subyearling chinook, and one clipped subyearling chinook.

19 April- one clipped yearling chinook, one unclipped coho.

20 April- one clipped yearling chinook, and one unclipped coho.

21 April- one unclipped yearling chinook, two clipped yearling chinook, one unclipped subyearling chinook, and three unclipped coho.

24 April- one clipped yearling chinook, two unclipped subyearling chinook, and one unclipped coho.

6.3 On 19 April gate hoists for spillbays 1 and 18 were found to be damaged and a safety hazard.  Bays 1 and 18 were dogged off at 2 dogs (2.9 feet).  On 20 April, the bays were dropped to one dog (one foot) in an effort to reduce turbulence at the fishway entrances.  Per a 21 April teletype, spill will be compensated through adjacent bays 2 and 17 and through 3 and 16 in one foot increments.

6.4 On 21 April, USGS had a minor spill incident on Project and at the Hamilton Island boat launch.  Project ECC informed the USGS that, as the responsible party, they are required to contact the National Response Center (NRC) and report the spill they had caused.  USGS made the required notification around 1800 and attempted clean up of the boat ramp and roadway.  The USGS boat will not be allowed back on the water until repairs have been made to prevent discharge of oily bilge water.

JAMES R. MAHAR, P.E.

Operations Manager

//S//

1
3

