CENWP-OP-B                                                                                                                                     7 July 2004

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 27 of 2004, which covers the period 27 June through 3 July 2004.

1.  OPERATION SUMMARY.

1.1 Daily river flow averaged 162.1 to 216.6 kcfs.  Daily project tailwater elevation averaged 13.9’ to 16.7’ msl.  Daily project forebay elevation averaged 74.1’ to 76.0’ msl.  Daily river temperature averaged 63 to 65° F.  Daily turbidity measured 4.5 to 6.0 on the secchi disk.  
1.2 Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B Branch respectively, are part of the FPP spill pattern.  Spill averaged 50.9 to 94.9 kcfs.   BiOp versus 50kcfs testing started on 17 June. 
1.3 Unit operation.  

1.3.1 Unit 1 remains out of service for turbine rehab.

1.3.2 Unit 11 went out of service from 0710 to 1050 on 29 June for transducer repair.

1.3.3 Unit 14 went out of service from 1100 to 1359 on 29 June for transducer repair.

1.3.4 Unit 15 remains on manual 3D cam adjustment.  New modules are on order.  

1.3.5 Unit 18 went out of service from 0704 to 1112 on 30 June for transducer repair.

1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.  

1.5 Gatewell drawdowns are measured on Wednesdays.  Drawdowns are measured daily on Unit 17.
2.  ACTIVITIES. 
2.1 AWS Closures.  The following AWS valves were closed this reporting week:

FV 3-7, 3-9 - closed for cleaning from 0710 until 0730 on 1 July.

2.2 STS/VBS Inspections.  

	Unit
	Date
	Run Hours
	Comments

	11
	6/30
	1003
	 

	12
	6/30
	994
	

	13
	6/30
	979
	

	14
	6/30
	913
	

	15
	7/1
	331
	

	16
	7/1
	308
	

	17
	7/1
	1189
	

	18
	7/1
	967
	


2.3 Dewatering and Fish Salvage.  No dewatering or salvage activities occurred this reporting week.

3.  RESEARCH.
3.1 Battelle/BAE Systems FGE testing.  Data collection continues.

3.2 CRITFC.  Sampling twice a week.

3.3 ERDC TDG.  Sensors remain installed in the tailrace.

3.4 ODFW Pikeminnow Tagging.  Electrofishing in Bonneville’s forebay and tailrace will continue on 28 June through 2 July.

3.5 NOAA FISH lamprey.  Lamprey ladder installation complete.  

3.6 PSMFC SMF.  No fish were observed sliding over the separator bars this week.   

3.7 USFWS.  Lower Columbia River Fish Health Center collecting shad from the AFF.   

3.8 USGS FPE testing.  Releasing of radio tagged fish continues.   Last fish release scheduled for 31 July.  PSMFC collected 1,120 subyearling chinook for USGS RT testing.

3.9 University of Idaho.  Sampling daily in the AFF.  One steelhead mortality on 2 July.  See misc. 6.2
3.10 Wildlife Services/Avian Abatement.  Birds were hazed this reporting week with rockets, cracker shells, and 15mm pyrotechnics.  None were lethally taken.

3.11 WDFW Fish Counting.  16-hr visual counts at Washington Shore and Bradford Island.

4.  FISHWAYS 
4.1 Project biologists inspected 27 June through 3 July this reporting week.  

4.2 Fish Passage Plan Observations.

	DATE
	LOCATion
	FPP Deviation
	CAUSE
	RESPONSE
	TR#

	All week
	PH1 PCC
	Velocity <1.5 fps
	Meter in slowest part of the channel
	This is a normal condition.
	

	All week
	FG3-4
	Incorrect position for TW
	Bent shaft
	No action taken.
	

	All week
	PH2 PCC
	Velocity < 1.5 fps
	No acoustic sensor
	Will repair during winter 04/05
	


4.3 Fish Ladder Temperatures.   Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Fahrenheit.

	Date
	Wash. Shore
	Bradford Is.
	AFF exit

	6/27
	65.6
	65.9
	 - - 

	6/28
	66.0
	66.1
	66.0

	6/29
	66.4
	66.5
	66.4

	6/30
	66.6
	66.8
	66.7

	7/1
	67.2
	67.3
	67.2

	7/2
	67.3
	67.5
	67.3

	7/3
	67.7
	68.0
	67.7


4.4 Sensor Calibration.  Calibration was checked on 2 July this reporting week.

4.5 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week. 

4.6 Fishway Status.  

4.6.1 Cascade Island returned to auto. 

5 CONSTRUCTION

5.1 PH 2 Corner Collector Hi-Q PIT tag detector.  If feasible, the prototype antenna will then be constructed into the existing channel during the in-water work period during FY05.  
5.2 SMF hydrocannons.  The hydro cannons at the SMF are not operational at this time.  Repairs to the avian cannon are scheduled for completion by 15 July.  Wildlife services will conduct full time hazing activities until 17 July or until the hydro cannon is operational.     

5.3 Adult PIT tag exit detectors.  Current schedule is set for installation in WA ladder for Winter 04/05.

5.4 PH 1 Gantry Crane Delivery and Erection.  Scheduled for 9 August through 4 December.    

6 MISCELLANEOUS AND OTHER NOTEWORTHY ITEMS

6.1 On 17 June, a new spill teletype was sent, detailing the dates for BiOp (75kcfs) and 50kcfs spill.  The pre-test started on 17 June.  The testing period is from 20 June through 07 August.

6.2 On 2 July, U of I had a radio tagged unclipped adult steelhead and a summer chinook in their hauling tank that were listless, paled, and had bruise marks along their posterior dorsal surfaces.  They were moved to a recovery tank and appeared as if they would not survive.  The radio tags were removed and the fish spent about one hour in the recovery tank.  After the fish were able to stay upright, they were released into the brail pool.  The gate was opened and they swam on their own volition into the ladder.  On 5 July, a steelhead carcass was found in the glory hole.  A PIT tag scan confirmed it was the same steelhead from 2 July.  

JAMES R. MAHAR, P.E.

Operations Manager

//S/

1
2

