CENWP-OP-B                                                                                                                                     19 July 2004

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 29 of 2004, which covers the period 11 July through 17 July 2004.

1.  OPERATION SUMMARY.

1.1 Daily river flow averaged 145.2 to 182.9 kcfs.  Daily project tailwater elevation averaged 11.9’ to 14.3’ msl.  Daily project forebay elevation averaged 72.4’ to 75.0’ msl.  Daily river temperature averaged 65 to 68°F.  Daily turbidity measured 4.0 to 7.0 on the secchi disk.  

1.2 Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B Branch respectively, are part of the FPP spill pattern.  Spill averaged 49.3 to 93.9 kcfs.   BiOp versus 50kcfs testing started on 17 June. 

1.3 Unit operation.  

1.3.1 Unit 1 remains out of service for turbine rehab.

1.3.2 Unit 15 remains on manual 3D cam adjustment.  New modules are on order.  

1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.  

1.5 Gatewell drawdowns are measured on Wednesdays.  Drawdowns are measured daily on Unit 17.

2.  ACTIVITIES. 
2.1 AWS Closures.  No AWS valves were closed this reporting week:

2.2 STS/VBS Inspections.    No inspections were conducted this week. 

2.3 Dewatering and Fish Salvage.  No dewatering or salvage activities occurred this reporting week.

3.  RESEARCH.
3.1 Battelle/BAE Systems FGE testing.  Data collection ended on 16 July.

3.2 CRITFC.  Sampling twice a week.

3.3 ERDC TDG.  Sensors remain installed in the tailrace.

3.4 NOAA FISH lamprey.  Lamprey ladder installation complete.  

3.5 PSMFC SMF.  No fish were observed sliding over the separator bars this week.   

3.6 USGS FPE testing.  Releasing of radio tagged fish continues.   Last fish release scheduled for 31 July.  PSMFC collected 1,120 subyearling chinook for USGS RT testing.

3.7 University of Idaho.  Sampling daily in the AFF.  

3.8 Wildlife Services/Avian Abatement.  Birds were hazed this reporting week with rockets, cracker shells, and 15mm pyrotechnics.  None were lethally taken.

3.9 WDFW Fish Counting.  16-hr visual counts at Washington Shore and Bradford Island.

4.  FISHWAYS 
4.1 Project biologists inspected 11 July through 17 July this reporting week.  

4.2 Fish Passage Plan Observations.

	DATE
	LOCATion
	FPP Deviation
	CAUSE
	RESPONSE
	TR#

	All week
	PH1 PCC
	Velocity <1.5 fps
	Meter in slowest part of the channel
	This is a normal condition.
	

	7/14, 7/16-7/17
	FG3-4
	Incorrect position for TW
	Bent shaft
	No action taken.
	

	All week
	PH2 PCC
	Velocity < 1.5 fps
	No acoustic sensor
	Will repair during winter 04/05
	


4.3 Fish Ladder Temperatures.   Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Fahrenheit.

	Date
	Wash. Shore
	Bradford Is.
	AFF exit

	7/11
	68.4
	68.4
	68.4

	7/12
	68.9
	69.1
	68.9

	7/13
	69.5
	69.6
	69.6

	7/14
	70.0
	70.2
	70.0

	7/15
	70.2
	70.4
	70.3

	7/16
	70.5
	70.7
	70.5

	7/17
	70.6
	70.8
	70.6


4.4 Sensor Calibration.  Calibration was checked on 17 July this reporting week.

4.5 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week. 

4.6 Fishway Status.  

4.6.1 FV 4-3 went to manual at 1421 on 8 July due to a bad ladder elevation sensor.  FV 4-3 returned to auto on 16 July.  

4.6.2 Orifice 17A-N in DSM2 has been sticking in the closed position.  A TR was submitted on 14 July.  

5 CONSTRUCTION

5.1 PH 2 Corner Collector Hi-Q PIT tag detector.  If feasible, the prototype antenna will be constructed into the existing channel during the in-water work period during FY05.  
5.2 SMF hydrocannons.  The hydro cannons at the SMF are operational at this time.  Repairs to the avian cannon were completed on 14 July. 

5.3 Adult PIT tag exit detectors.  Current schedule is set for installation in WA ladder for Winter 04/05.

5.4 PH 1 Gantry Crane Delivery and Erection.  Scheduled for 9 August through 4 December.    

6 MISCELLANEOUS AND OTHER NOTEWORTHY ITEMS

6.1 On 17 June, a new spill Teletype was sent, detailing the dates for BiOp (75kcfs) and 50kcfs spill.  The pre-test started on 17 June.  The testing period is from 20 June through 07 August.

6.2 On 6 July, a new Teletype was sent indicating a restricted Forebay elevation not to exceed 72.0 MSL between 0700 and 1700 hours on 12 and 13 July to assist final work on the Drano Lake boat launch. 

JAMES R. MAHAR, P.E.

Operations Manager

//S/

1
2

