CENWP-OP-B                                                                                                                                     3 August 2004

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 31 of 2004, which covers the period 25 July through 31 July 2004.

1.  OPERATION SUMMARY.

1.1 Daily river flow averaged 131.3 to 172.0 kcfs.  Daily project tailwater elevation averaged 10.4’ to 13.9’ msl.  Daily project forebay elevation averaged 75.6’ to 76.2’ msl.  Daily river temperature averaged 69 to 70°F.  Daily turbidity measured 4.5 to 7.0 on the secchi disk.  

1.2 Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B Branch respectively, are part of the FPP spill pattern.  Spill averaged 50.6 to 93.8 kcfs.   BiOp versus 50kcfs testing started on 17 June. 

1.3 Unit operation.  

1.3.1 Unit 0 went out of service at 0805 on 29 July for a 5-year overhaul.

1.3.2 Unit 1 remains out of service for turbine rehab.

1.3.3 Unit 15 remains on manual 3D cam adjustment.  New modules are on order. 

1.3.4 Unit 18 remains out of service for 4-year overhaul.   

1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.  

1.5 Gatewell drawdowns are measured on Wednesdays.  Drawdowns are measured daily on Unit 17.

2.  ACTIVITIES. 
2.1 AWS Closures.  No AWS valves were closed this reporting week:

2.2 STS/VBS Inspections.    No STS/VBS inspections were conducted this week. 

2.3 Dewatering and Fish Salvage.  The following dewatering or salvage activities occurred this reporting week.  Unit 0 was dewatered on 29 July.   Biologists removed - One dead lamprey, one dead juvenile smallmouth bass, one dead sturgeon, one live subyearling chinook, and one live 3-spine stickleback. 
3.  RESEARCH.
3.1 CRITFC.  Sampled one day this reporting week.

3.2 ERDC TDG.  Sensors remain installed in the tailrace.

3.3 NOAA FISH lamprey.  Lamprey tagging continues.  

3.4 PSMFC SMF.  No salmonids were observed sliding over the separator bars this week.   

3.5 USGS FPE testing.  Last tagged fish release occurred on 31 July.  PSMFC collected 1,790 subyearling chinook for USGS RT testing.

3.6 University of Idaho.   Sampled one day this reporting week.

3.7 WDFW Fish Counting.  16-hr visual counts at Washington Shore and Bradford Island.

4.  FISHWAYS 
4.1 Project biologists inspected 25-28 and 30-31 July this reporting week.  

4.2 Fish Passage Plan Observations.

	DATE
	LOCATion
	FPP Deviation
	CAUSE
	RESPONSE
	TR#

	All week
	PH1 PCC
	Velocity <1.5 fps
	Meter in slowest part of the channel
	This is a normal condition.
	

	All week
	PH2 PCC
	Velocity < 1.5 fps
	No acoustic sensor
	Will repair during winter 04/05
	

	7/29
	Bonneville Fishways
	No inspection by Fisheries 
	
	
	

	7/30-31
	PH1 chaingates
	Chaingates 10B and 10C were open.
	Unknown
	Riggers to reset gates.
	


4.3 Fish Ladder Temperatures.   Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Measurements for the AFF are taken in the exit ladder.  Temperatures are in Fahrenheit.

	Date
	Wash. Shore
	Bradford Is.
	AFF exit

	7/25
	72.0
	72.0
	72.1

	7/26
	71.9
	71.8
	72.1

	7/27
	72.0
	72.1
	72.2

	7/28
	72.5
	72.5
	72.6

	7/29
	72.3
	72.5
	72.4

	7/30
	72.4
	72.3
	72.6

	7/31
	72.5
	72.5
	72.7


4.4 Sensor Calibration.  Calibration was checked on 30 July this reporting week.

4.5 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.  

5 CONSTRUCTION

5.1 PH 2 Corner Collector Hi-Q PIT tag detector.  Tests are not proving feasible.  Development of the final antenna and contract have been halted. 
5.2 Adult PIT tag exit detectors.  Current schedule is set for installation in WA ladder for Winter 04/05.

5.3 PH 1 Gantry Crane Delivery and Erection.  Scheduled for 9 August through 4 December.    

6 MISCELLANEOUS AND OTHER NOTEWORTHY ITEMS

6.1 From 25-26 July, Cascades Island computer was in manual due to failure to properly control.

6.2 On 30 July a revision to the spill teletype from 17 June was sent out.  As of 01 August at 0500, spill will be provided for fish passage according to the BiOp.  BiOp spill is 75 Kcfs during the day and to the 120% gas cap at night.

JAMES R. MAHAR, P.E.

Operations Manager

//S/

1
2

