CENWP-OP-B                                                                                                                                  31 October 2004

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 44 of 2004, which covers the period from 24 through 30 October 2004.

1.  OPERATION SUMMARY

1.1 Daily river flow averaged 104.8 to 127.8 kcfs.  Daily project tailwater elevation averaged 10.7’ to 12.5’ msl.  Daily project forebay elevation averaged 74.8’ to 76.2’ msl.  Daily river temperature averaged 58 to 61(F.  Daily turbidity measured 7.0 on the secchi disk.  

1.2 Spill Bays 1 and 18, which supply adult attraction flow for Cascades Island and B Branch respectively, are open during daylight hours (0500 until 1930).  Spill averaged 2.2 to 2.3 kcfs.  

1.3 Unit operation.  

1.3.1 Unit 0 remains out of service for a 5-year overhaul.

1.3.2 Unit 2 remains out of service for turbine rehab.

1.3.3 Unit 10 remains out of service for gantry crane three construction.

1.3.4 Unit 15 3D CAM remains in manual.  

1.3.5 Unit 18 remains out of service for 4-year overhaul.   

1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.   

1.5 Gatewell drawdowns are measured on Wednesdays.  Drawdowns are measured daily on Unit 17.

2.  ACTIVITIES

2.1 AWS Closures.  No AWS valves were closed this reporting week:

2.2 STS/VBS Inspections.  No STSs/VBSs were inspected this reporting week. 

2.3 Dewatering and Fish Salvage.  No dewatering or salvage activities occurred this reporting week.  
3.  RESEARCH
3.1 CRITFC.  Sampling concluded on 26 October.

3.2 ERDC TDG.  Sensors remain installed in the tailrace.

3.3 PSMFC SMF.   One steelhead and two salmon were observed sliding over the separator bars this week.   Monitoring concluded at 0700 on 29 October.

3.4 USGS.  Monitoring continues.

3.5 WDFW Fish Counting.  16-hr visual counts at Washington Shore and Bradford Island.

4.  FISHWAYS

4.1 Project biologists inspected 24, 25, 27-30 October this reporting week.  

4.2 Fish Passage Plan Observations.

	DATE
	LOCATion
	FPP Deviation
	CAUSE
	RESPONSE
	TR#

	All week
	PH1 PCC
	Velocity <1.5 fps
	Meter in slowest part of the channel
	This is a normal condition.
	

	All week
	PH2 PCC
	Velocity < 1.5 fps
	No acoustic sensor
	Will repair during winter 04/05
	

	10/26
	Bonneville Dam
	No inspection by Fisheries
	Scheduling conflicts
	
	

	All week
	Bonneville Dam
	No calibration check.
	Scheduling conflicts
	
	

	10/30
	B-Branch
	TW/Entrance head differential <1.0’
	Unknown
	Operators corrected
	


Table 2.  FPP Out of Criteria items.

4.3 Fish Ladder Temperatures.   Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Fahrenheit.

	Date
	Wash. Shore daily avg.
	Bradford Is. daily avg.

	10/24
	60.5
	60.3

	10/25
	59.9
	59.8

	10/26
	59.3
	59.2

	10/27
	59.5
	59.4

	10/28
	58.9
	58.9

	10/29
	58.2
	58.1

	10/30
	57.8
	57.7


Table 3.  Fishway Temperatures.

4.4 Sensor Calibration.  Calibration was not checked this reporting week.

4.5 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.  

5.   CONSTRUCTION

5.1 Adult PIT tag exit detectors.  Washington Shore construction to start 29 November.

5.2 PH 1 Gantry Crane Delivery and Erection.  Construction will continue through 04 December.    

5.3 North Monolith repairs.  Plans and specs not yet completed.  Hope to have repairs completed by 28 February.

6.  FISHWAY STATUS

6.1 Adult Fishways.  

6.1.1 Due to the Cascades Island computer not properly controlling, the fishway is now in manual and maintaining criteria.  

6.2 Juvenile Fishways.

6.2.1 DSM1 continues to run south with STSs and VBSs in Units 1 and 3.

6.2.2 The SMF was switched to bypass at 0700 on 29 October.  

7.  HAZMAT, SPILLS AND CLEANUP
Nothing to report this week.

8.  MISCELLANEOUS AND OTHER NOTEWORTHY ITEMS
8.1 A teletype issued on 25 October requests a forebay soft constraint of 76’ msl at 0600 on 26 October.  This is to facilitate the removal of a pleasure craft from a sand bar at the Klickitat river.

8.2 A teletype issued on 29 October requests a tailwater hard constraint minimum of 10.5’ msl 24 hours a day and between 10.5’ to 12’ msl between 0700 until 1900.  These constraints go into effect starting 01 November and continuing until further notice.  This operation is to maintain water levels over spawning habitat below Bonneville Dam.

JAMES R. MAHAR, P.E.

Operations Manager

//S/

1

