CENWP-OP-B                                                                                                                                  28 November 2004

MEMORANDUM FOR CENWP-OP, ATTN: Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 48 of 2004, which covers the period from 21 November through 27 November 2004.

1.  OPERATION SUMMARY

1.1 Daily average river flows ranged from 114.5 to 134.0 kcfs.  Daily average project tailwater elevation ranged from 11.4’ to 12.4’ msl.  Daily average project forebay elevation ranged from 73.6’ to 76.2’ msl.  Daily average river temperature ranged stayed at 51(F.  Daily turbidity measured 7.0 on the secchi disk.  

1.2 Spill Bays 1 and 18, which supply adult attraction flow for Cascades Island and B Branch respectively, are open during daylight hours (0500 until 1930).  Daily average spill ranged from 2.2 to 2.3 kcfs.  

1.3 Unit operation.  

1.3.1 Unit 0 remains out of service for a 5-year overhaul.

1.3.2 Unit 2 remains out of service for turbine rehab.

1.3.3 Unit 10 remains out of service for gantry crane three construction.

1.3.4 Unit 11 remains out of service for unit overhaul.

1.3.5 Unit 12 had a 1% violation on 23 November due to STS inspections.

1.3.6 Unit 13 had a 1% violation on 22 November due to STS inspections.

1.3.7 Unit 15 was out of service from 1218 on 21 November until 0845 on 24 November due to a loss of governor control.

1.3.8 Unit 16 was out of service from 1242 until 1825 on 22 November and from 0940 until 1735 on 23 November for index testing.

1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.   

1.4.1 Fish Unit 1 went out of service at 0823 on 27 November to facilitate the Washington Shore fishway dewatering.

1.4.2 Fish Unit 2 went out of service at 0824 on 27 November to facilitate the Washington Shore fishway dewatering.

1.5 Gatewell drawdowns are measured on Wednesdays.  Drawdowns are measured daily on Unit 17.

2.  ACTIVITIES

2.1 AWS Closures.  The following AWS valves were closed this reporting week.

2.1.1 FV 6-9- closed at 0748 on 27 November due to the Washington Shore fishway going to orifice flow.

2.2 STS/VBS Inspections.  The following  STSs/VBSs were inspected this reporting week. 

	Unit
	Date
	Run Hours
	Comments

	12
	11/23
	623
	

	13
	11/22
	902
	

	14
	11/22
	832
	

	15
	11/22
	780
	

	16
	11/22
	763
	

	17
	11/22
	961
	

	18
	11/22
	145
	


Table 1.  STS/VBS Inspections. 

2.3 Dewatering and Fish Salvage.  No dewatering or salvage activities occurred this reporting week.  
3.  RESEARCH
3.1 ERDC TDG.  Sensors remain installed in the tailrace.

3.2 WDFW Chum.  Chum trapping and tagging concluded 26 November.

3.3 WDFW Fish Counting.  Video counting continues at Washington Shore and Bradford Island and started at Cascades Island on 24 November.

4.  FISHWAYS

4.1 Project biologists inspected 21-27 November.  

4.2 Fish Passage Plan Observations.

	DATE
	LOCATion
	FPP Deviation
	CAUSE
	RESPONSE
	TR#

	All week
	PH1 PCC
	Velocity <1.5 fps
	Slow water
	Normal condition.
	

	All week
	PH2 PCC
	Velocity < 1.5 fps
	No acoustic sensor
	Replace this winter
	

	11/23-24, 26
	Weir 37 &38
	<1.0’>
	AFF trapping
	No action
	

	11/24
	FG3-4
	Closed
	Broken diffuser
	Repairs in 05/06
	

	All week
	Calibration
	Not checked
	
	
	


Table 2.  FPP Out of Criteria items.

4.3 Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are in Fahrenheit.

	Date
	Wash. Shore daily avg.
	Bradford Is. daily avg.

	11/21
	51.2
	51.3

	11/22
	50.9
	50.9

	11/23
	50.4
	50.7

	11/24
	50.9
	51.1

	11/25
	51.1
	51.3

	11/26
	50.8
	51.0

	11/27
	50.5
	50.6


Table 3.  Fishway Temperatures.

4.4 Sensor Calibration.  Calibration was  not checked this reporting week.

4.5 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.  

5.   CONSTRUCTION

5.1 Adult PIT tag exit detectors.  Washington Shore construction to start 29 November.

5.2 PH 1 Gantry Crane Delivery and Erection.  Construction continues.    

5.3 North Monolith repairs.  Plans and specs not yet completed.  

5.4 B2 Follow on.  Mods to the perf plate and the bypass flume to be completed in February 2005.

6.  FISHWAY STATUS

6.1  Adult Fishways.    
6.1.1 Cascades Island fishway PLC remains in manual and maintaining criteria.  
6.1.2 Cascades Island exit was opened and the north UMT bulkhead was installed on 24 November.
6.1.3 Washington Shore and the AFF went to orifice flow on 27 November.
6.2 Juvenile Fishways.  

6.2.1 DSM1 continues to run south with STSs and VBSs in Units 1 and 3.
6.2.2 DSM2 mechanical screen sweeps were operated on 21 and 22 November.  
6.2.3 The Fish Unit orifices were opened on 22 November to compensate for Unit 11 orifices being closed.

6.3 UPCOMING ACTIVITIES

6.3.1 Washington Shore fishway dewatering and fish salvage on 29 November.

6.3.2 UMT dewatering and fish salvage on 30 November.

6.3.3 South Monolith dewatering pumping to begin 06 December, fish salvage on 13 December.

6.3.4 STS removal for both powerhouses- 15 December

6.3.5 DSM1 channel dewatering and fish salvage on 16 December.

6.3.6 DSM2/Two mile pipe dewatering and fish salvage on 20/21 December.

7.  HAZMAT, SPILLS AND CLEANUP- Nothing to report this week.  
8.  TELETYPES AND OTHER NOTEWORTHY ITEMS
8.1 A teletype issued on 19 November requests the following tailwater elevations to facility a chum spawning study on 24-27 November.  The study requires ramping flows up and then down over a 10 hour period to determine how flow velocity affects chum behavior.  This will be the second of five day/night tests.

8.1.1 24 November-

0500 hrs increase TW from 11.5’ to 12.7’

0700 hrs increase TW from 12.7’ to 13.9’

0900 hrs increase TW from 13.9’ to 15.1’

1100 hrs decrease TW from 15.1’ to 13.9’

1300 hrs decrease TW from 13.9’ to 12.7’

1500 hrs decrease TW from 12.7’ to 11.5’

8.1.2 26 November through 27 November-

1600 hrs. increase TW from 11.5’ to 12.7’

1800 hrs. increase TW from 12.7’ to 13.9’

2000 hrs. increase TW from 13.9’ to 15.1’

2200 hrs. decrease TW from 15.1’ to 13.9’

2400 hrs. decrease TW from 13.9’ to 12.7’

0200 hrs. decrease TW from 12.7’ to 11.5’

8.2 A 26 November teletype requests a change in powerhouse priority and PH2 unit priority in preparation for the Washington Shore fishway dewatering on 29 November.  The changes are effective at 0000 hours on 27 November.  

8.2.1 Switch powerhouse operating priority from powerhouse 2 to powerhouse 1.

8.2.2 Change powerhouse 2 unit operating priority to 18, 12, 17, 13-16 until Unit 11 comes back in service.

8.3 Embrace a Stream grant application for Bass Lake monitoring and public workshop to be submitted to Trout Unlimited by 20 December.

JAMES R. MAHAR, P.E.

Operations Manager

//S/

1

