CENWP-OP-B                                                                                                                                  07 February 2005

MEMORANDUM FOR CENWP-OP, ATTN: Randy Bailey

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 06 of 2005, which covers the period from 30 January through 05 February 2005.

1.  OPERATION SUMMARY

1.1 Daily average river flows ranged from 130.8 to 167.5 kcfs.  Daily average project tailwater elevation ranged from 12.8’ to 14.9’ msl.  Daily average powerhouse forebay elevation ranged from 74.7’ to 75.6’ msl.  Daily average river temperature averaged 39(F.  Daily turbidity measured 7.0 on the secchi disk.  

1.2 Spill Bay 1, which supplies adult attraction flow for Cascades Island, is open during daylight hours.   Daily average spill ranged from 1.0 to 1.1 kcfs.    

1.3 Unit operation.  

1.3.1 Unit 0 remains out of service for a 5-year overhaul.

1.3.2 Unit 2 remains out of service for turbine rehab.

1.3.3 Unit 10 remains out of service for gantry crane three construction.

1.3.4 Unit 11 returned to service at 1153 on 31 January.  Unit 11 was out of service from 2135 on 03 February until 1707 on 04 February due to a PSS failure.

1.3.5 Unit 12 was out of service from 0648 on 31 Jan until 1355 on 02 February for bulkhead mods.

1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.  

1.4.1 Fish Unit 1 remains out of service for winter maintenance at the Washington Shore fishway.

1.4.2 Fish Unit 2 remains out of service for winter maintenance at the Washington Shore fishway.

1.5 Gatewell drawdowns are measured on Wednesdays.  Drawdowns are measured daily on Unit 17.

2.  ACTIVITIES

2.1 AWS Closures.  The following AWS valves activity occurred this reporting week.

2.1.1 FV3-9- Closed for cleaning from 1245 until 1300 on 05 February.

2.1.2 FV 6-9- remains closed.

2.2 STS/VBS Inspections.  STSs/VBSs for PH2 have been scheduled for installation the week of 21 February. 

2.3 Dewatering and Fish Salvage.  No dewatering or salvage activities occurred this reporting week.  
3.  RESEARCH
3.1 ERDC TDG.  Sensors remain installed in the tailrace.

3.2 Univ of Idaho.  Installing and repairing antennas in the north monolith and Washington Shore fishway.  Installation of a half duplex PIT tag detector system is scheduled for mid to late February.

3.3 NOAA Fish.  Prototype lamprey flume to be installed 24 February.  Additional installation of above water equipment will occur in April.
3.4 WDFW Fish Counting.  Video counting continues at Bradford Island and Cascades Island.

4.  FISHWAYS

4.1 Project biologists inspected 31 January through 03 February.

4.2 Fish Passage Plan Observations.

	DATE
	LOCATion
	FPP Deviation
	CAUSE
	RESPONSE
	TR#

	All week
	PH1 N PCC
	Velocity <1.5 fps
	Slow water
	Normal condition.
	


Table 2.  FPP Out of Criteria items.

4.3 Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  Temperatures are not recorded during the winter maintenance period.

4.4 Sensor Calibration.  Calibration was checked on 03 February this reporting week.

4.5 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.  

5.   CONSTRUCTION

5.1 Adult PIT tag exit detectors.  Washington Shore construction continues.

5.2 PH 1 Gantry Crane Delivery and Erection.  Construction continues.    

5.3 North Monolith repairs.  Construction tentatively scheduled for February.  

5.4 B2 Follow on.  Mods to the perf plate and the bypass flume to be completed in February 2005.

6.  FISHWAY STATUS

6.1  Adult Fishways.    
6.1.1 Bradford Island and Cascades Island fishway PLC remains in manual and maintaining criteria.   The Cascades Island exit is open.
6.1.2 A-branch diffusers have been reprogrammed to compensate for FG3-4 being out of service.  FG3-4 will remain out of service until the winter maintenance period 2005/06.
6.1.3 Washington Shore fishway, AWS, and AFF remain dewatered.  
6.2 Juvenile Fishways.  

6.2.1 DSM 1 remains dewatered.

6.2.2 DSM2 remains dewatered.

6.3 Upcoming Events.

6.3.1 ROV inspection of BI and Cascades Island scheduled for 22 February.
6.3.2 Installation of a guide wall, lamprey flume and half –duplex PIT tag detectors will occur on 24 February at the north monolith NDE.  Divers are expected to complete the work in one day.
7.  HAZMAT, SPILLS AND CLEANUP – Nothing to report this week.
8.  TELETYPES AND OTHER NOTEWORTHY ITEMS – Nothing to report this week.
JAMES R. MAHAR, P.E.

Operations Manager

//S/

1

