CENWP-OP-B                                                                                                                                  10 April 2005

MEMORANDUM FOR CENWP-OP, ATTN: Randy Bailey

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 15 of 2005, which covers the period from 03 through 09 April 2005.

1.  OPERATION SUMMARY

1.1 Daily average river flows ranged from 122.1 to 170.6 kcfs.  Daily average project tailwater elevation ranged from 12.4’ to 15.1’ msl.  Daily average powerhouse forebay elevation ranged from 75.0’ to 75.8’ msl.  Daily average river temperature ranged from 46 to 48(F.  Daily turbidity measured 4.0 to 6.0 on the secchi disk.  

1.2 Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-Branch respectively, are open 0.5’ during daylight hours.   Daily average spill ranged from 1.4 to 1.6 kcfs.    

1.3 Unit operation.  

1.3.1 Unit 0 remains out of service for a 5-year overhaul.

1.3.2 Unit 2 remains out of service for turbine rehab.

1.3.3 Unit 8 was out of service for crane testing from 1020 until 1615 on 04 April; 1236 until 1635 on 06 April; and 0806 until 1330 on 07 April.

1.3.4 Unit 9 was out of service due to breaker problems from 0745 until 1053 on 05 April and from 0711 until 0854 on 06 April.  Unit 9 was also out of service from 1602 until 1934 on 06 April due to oil level problems.

1.3.5 Unit 10 remains out of service for gantry crane three construction.

1.3.6 Unit 11 was out of service from 0656 until 1654 on 05 April to facilitate the U11/12 trashrack dive, and from 0727 until 1159 on 06 April to facilitate the DIDSON barge installation at the B2CC.

1.3.7 Unit 12 was out of service from 0657 until 1649 on 05 April to facilitate the U11/12 trashrack dive.

1.3.8 Unit 13 was out of service from 0641 until 1128 on 05 April to facilitate the U11/12 trashrack dive.

1.3.9 Unit 15 was out of service from 1126 until 1617 on 04 April for a motor adjustment.

1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.  

1.4.1 Fish Unit 1 was out of service from 0140 until 0247 due to a loss of governor control.

1.5 Gatewell drawdowns are measured on Wednesdays.  Drawdowns are measured daily on Unit 17.

2.  ACTIVITIES

2.1 AWS Closures.  The following AWS valves were closed this reporting week.  

2.1.1 FV3-9.  Closed for cleaning from 1459 until 1522 on 04 April.

2.2 STS/VBS Inspections.   No STS/VBSs were inspected this reporting week.  

2.3 Dewatering and Fish Salvage.  No dewatering or salvage activities occurred this reporting week.  
3.  RESEARCH
3.1 CRITFC.  Will begin sampling the week of 10 April.

3.2 ERDC TDG.  Sensors remain installed in the tailrace.

3.3 Univ of Idaho.  Nothing new to report.  

3.4 NOAA Fish.  Nothing new to report.
3.5 PSMFC.  One live kelt was removed and 18 adult steelhead were observed crossing the separator bars. 
3.6 USGS FPE and Survival.  Installing radio telemetry equipment. 
3.7 Battelle FPE and FGE.  DIDSON barge installed on 06 April.  Installing hydroacoustic transducers.    
3.8 WDFW Fish Counting.  16-hr visual counting at Washington Shore and Bradford Island. 

3.9 USDA Wildlife Services.   Hazing activities of gulls continues.  

4.  FISHWAYS

4.1 Project biologists inspected 03 through 09 April.  

4.2 Fish Passage Plan Observations.

	DATE
	LOCATion
	FPP Deviation
	CAUSE
	RESPONSE
	TR#

	All week
	PH1 N PCC
	Velocity <1.5 fps
	Slow water
	Normal condition.
	

	All week
	PH2 SCCH
	Velocity <1.5 fps
	Improper installation
	No action taken until S. monolith can be dewatered.
	

	4/3-7
	Cascades Island
	FG6-12 OOS
	Bent shaft
	FG6-11 to compensate see 6.1.2
	

	4/4
	Weir 37
	Reading <1.0’
	AFF water-up
	Allow water levels to equalize
	

	4/8-9
	Gatewells 15B and 16A
	Debris cover >50%
	Debris load in the river
	Riggers to clean first thing Monday.
	


Table 2.  FPP Out of Criteria items.

4.3 Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island as well as at the junction of the UMT and the main ladder at Washington shore.  

	Date
	Wash. Shore
	Bradford Is.

	4/3
	46.5
	46.8

	4/4
	46.8
	46.9

	4/5
	46.9
	47.1

	4/6
	47.2
	47.3

	4/7
	47.2
	47.4

	4/8
	47.3
	47.4

	4/9
	47.3
	47.5











  Table 3.  Bonneville Fishway temperatures.

4.4 Sensor Calibration.  Calibration was checked on 08 April.  

4.5 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.  

5.   CONSTRUCTION

5.1 PH 1 Gantry Crane Delivery and Erection.  Crane testing performed this reporting week.

5.2 B2 VBS modifications.  Gatewell modifications are scheduled to begin in Winter 05/06.

5.3 PH1 STS/VBS removal and TIES removal.  Six TIES and 23 PH1 STSs/VBSs will be removed in August.

6.  FISHWAY STATUS

6.1  Adult Fishways.    
6.1.1 Bradford Island and Cascades Island fishway PLCs are in manual.  
6.1.2 Cascades Island diffuser FG6-11 will be used to compensate for FG6-12’s bent shaft.  FG6-12 will remain out of service until winter 2006/07.
6.1.3 A-branch has been reprogrammed.  FG3-4 will remain out of service until winter 2005/06.
6.1.4 Bradford Island window washer #1 returned to service on 06 April.
6.2 Juvenile Fishways.  

6.2.1 SMF booster pump is on order. 

6.2.2 DSM2 mechanical screen sweeps operated on 08 April.

6.2.3 PH1 ITS closed from 1425 until 1432 on 07 April to facilitate log removal activities.

7.  HAZMAT, SPILLS AND CLEANUP – Nothing to report this week.
8.  TELETYPES AND OTHER NOTEWORTHY ITEMS.
8.1 Marine mammals have been observed in the tailrace, the forebay, and the fishways.  The project is not taking action to harass, harm, or otherwise annoy these animals.

8.2 Kelts were seen in front of chaingate 7A this week.

JAMES R. MAHAR, P.E.

Operations Manager

//S/

1

