CENWP-OP-B                                                                                                                                  17 October 2005

MEMORANDUM FOR CENWP-OP, ATTN: Randy Bailey

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 42 of 2005, which covers the period from 09 through 15 October 2005.

1.  OPERATION SUMMARY

1.1 Daily average river flows ranged from 107.4 to 120.0 kcfs.  Daily average project tailwater elevation ranged from 10.2’ to 10.8’ msl.  Daily average powerhouse forebay elevation ranged from 69.2’ to 75.0’ msl.  Daily average river temperature ranged from 62 to 64(F.  Secchi disk measurements averaged 7.0 feet.

1.2 Daily average spill ranged from 1.1 to 1.3 kcfs.   Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-Branch respectively, are open 6” during daylight hours.  

1.3 Unit operation.

1.3.1 Unit 0 remains out of service due to governor trouble.

1.3.2 Unit 2 remains out of service for turbine rehab. 

1.3.3 Unit 3 was out of service from 0743 until 1415 on 15 October for BPA RC#1 outage.  

1.3.4 Unit 4 was out of service from 0743 until 1415 on 15 October for BPA RC#1 outage.  

1.3.5 Unit 5 was out of service from 0741 until 1415 on 15 October for BPA RC#2 outage.  

1.3.6 Unit 6 remains out of service for a five-year overhaul.  

1.3.7 Unit 7 was out of service from 0741 until 1415 on 15 October for BPA RC#2 outage.

1.3.8 Unit 8 was out of service from 0745 until 1415 on 15 October for BPA RC#2 outage.

1.3.9 Unit 10 remains out of service for gantry crane three construction.

1.3.10 Unit 11 remains out of service due to a rotor failure.

1.3.11 Unit 12 went out of service at 0436 on 13 October for MUB replacement.

1.3.12 Unit 13 was out of service from 0436 until 1156 on 13 October for T11 outage.  Was out of service from 0748 until 1433 on 15 October for BPA RC#3 outage.

1.3.13 Unit 14 was out of service from 0437 until 1156 on 13 October for T11 outage.  Was out of service from 0748 until 1426 on 15 October for BPA RC#3 outage.

1.3.14 Unit 15 was out of service from 0303 until 1659 on 11 October for T12 outage. 

1.3.15 Unit 16 was out of service from 0307 on 11 October until 1058 on 13 October for T12 outage.

1.3.16 Unit 17 returned to service at 1506 on 12 October.   

1.3.17 Unit 18 returned to service at 1048 on 13 October.   

1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby from approximately 0000-0300 nightly if drawdown exceeds 1.0’.

1.5 Gatewell drawdowns are measured on Wednesdays.  Drawdowns are measured daily on Unit 17.

2.  ACTIVITIES

2.1 AWS Closures.  The following AWS valves were closed this reporting week

2.1.1 FV3-7- closed from 1048 until 1105 on 11 October for cleaning.

2.1.2 FV3-9- closed from 1048 until 1105 on 11 October for cleaning.

2.2 STS/VBS Inspections.  No STSs/VBSs were inspected this reporting week.  

2.3 Dewatering and Fish Salvage.  No dewatering or salvage activities occurred this reporting week. 
3.  RESEARCH
3.1 CRITFC.  Sampling two days a week continues.  

3.2 ERDC TDG.  Sensors remain installed in the tailrace.

3.3 NOAA Fish PIT tag.  Tagging finished.

3.4 PSMFC.  Three fallback salmon were observed crossing the separator bars this week.  A power outage occurred at the facility on 13 October. The facility switched to bypass mode.  Approximately 30 minutes of sample collections and PIT tag interrogation were interrupted before normal operations resumed.
3.5 WDFW Fish Counting.  16-hr visual counting at Washington Shore and Bradford Island.   

4.  FISHWAYS

4.1 Project biologists inspected 09 - 12 and 14 - 15 October.  NOAA Fisheries and FPC inspected on 13 October.

4.2 Fish Passage Plan Observations.

	DATE
	LOCATion
	FPP Deviation
	CAUSE
	RESPONSE
	TR#

	All week
	PH1 N PCC
	Velocity <1.5 fps
	Slow water
	Normal condition.
	

	All week
	PH2 SCCH
	Velocity <1.5 fps
	Improper install.
	Will repair in 06/07
	

	All week
	B-Branch 
	North gate closed
	Separated from shaft
	See 6.1.4
	

	Since 01 October
	PH1 ITS
	Chaingates not in proper configuration.
	
	Riggers to re-set chaingates on 17 October.
	

	10/09
	B branch
	Tailwater/Entrance differential high
	FV 4-4 in auto
	Operators corrected
	

	10/11
	Cascades Is
	FG6-14 open
	System in manual
	Operators closed diffuser
	

	10/11, 14
	Cascades Is
	Tailwater/Entrance differential high
	
	Operators corrected
	

	10/15
	PH 1 and 2 
	Unit priority
	River Crossing outage - BPA
	
	


Table 2.  FPP Out of Criteria items.

4.3 Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island as well as at the junction of the UMT and the main ladder at Washington shore.  

	Date
	WA Shore Average
	Bradford Is Average

	10/09
	63.6
	63.6

	10/10
	63.3
	63.2

	10/11
	63.1
	63.1

	10/12
	62.9
	62.9

	10/13
	62.5
	62.5

	10/14
	62.3
	62.4

	10/15
	62.2
	62.2


Table 3.  Bonneville Fishway temperatures.

4.4 Sensor Calibration.  Calibration was checked on 15 October.  

4.5 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.  

5.   CONSTRUCTION

5.1 B2CC PIT Tag Detector.  Installation is planned for winter 05/06.

5.2 B2 Fish Unit Trash rake.  Delivery and installation sometime before winter.

5.3 B2 VBS modifications.  Gatewell modifications to begin in Unit 13 starting early November.

5.4 Bradford Island Adult PIT Tag Detectors.  Installation scheduled for winter 05/06.

5.5 Main Unit Circuit Breaker.  Mods finished on Units 17/18.  Moving to Units 11/12.

5.6 PH1 STS/VBS removal and TIES removal.  Six TIES and 23 PH1 STSs/VBSs will be scrapped.

5.7 SMF Full flow PIT Tag Detectors.  Site visit occurred for 11 October.

6.  FISHWAY STATUS

6.1  Adult Fishways.    
6.1.1 B-Branch fishway PLC is in manual.  Cascades Island PLC is in manual.
6.1.2 Cascades Island diffuser FG6-12 has a bent shaft and remains out of service until winter 2006/07.  FG6-11 is used to compensate.
6.1.3 A-branch has been reprogrammed.  FG3-4 remains out of service until winter 2005/06.
6.1.4 B-Branch north gate has separated from the shaft.  The south gate compensates when possible.  

6.2 Juvenile Fishways.
6.2.1 DSM2 screen sweeps are exercised on Sunday evenings.

6.3 Upcoming Events.
6.3.1 The JMF will go to bypass at 0700 on 31 October.

6.3.2 The UMT is scheduled to go OOS (to accommodate VBS modification work) on 31 October.

6.3.3 Cascades Island is scheduled to open on 31 October.

7.  HAZMAT, SPILLS AND CLEANUP 
8.  TELETYPES AND OTHER NOTEWORTHY ITEMS.
8.1 Proposed FPP changes to Appendix G and the inclusion of a new appendix for the JMF are available from Project Fisheries.

8.2 The 2005/06 Winter Maintenance Schedule is available from Project Fisheries.

JAMES R. MAHAR, P.E.

Operations Manager

//S/

1

