CENWP-OP-B                                                                                                                                  27 February 2006

MEMORANDUM FOR CENWP-OP, ATTN: Randy Bailey

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 08 of 2006, which covers the period from 19 through 25 February 2006.
1.  OPERATION SUMMARY

1.1 Daily average river flows ranged from 154.5 to 192.7 kcfs.  Daily average project tailwater elevation ranged from 14.0’ to 16.4’ msl.  Daily average powerhouse forebay elevation ranged from 72.9’ to 75.6’ msl.  Daily average river temperature ranged from 39 to 40(F.  Secchi disk measurements averaged 4.3 to 6.0 feet.

1.2 Daily average spill averaged 0.6 to 0.9 kcfs.   Spill Bay 1, which supplies attraction flow for Cascades Island, is open 6” during daylight hours.  Spill Bay, which supplies attraction flow for B-Branch, was opened 6”at 1319 on 25 February.   
1.3 Unit operation.

1.3.1 Unit 0 remains out of service due to governor trouble.

1.3.2 Unit 2 remains out of service for turbine rehab.
1.3.3 Unit 3 out of service from 1439 until 1636 on 24 February for trash raking. 
1.3.4 Unit 5 out of service from 1136 until 1422 on 24 February for trash raking.

1.3.5 Unit 6 out of service from 0915 until 1134 on 24 February for trash raking.
1.3.6 Unit 7 out of service from 0915 until 1150 on 23 February for trash raking.

1.3.7 Unit 8 out of service from 1235 until 1632 on 23 February for trash raking.

1.3.8 Unit 9 out of service from 0807 until 0912 on 24 February for trash raking.

1.3.9 Unit 10 remains out of service for turbine rehab.

1.3.10 Unit 11 remains out of service due to a rotor failure.

1.3.11 Unit 12 remains out of service for VBS modifications.
1.3.12 Unit 13 out of service from 0836 until 0915 on 23 February for trash raking.

1.3.13 Unit 15 out of service from 0824 until 1017 on 22 February for trash raking.

1.3.14 Unit 16 out of service from 1020 until 1113 on 22 February for trash raking.

1.3.15 Unit 17 out of service from 1117 until 1315 on 22 February for trash raking.

1.3.16 Unit 18 out or service from 1315 until 1356 on 22 February for trash raking.
1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris.

1.5 Gatewell drawdowns are measured on Wednesdays.  Drawdowns are measured daily on Unit 17.

2.  ACTIVITIES

2.1 AWS Closures.  The following AWS valves were closed this reporting week.

2.1.1 FV1-1 closed for BI fishway overhaul.  Returned to service at 1022 on 24 February. 
2.1.2 FV3-7 closed for BI fishway overhaul.  Returned to service at 1331 on 25 February. 

2.1.3 FV3-9 closed for BI fishway overhaul.  Returned to service at 1310 on 25 February.
2.1.4 FV4-3/ 4-4 closed for BI fishway overhaul.  Returned to service at 1313 on 25 February. 

2.2 STS/VBS Inspections.  STSs/VBSs are scheduled for installation the week of 27 February.
2.3 Dewatering and Fish Salvage.  No dewatering or salvage activities occurred this reporting week. 
3.  RESEARCH
3.1 Battelle/Normandeau.  Sensor fish testing scheduled for mid to late March.
3.2 ERDC TDG.  Sensors remain installed in the tailrace.

3.3 NOAA Fisheries lamprey.  Permanent installation of the BI AWS lamprey ramp completed.
3.4 WDFW Fish Counting.  16-hr video counting at Cascades Island and Washington Shore.  

3.5 USGS.  Finished sampling for Bradford Island remediation the week of February 12th. 

4.  FISHWAYS

4.1 Project biologists inspected 21-23 February.
4.2 Fish Passage Plan Observations.

	DATE
	LOCATion
	FPP Deviation
	CAUSE
	RESPONSE
	TR#

	All week
	PH2 SCCH
	Velocity <1.5 fps
	Improper installation
	Will repair in 06/07
	

	All week
	Cascades Is.
	FG6-12 closed
	Bent shaft
	Will repair in 06/07
	


Table 2.  FPP Out of Criteria items.

4.3 Fish Ladder Temperatures.  Temperatures will be recorded starting 01 March.
4.4 Sensor Calibration.  PH1 collection channel and B-Branch were watered up on 25 February.  Sensors to be calibrated next week.  Cascades Island is out of calibration until sensors can be replaced and recalibrated.

4.5 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.  

5.   CONSTRUCTION

5.1 B1 ITS Wall Removal.  A letter report is expected in 2006.

5.2 B2CC PIT Tag Detector.  Construction continues.  The antenna has arrived and has been assembled in PH2.
5.3 B2 VBS modifications.  Mods continue on Unit 12.  VBS delivery is scheduled for the week of 27 February.
5.4 B2 Fish Unit trashrake.  According to the structural crew, the new rake does not work as well as the old rake.  The new rake is not being used at this time.

5.5 Bradford Island Adult PIT Tag Detectors.  A walk through occurred on 20 February.  PSMFC testing began 21 February.
5.6 PH1 STS/VBS removal and TIES removal.  Six TIES and 23 PH1 STSs/VBSs will be scrapped.

5.7 SMF Full flow PIT Tag Detectors.  Invert measurements and pipe inspection scheduled for 27 February.
6.  FISHWAY STATUS

6.1  Adult Fishways.    
6.1.1 Cascades Island PLC is in manual.  New sensors are scheduled to be installed and calibrated before fish passage season for both CI and B-Branch.
6.1.2 Cascades Island diffuser FG6-12 has a bent shaft and remains out of service until winter 2006/07.  
6.1.3 SLADDs and SLEDs are installed at PH2.  Installation at PH1 and the Main Dam fishways to occur throughout the month.
6.1.4 Bradford Island returned to service at 1300 on 25 February.
6.2 Juvenile Fishways.
6.2.1 DSM1 remains out of service until 15 September.

6.2.2 DSM2 scheduled to come back in service on 27 February.
6.2.3 SMF scheduled to go to sample at 0700 on 01 March.

6.3 Upcoming Events.
6.3.1 Spring Creek release scheduled for 02 March.

6.3.2 B2CC scheduled to open for Spring Creek fish passage from 03-08 March.
7.  HAZMAT, SPILLS AND CLEANUP.  Nothing to report.
8.  TELETYPES AND OTHER NOTEWORTHY ITEMS.  
8.1 A teletype issued on 31 December authorizes a minimum tailwater of 11.5’ for all hours, starting at 0000 on 01 January until further notice.  This operation is to provide chum incubation flows below Bonneville.

JAMES R. MAHAR, P.E.

Operations Manager

//S/

1

