CENWP-OP-B                                                                                                                                  13 March 2006

MEMORANDUM FOR CENWP-OP, ATTN: Randy Bailey

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 10 of 2006, which covers the period from 05 through 11 March 2006.
1.  OPERATION SUMMARY

1.1 Daily average river flows ranged from 145.4 to 185.8 kcfs.  Daily average project tailwater elevation ranged from 13.4’ to 16.2’ msl.  Daily average powerhouse forebay elevation ranged from 73.9’ to 76.0 msl.  Daily average river temperature ranged from 40 to 41(F.  Secchi disk measurements averaged 4.0 to 5.0 feet.

1.2 Daily average spill averaged 1.3 to 1.4 kcfs.   Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-Branch respectively, are open 0.5’ during daylight hours.
1.2.1 Unit 0 remains out of service due to governor trouble.

1.2.2 Unit 2 remains out of service for turbine rehab.
1.2.3 Unit 10 remains out of service for turbine rehab.

1.2.4 Unit 11 remains out of service due to a rotor failure.

1.2.5 Unit 12 remains out of service for VBS modifications.
1.3 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris.

1.4 Gatewell drawdowns are measured on Wednesdays.  Drawdowns are measured daily on Unit 17.
2.  ACTIVITIES

2.1 AWS Closures.  No AWS valves were closed this reporting week.

2.2 STS/VBS Inspections.  No STSs/VBSs were inspected this reporting week. 
2.3 Dewatering and Fish Salvage.  No dewatering or salvage activities occurred this reporting week. 
3.  RESEARCH
3.1 Battelle/Normandeau.  Sensor fish testing scheduled for late March.
3.2 ERDC TDG.  Sensors remain installed in the tailrace.
3.3 NOAA Fisheries lamprey.  Permanent installation of the BI AWS lamprey ramp completed.
3.4 PSMFC.  Sample collections and PIT tag interrogation at the Smolt Monitoring Facility (SMF) continues.  Three adult steelhead were observed crossing the separator bars this week.
3.5 USDA Wildlife Services.  Sea lion hazing began on 07 March using 15mm cracker/screamer shells and rubber bullets.

3.6 WDFW Fish Counting.  16-hr video counting at Cascades Island and Washington Shore.   
4.  FISHWAYS 
4.1 Project biologists inspected 05 March – 11 March.
4.2 Fish Passage Plan Observations.
	DATE
	LOCATion
	FPP Deviation
	CAUSE
	RESPONSE
	TR#

	All week
	PH2 SCCH
	Velocity <1.5 fps
	Improper installation
	Will repair in 06/07
	

	All week
	Cascades Is.
	FG6-12 closed
	Bent shaft
	Will repair in 06/07
	

	All week
	PH1 CCH Diffusers
	FG2-22 closed, 2-19 stuck half open
	Unknown.  Failed during water up.
	Will repair in 07/08
	

	3/7
	Cascade Is. exit
	Exit drawdown exceeded
	Debris
	Riggers notified
	


Table 2.  FPP Out of Criteria items.

4.3 Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  
	Date
	Wash. Shore
	Bradford Is.

	3/5
	40.9
	41.1

	3/6
	41.2
	41.3

	3/7
	41.5
	41.6

	3/8
	41.7
	41.9

	3/9
	41.6
	41.8

	3/10
	41.6
	41.7

	3/11
	41.6
	41.8


4.4 Sensor Calibration.  Sensors were calibrated 05 March.  Cascades Island is out of calibration until sensors can be replaced and recalibrated.

4.5 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.  

5.   CONSTRUCTION

5.1 B1 ITS Wall Removal.  A letter report is expected in 2006.

5.2 B2CC PIT Tag Detector.  Construction continues.  The antenna has arrived and has been assembled in PH2.
5.3 B2 VBS modifications.  Mods continue on Unit 12.  
5.4 B2 Fish Unit trash rake.  According to the structural crew, the new rake does not work as well as the old rake.  The new rake is not being used at this time.

5.5 PH1 STS/VBS removal and TIES removal.  Six TIES and 23 PH1 STSs/VBSs will be scrapped.

5.6 SMF Full flow PIT Tag Detectors.  Construction continues.
6.  FISHWAY STATUS

6.1  Adult Fishways.    
6.1.1 Cascades Island PLC is in manual.  
6.1.2 New sensors for B-branch and Cascades Island tailrace, entrance and ladder have been installed and calibrated.  
6.1.3 Cascades Island diffuser FG6-12 has a bent shaft and remains out of service until winter 2006/07. 
6.1.4 Cascades Island exit was closed from 1010 to 1020 on 07 March for cleaning. 
6.1.5 SLADDs and SLEDs are installed at all fishway entrances. 
6.1.6 A sea lion was observed in the Washington Shore ladder on 09 March.
6.2 Juvenile Fishways.
6.2.1 DSM1 remains out of service until 15 September.

6.2.2 B2CC was closed at 1120 on 07 March after the Spring Creek fish passage. 
7.  HAZMAT, SPILLS AND CLEANUP
8.  TELETYPES AND OTHER NOTEWORTHY ITEMS.  
8.1 A teletype issued on 31 December authorizes a minimum tailwater of 11.5’ for all hours, starting at 0000 on 01 January until further notice.  This operation is to provide chum incubation flows below Bonneville.
8.2 A teletype issued on 07 March requested that the Powerhouse 2 Corner Collector be closed at 1100 and tailwater be held to a minimum elevation of 11.5 feet
8.3 The navigation lock was taken out of service for annual maintenance at 0600 on 11 March.

JAMES R. MAHAR, P.E.

Operations Manager

//S/

1

