CENWP-OP-B                                                                                                                                  25 June 2006
MEMORANDUM FOR CENWP-OP, ATTN: Randy Bailey

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 25 of 2006, which covers the period from 18 through 24 June 2006.
1 OPERATION SUMMARY

1.1 Daily average river flows ranged from 269.2 to 326.8 kcfs.  Daily average project tailwater elevation ranged from 20.8’ to 24.0’ msl.  Daily average powerhouse forebay elevation ranged from 74.9’ to 75.8 msl.  Daily average river temperature ranged between 62( and 63(F.  Secchi disk measurements averaged 2.8 to 4.0 feet.

1.2 Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-Branch respectively, are included in the spill pattern.  Daily average spill ranged from 96.9 to 109.5 kcfs.
1.3 Unit Operation.

1.3.1 Unit 0 remains out of service due to governor trouble.
1.3.2 Unit 2 remains out of service for turbine rehab.
1.3.3 Unit 7 was out of service from 0434 until 1750 on 19 June for semi-annual maintenance.
1.3.4 Unit 8 was out of service from 0432 until 1718 on 20 June for semi-annual maintenance.

1.3.5 Unit 9 was out of service from 0444 until 1730 on 21 June for semi-annual maintenance, and from 0720 until 0900 on 22 June for slip ring work.
1.3.6 Unit 10 remains out of service for turbine rehab.

1.3.7 Unit 11 returned to service at 1549 on 19 June.  Unit 11 was out of service from 2008 until 2020 on 20 June for an emergency shutdown due to a clogged filter.
1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris. 

1.5 Gatewell drawdowns are measured on Wednesdays; VBS drawdowns are measured daily on modified units.
2 ACTIVITIES

2.1 AWS Closures.  No AWS valves were closed this reporting week.
2.2 STS/VBS Inspections.  No STSs/VBSs were inspected this reporting week.
2.3 Dewatering and Fish Salvage.  The following dewatering activities occurred this reporting week.
2.3.1 Unit 11 tail logs were pulled on 19 June.  Biologists removed nine lamprey, two catfish, one sculpin and one bass.  All fish were released to the tailrace.
3 RESEARCH
3.1 CRITFC.  Sampling adult fish in the AFF.  

3.2 CTUIR.  Collecting adult lamprey for continuation of Umatilla Basin lamprey re-introductions. 

3.3 ERDC TDG.  Sensors remain installed in the tailrace.
3.4 NOAA Fisheries.  Salt water rearing subyearling chinook at the SMF continues.    
3.5 NOAA Fisheries lamprey.  Fish collection and monitoring continues.
3.6 PSMFC smolt monitoring.  One salmon was observed crossing the separator bars.   330 subyearling chinook were collected for NOAA researchers.  
3.7 PSMFC PIT tag.  Monthly testing of the B2CC detectors June through October.
3.8 U of I adults.  South Fork Salmon R. and shad sampling continues through late July.   

3.9 U of I lamprey.  Collection and testing continue.
3.10 WDFW Fish Counting.  Visual fish counting continues.

4 FISHWAYS 

4.1 Project biologists inspected 11 through 17 June 2006.  Gary Fredricks, NOAA Fisheries inspected on 23 June.
4.2  Fish Passage Plan Observations.
	DATE
	LOCATion
	FPP Deviation
	CAUSE
	RESPONSE
	TR#

	All week
	DSM2 airburst
	Screens 1, 4-5, 11, 12 not working properly.
	Electrical
	See 6.2.3
	06-40863

	6/18, 6/23-24
	Weir 37
	Reading >1.3’
	Blocked drain
	See 6.1.2
	

	6/20
	A&B branches 
	Reading >1.3’
	Unknown
	No action taken
	

	6/22
	Cascades Is.
	FG6-8 open
	System in manual
	Operators adjusted.
	

	6/23-24
	DSM2 add-in
	Fish in the add in 
	Unknown
	Attempt rescue.
	


Table 2.  FPP Out of Criteria items.
4.2.1 PH1 electronic velocity meter is in an eddy.  Due to this placement, the velocity meter reads less than 1.5 fps.  Readings to meet FPP criteria are taken from the south end mechanical meter.
4.2.2 PH2 electronic velocity meter was incorrectly installed in the winter of 04/05.

4.2.3 Cascades Island FG6-12 is closed due to a bent shaft.  Repairs can not be made until winter 06/07.

4.2.4 PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Diffuser failure occurred during water up.  Repairs are scheduled for winter 07/08.  For now, the collection channel is operating within FPP criteria.
4.3 Fish Ladder Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  
	Date
	WS avg.
	BI avg.
	AFF max
	SMF max

	6/18
	62.3
	62.3
	62.4
	61.1

	6/19
	62.3
	62.3
	62.3
	61.0

	6/20
	62.3
	62.2
	62.4
	61.0

	6/21
	62.4
	62.4
	62.4
	61.2

	6/22
	62.6
	62.6
	62.5
	61.6

	6/23
	62.8
	62.8
	62.8
	62.0

	6/24
	63.3
	63.4
	63.3
	62.5


Table 3.  Bonneville Fishway temperatures
4.4 Sensor Calibration.  Calibration was checked on 24 June.  
4.5 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.  

5 CONSTRUCTION
5.1 B1 ITS Wall Removal.  A letter report is expected in June 2006.
5.2 PH1 STS/VBS removal and TIES removal.  Six TIES and 23 PH1 STSs/VBSs will be scrapped.

5.3 SMF Full flow PIT Tag Detectors.  Fiber optic testing occurred on 23 June.
6 FISHWAY STATUS

6.1  Adult Fishways.    
6.1.1 Cascades Island PLC is in manual.  
6.1.2 Due to the numbers of shad blocking the weir 37 drain, the southern most picket lead will be up after 1300 Mon-Thurs and at all times Fri-Sun and holidays.  
6.1.3 SLEDS were removed from PH2 on 21 June; Cascades Island and B-Branch on 22 June; PH1 on 23 June.
6.1.4 Removal of the old PH1 sluice and orifice gates and associated control equipment is complete.  These gates have been bulkheaded since 2003.  Removal of them helps streamline the channel and removes potential sources of mechanical oil/grease contaminants.
6.2 Juvenile Fishways.
6.2.1 DSM1 remains out of service until 15 September.
6.2.2 TIES remain installed.

6.2.3 DSM2 airburst screens 1, 4, 5, 11 and 12 do not operate consistently.  The Project is waiting on contractors before repairs may be completed.

6.2.4 There are adult fish in the DSM2 add-in water.  Attempts will be made on 26 June to return them to the river.

7 HAZMAT, SPILLS AND CLEANUP - Nothing to report.
8 TELETYPES AND OTHER NOTEWORTHY ITEMS. 
8.1 A teletype sent on 21 June request spill for juvenile passage be 105 Kcfs or the gas cap, whichever is less.  For spill 92 kcfs and under, use the spill patters in the 28 April spreadsheets, provided by Dennis Schwartz.

8.2 A teletype sent on 23 June requests Bonneville pool be operated with a hard constraint of 75.0’ – 76.5’ and a soft constraint of 75.5’ – 76.5’ from 0600 on 27 June until 1800 on 29 June.  This request is to reduce the wear and tear on tribal fishing gear.

8.3 A teletype sent on 23 June requests that the priority of units to be operated last on, first off be changed from Unit 8 to Unit 9.  This operation will be in effect for approximately two weeks.  This is to facilitate rehab work on Unit 10
JAMES R. MAHAR, P.E.

Operations Manager

1

