CENWP-OP-B                                                                                                                          17 September, 2006
MEMORANDUM FOR CENWP-OP, ATTN: Randy Bailey

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 37 of 2006, which covers the period from 10 through 16 September 2006.
1 OPERATION SUMMARY

1.1 Daily average river flows ranged from 84.5 to 110.9 kcfs.  Daily average project tailwater elevation ranged from 7.9’ to 10.3’ msl.  Daily average powerhouse forebay elevation ranged from 75.9’ to 76.4’ msl.  Daily average river temperature ranged between 67( and 70(F.  Secchi disk measurements averaged 6.5 to 7.0 feet.

1.2 Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-Branch respectively, are included in the spill pattern.  Daily average spill ranged from 1.4 to 1.5 kcfs.  
1.3 Unit Operation.

1.3.1 Unit 0 remains out of service due to governor trouble.

1.3.2 Unit 2 remains out of service for turbine rehab.
1.3.3 Unit 3 returned to service at 1645 on 11 September.

1.3.4 Unit 7 was operated at the top of 1% to flush fish, in preparation for Unit 8 going out of service, on 11 September.

1.3.5 Unit 8 went out of service at 0750 on 12 September for turbine rehab.  

1.3.6 Unit 9 was operated at the top of 1% to flush fish, in preparation for Unit 8 going out of service, on 11 September.
1.3.7 Unit 10 remains out of service for turbine rehab.
1.3.8 Unit 13 remains out of service for a four year overhaul.

1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris. 

1.5 Gatewell drawdowns are measured on Wednesdays; VBS drawdowns are measured daily on modified units.
2 ACTIVITIES

2.1 AWS Closures.  No AWS valves were closed.
2.2 STS/VBS Inspections.  The following STSs/VBSs were inspected this reporting week.
	Unit
	Date
	Run Hours
	Comments

	11-18
	NA
	NA
	NA


Table 1.  STS and VBS inspections.

2.3 Dewatering and Fish Salvage.  The following dewatering activities occurred this reporting week.
2.3.1 Unit 3 tail logs were pulled on 11 September.  Three bluegill and six sculpin were released to the tailrace.
2.3.2 Unit 8 draft tube was dewatered on 13 September.  Seven catfish were removed and released to the river.
3 RESEARCH
3.1 CRITFC.  Sampling adult fish in the AFF.  

3.2 ERDC TDG.  Sensors remain installed in the tailrace.

3.3 NOAA Fisheries- Gilbreath.  Salt water rearing subyearling chinook at the SMF continues.   
3.4 NOAA Fisheries- Jacobson.   Subyearling chinook collections concluded.  
3.5 PSMFC smolt monitoring.  Fallbacks observed: three salmon and one steelhead.
3.6 USGS lamprey.  Lamprey held in the FERL for tagging evaluation.
3.7 WDFW Fish Counting.  Visual fish counting continues.

3.8 WDFW and ODFW.  Tule counting will continue from 0700 until 1600 through the end of September.
4 FISHWAYS 

4.1 Project biologists inspected 10 through 16 September 2006.  NOAA Fisheries inspected on 15 September.
4.2  Fish Passage Plan Observations.
	DATE
	LOCATion
	FPP Deviation
	CAUSE
	RESPONSE
	TR#

	9/10-11
	A-branch
	Reading <1.0’>
	Unknown
	
	

	9/10-11
	B-branch
	Reading <1.0’>
	Unknown
	
	

	9/10-11
	Unit 3
	Fish held <96 hrs
	See 7.4
	See 7.4
	

	9/13
	FG3-27
	Diffuser open
	System in manual
	Operators closed.
	

	9/14
	FG6-14
	Diffuser open
	System in manual
	Operator closed.
	

	9/14
	Weir 37 
	Reading <1.0’>
	Unknown
	Operators adjusted
	

	9/16
	FV1-1
	Not in service
	Oil leak
	Mechanics fixing leak.
	

	9/16
	CI and B-branch entrance  gates
	Not in correct positions.
	System in manual
	Operators adjusted.
	


Table 2.  FPP Out of Criteria items.

4.2.1 PH1 electronic velocity meter is in an eddy.  Due to this placement, the velocity meter reads less than 1.5 fps.  Readings to meet FPP criteria are taken from the south end mechanical meter.
4.2.2 PH2 electronic velocity meter was incorrectly installed in the winter of 04/05.

4.2.3 Cascades Island FG6-12 is closed due to a bent shaft.  Repairs can not be made until winter 06/07.  this diffuser is out of criteria when tailwater exceeds 13’
4.2.4 PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Diffuser failure occurred during water up.  Repairs are scheduled for winter 07/08.  For now, the collection channel is operating within FPP criteria.

4.2.5 DSM2 airburst screens 1, 4, 5, 11 and 12 do not operate consistently.  A portion of the PLC has been reprogrammed.  TR#06-40863.
4.2.6 DSM2 orifice F2 B-S has separated from the shaft.  The orifice is ready for welding.  TR# 06-50979.
4.3 Fishway Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.  
	Date
	WS avg.
	BI avg.
	AFF max
	SMF max

	9/10
	69.2
	69.1
	69.2
	68.2

	9/11
	69.3
	69.1
	69.3
	68.4

	9/12
	69.2
	68.9
	69.2
	68.2

	9/13
	68.9
	68.7
	68.8
	67.9

	9/14
	67.8
	67.7
	67.8
	66.9

	9/15
	67.4
	67.3
	67.4
	66.6

	9/16
	67.3
	67.2
	67.3
	66.5


Table 3.  Bonneville Fishway temperatures.
4.4 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.  

4.5 Adult Fishways. 
4.5.1 Calibration is checked on the weekends.  B-branch tailwater is out of calibration.
4.5.2 Cascades Island and B-Branch PLCs are in manual.  
4.5.3 Stab plates remain installed in the PH2 FOGs.
4.5.4 FV3-9 was in manual from 07 September until 09 September.  BI water elevations are running high.  The electricians will look at the valve on 11 September.
4.5.5 Fish passage numbers reached 25,000 on 09 September.  Flows were split between the two powerhouses on 10 September.  Due to fish numbers dropping below 20,000 fish on 12 September, flows are no longer split.
4.6 Juvenile Fishways.
4.6.1 DSM1, with regional concurrence, will remain OOS.  Screens will not be installed on 15 September.

4.6.2 DSM2 orifice F2 B-S has separated from the shaft.  The orifice is in local and closed.  F2 B-N is open to compensate.

5 CONSTRUCTION
5.1 B1 ITS Wall Removal.  Plans are to remove the wall, slope the floor, and install three automated gates.
5.2 PH1 STS/VBS removal and TIES removal.  Six TIES and 32 PH1 STSs/VBSs will be scrapped.

5.3 PH2 VBS mods.  Unit 11 will go OOS in early Oct, followed by units 15 and 16.

5.4 SMF Full flow PIT Tag Detectors.  System running very well.
6 HAZMAT, SPILLS AND CLEANUP - Nothing to report.
7 TELETYPES AND OTHER NOTEWORTHY ITEMS. 
7.1 A teletype sent on 28 June requests that Bonneville start summer spill at or above 75 KCFS during daytime hours and spill to the 120% gas cap at night until further notice.
7.2 On 06 September, Unit 3 was ready for dewatering and fish salvage.  Due to a drain valve problem, fish could not be salvaged.  On 08 September, it was determined that the valve could not be repaired.  Tail logs will be pulled on the morning of 11 September.

7.3 A teletype sent on 15 September modifies the dates requested for Bonneville pool be operated with a hard constraint 75.0 -76.5 feet, soft constraint 75.5 – 76.5 feet to reduce wear and tear on tribal nets. 



0600 on 11 SEP – 1800 on 15 SEP



0600 on 18 SEP – 1800 on 22 SEP

JAMES R. MAHAR, P.E.

Operations Manager

1

