CENWP-OP-B                                                                                                                          26 February 2007
MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist
SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 08 of 2007, which covers the period from 18 February through 24 February 2007.
1 OPERATION SUMMARY

1.1 Daily average river flows ranged from 124.6 to 162.4 kcfs.  Daily average project tailwater elevation ranged from 13.2’ to 15.5’ msl.  Daily average powerhouse forebay elevation ranged from 75.6’ to 75.9’ msl.  Daily average river temperature ranged between 40(F and 41(F.  Secchi disk measurements averaged 3.0’ to 6.0’.

1.2 Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-Branch respectively, are included in the spill pattern.  Daily average spill averaged 0.6 to 0.7 kcfs.  
1.2.1 Spill Bay 1 remains closed during all hours.  

1.2.2 Spill Bay 18 is open during daylight hours.
1.3 Unit Operation.  First Powerhouse priority.
1.3.1 Unit 2 remains out of service for turbine rehab.
1.3.2 Unit 8 remains out of service for turbine rehab. 

1.3.3 Unit 10 remains out of service for turbine rehab.
1.3.4 Unit 11 was out of service from 0710 until 0842 on 20 February for trashrack raking.

1.3.5 Unit 12 was out of service from 0849 until 1020 on 20 February for trashrack raking; from 1154 until 1600 on 21 February and 1358 until 1637 on 22 February for STS installation.
1.3.6 Unit 13 remains out of service for a four year overhaul.
1.3.7 Unit 14 was out of service from 1025 until 1112 on 20 February for trashrack raking and from 1319 until 1442 on 22 February for STS installation.

1.3.8 Unit 15 remains out of service, as of 08 February, to facilitate VBS/gatewell modifications.  
1.3.9 Unit 16 was out of service from 1115 until 1143 on 20 February and from 0637 until 1316 on 22 February for STS installation.

1.3.10 Unit 17 was out of service from 1231 until 1309 on 20 February and from 0823 until 1149 on 21 February for STS installation.

1.3.11 Unit 18 was out of service from 1313 until 1616 on 20 February and from 0640 until 0819 on 21 February for STS installation.
1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris. 
1.4.1 F1 returned to service at 1521 on 21 February.

1.4.2 F2 returned to service at 0939 on 21 February.

1.5 Gatewell drawdowns are measured on Wednesdays; VBS drawdowns are measured daily on modified units.
2 ACTIVITIES

2.1 AWS Closures.  The following AWS valves are closed.
2.1.1 FV3-9 was closed from 0953 until 1200 on 22 February to facilitate the trashrack installation.
2.1.2 FV5-3, FV5-4, and FV5-9 remain closed for winter maintenance in the Cascades Island fishway.

2.1.3 FV6-9 remains closed due to the UMT remaining OOS for VBS modification.
2.2 STS/VBS Inspections.  STSs/VBSs have been removed for winter maintenance.  
Table 1.  STS and VBS inspections.

	Unit
	Date
	Run Hours
	Comments

	11
	
	
	Waiting for new VBS delivery

	12
	2/21
	
	Screens installed

	13
	
	
	Currently out of service

	14
	2/22
	
	Screens installed

	15
	
	
	Currently out of service for gatewell mods.

	16
	2/22
	
	Screens installed

	17
	2/21
	
	Screens installed

	18
	2/21
	
	Screens installed.


2.3 Dewatering and Fish Salvage.  No dewatering or fish salvage activities occurred this week.
3 RESEARCH
3.1 Battelle.  Will be monitoring with hydroacoustics near B2CC entrance to study kelt passage.  
3.2 ERDC TDG.  Sensors remain installed in the tailrace.

3.3 Fish Field Unit.  Pinniped observations continue.

3.4 NOAA Lamprey.  Construction of the lamprey ramp in the WA shore AWS began in mid January.
3.5 USGS.  Lamprey held in the FERL for tagging evaluation.
3.6 WDFW Fish Counting.  Video counting continues at Bradford Island.
4 FISHWAYS 

4.1 Project biologists inspected 20 through 22 February 2007.
4.2  Fish Passage Plan Observations.
Table 2.  FPP Out of Criteria items.

	DATE
	LOCATion
	FPP itEMS
	CAUSE
	RESPONSE
	TR#

	BRADFORD ISLAND AND BRANCHES

	2/20
	A-Branch
	FG3-4 
	Diffuser not controlling properly
	Operators notified.  TR’d Jan 31st
	

	2/21
	B-Branch
	Differential > 1.5’
	System in auto
	Returned to manual control
	


4.2.1 PH1 electronic velocity meter is in an eddy.  Due to this placement, the velocity meter reads less than 1.5 fps.  Readings to meet FPP criteria are taken from the south end mechanical meter.

4.2.2 PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Repairs scheduled for 07/08.  

4.2.3 FG3-3 stuck partially open.  Repairs are scheduled for winter 07/08.  TR#07-2660

4.3 Fishway Temperatures.  Temperatures are taken 01 March through 30 November.
4.4 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.  

4.5 Adult Fishways. 
4.5.1 Calibration is checked on the weekends.  B-branch tailwater is out of calibration.
4.5.2 B-Branch PLC is in manual.  
4.5.3 Washington shore returned to service on 22 February.
4.5.4 Cascades Island remains out of service.
4.5.5 SLEDS are installed at PH1, PH2, and B-branch fishway entrances.  

4.6 Juvenile Fishways.
4.6.1 DSM1 will continue to remain OOS until removal in winter 07/08.
4.7 Lamprey Fishways.  Remains dewatered.
5 CONSTRUCTION
5.1 B1 ITS Wall Removal.  Plans are to remove the wall, slope the floor, and install three automated gates.
5.2 PH1 STS/VBS removal and TIES removal.  Six TIES and 24 PH1 STSs/VBSs will be scrapped.

5.3 PH2 VBS mods.  Units 11 and 16 are complete.  Unit 15 is currently OOS.
6 HAZMAT, SPILLS AND CLEANUP – nothing to report.
7 TELETYPES AND OTHER NOTEWORTHY ITEMS
7.1 A teletype sent on 22 December details tailwater operations for chum incubation.   Effective as soon as possible, a hard tailwater constraint to operate project tailwater no lower than 13.0 feet during all hours.  One pulse per day with a minimum duration of 20 minutes and minimum elevation of 13.5 feet.  
7.2 A teletype sent on 14 February outlines operations necessary for the project to complete a spillway operation/calibration test as part of a regular maintenance regime.  During hours of 0700 – 1500, Monday, 19 February, open one spillbay at a time as much as one foot.  Maximum instantaneous spill levels on tested bays will be 2.3 kcfs with an average spill of 1.5 kcfs through the period (in addition to normal spillbay 18 spill).  Spillbay 18 will remain open (normal operations per the Fish Passage Plan) while other bays are tested.

JAMES R. MAHAR, P.E.

Operations Manager

1

