CENWP-OP-B                                                                                                                          16 July 2007
MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist
SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 28 of 2007, which covers the period from 08 through 14 July 2007.
1 OPERATION SUMMARY

1.1 Daily average river flows ranged from 165.9 to 208.8 kcfs.  Daily average project tailwater elevation ranged from 14.4’ to 17.3’ msl.  Daily average powerhouse forebay elevation ranged from 73.9’ to 74.7’ msl.  Daily average river temperature ranged from 67 to 70(F.  Secchi disk measurements averaged 5.0’ to 7.0’.
1.2 Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-Branch respectively, are included in the spill pattern.  Daily average spill ranged from 88.8 to 95.2 kcfs.  
1.3 Unit Operation. Powerhouse 2 is the priority powerhouse.  
1.3.1 Unit 5 has remained out of service for annual maintenance since 11 June.
1.3.2 Unit 7 has remained out of service for turbine rehab since 04 June 2007.
1.3.3 Unit 8 has remained out of service for turbine rehab since 12 September 2006.
1.3.4 Unit 13 was forced out of service at 1700 hours on 9 July for a suspected oil leak.  The leak was restricted to the turbine shaft as designed.  No oil entered the river.  It returned to service at 1725 on 11 July.
1.3.5 Unit 15 was taken out of service at 0122 hours on 9 July for exciter replacement.  
1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris. 

1.4.1 F1  and F2 had reduced loading for lamprey passage studies from:

2200 on 08 July until 0400 on 09 July
2200 on 10 July until 0400 on 11 July
2200 on 11 July until 0400 on 12 July
2200 on 12 July until 0400 on 13 July
2200 on 14 July until 0400 on 15 July
1.5 Gatewell drawdowns are measured on Wednesdays; VBS drawdowns are measured daily on modified units.
2 ACTIVITIES

2.1 AWS Closures.  No AWS valves were closed this week.
2.2 STS/VBS Inspections.  No STS/VBS inspections occurred this week. 
Table 1.  STS and VBS inspections.  
	Unit
	Date
	Run Hours
	Comments

	11-18
	NA
	NA
	NA


2.3 Dewatering and Fish Salvage.  No dewatering and fish salvage activities occurred this week.
3 RESEARCH  
3.1 Battelle spillway survival.  Releasing yearling chinook in the main dam forebay to evaluate survival.
3.2 CRITFC.  Sampling chinook, coho, steelhead and sockeye throughout the season. 
3.3 CTUIR- lamprey.  Will receive AFF trapped lamprey from NOAA researchers for broodstock purposes.
3.4 CTWS- lamprey.  Will receive AFF trapped lamprey from NOAA researchers for ceremonial purposes.
3.5 NOAA-Gilbreath.  Salt water rearing at the Smolt Monitoring Facility.

3.6 NOAA- Ferguson.  B2CC detection evaluation underway.  

3.7 NOAA- McComas. Tagging juvenile yearling chinook at the Smolt Monitoring Facility. 

3.8 NOAA Lamprey.  Tagging of lamprey for LPS and UI continue.  
3.9 PSMFC Smolt Monitoring.  No fallback over the separator bars.  2,004 subyearling chinook were collected for NOAA and Battelle research.  One largescale sucker was collected for Bradford Island Remedial Investigations.  60 adult shad were held for USGS research.   http://www.fpc.org/CurrentDaily/Gbtsum.txt has GBT results.
3.10 University of Idaho- lamprey.  Block study for lamprey entrance evaluations at WS continue.
3.11 University of Idaho- salmon.  Sampling for S. Fork Salmon chinook continues. 
3.12 University of Idaho-shad.  Sampling for shad continues.

3.13 USDA Wildlife Services.  Avian hazing began on 07 May.
3.14 USGS.  Collected 60 shad from the AFF on 10 July.
3.15 WDFW Fish Counting.  Visual counting continues Bradford Island and Washington Shore
4 FISHWAYS 

4.1 Project biologists inspected daily from 08 through 14 July.  
4.2  Fish Passage Plan Observations.
Table 2.  FPP Out of Criteria items.

	DATE
	LOCATion
	FPP itEMS
	CAUSE
	RESPONSE
	TR#

	BRADFORD ISLAND AND BRANCHES

	
	
	
	
	
	

	7/9
	B-Branch
	FG3-25 closed
	System in manual
	Operators opened the diffuser.
	

	7/11
	B-Branch
	FG3-25 opened
	System in manual
	Operators closed diffusers.
	

	CASCADES ISLAND

	All week
	FG6-14
	½ way open
	Electricians need to do some rewiring.
	Electricians and riggers working out scheduling.
	

	7/9
	FG6-12
	Diffuser closed at incorrect TW
	System in manual
	Operators opened the diffuser.
	

	WASHINGTON SHORE

	All week
	DSM2
	Mech. Screen cleaners #1, #2 in manual
	Broken.
	Waiting on mechanical parts.
	

	All week
	DSM2
	Air burst screens 3, 6 and 9 not working.
	Bad controls.
	Will replace during winter maintenance.
	

	7/12, 7/14
	Weir 38
	Reading <1.3’
	Unknown
	Operator adjusted
	


4.2.1 PH1 electronic velocity meter is in an eddy.  Due to this placement, the velocity meter reads less than 1.5 fps.  Readings to meet FPP criteria are taken from the south end mechanical meter.

4.2.2 PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Repairs scheduled for 07/08.  

4.2.3 FG3-3 stuck partially open.  Repairs are scheduled for winter 07/08.  TR#07-2660

4.2.4 FG3-5 has a damaged shaft and the valve won’t operate.  Repairs are scheduled for winter 07/08.  

4.2.5 FG3-6 stuck partially open.  Repairs are scheduled for winter 07/08.  

4.2.6 FG6-10 closed.  Only out of criteria when tailwater is above 19.0’ msl.  
4.3 Fishway Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore. Temperatures are in Fahrenheit.
Table 3.  Bonneville Fishway Temperatures.

	Date
	WS average
	BI average
	AFF average
	JMF average

	7/08
	67.7
	67.6
	67.7
	66.7

	7/09
	67.9
	67.8
	68.0
	66.8

	7/10
	68.7
	68.5
	68.8
	67.8

	7/11
	69.4
	69.2
	69.4
	68.4

	7/12
	70.1
	69.9
	70.1
	69.0

	7/13
	70.4
	70.1
	70.4
	69.3

	7/14
	70.8
	70.6
	70.9
	69.9


4.4 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.  

4.5 Adult Fishways. 
4.5.1 Calibration was checked on 14 July.  
4.5.2 B-Branch and Cascades Island are in manual.
4.5.3 FV3-7 is in manual until the A-Branch diffusers are repaired and operating correctly.
4.5.4 Fish ladders remain in shad passage mode.
4.5.5 FG3-4 returned to service on 02 July.
4.6 Juvenile Fishways.
4.6.1 PH1 chaingates 1A and 1B remain closed at this time.
4.6.2 1C, 3C, and 6C remain set at 70’ msl.
4.6.3 DSM1 remains OOS.
4.6.4 DSM2 has normal water levels this reporting week.  Orifices and add-in are back to auto. 

4.7 Lamprey Fishways.  
4.7.1 The Bradford Island lamprey ramp remains in service.
4.7.2 Washington Shore NDE lamprey ramp was put into service on 29 May.
4.7.3 The Washington shore FV6-9 lamprey ramp was put into service on 18 June.
5 CONSTRUCTION
5.1 B1 ITS Wall Removal.  Plans are to remove the wall, slope the floor, and install three automated gates.
5.2 PH1 STS/VBS removal and TIES removal.  Six TIES and 23PH1 STSs/VBSs will be scrapped.

5.3 PH2 VBS mods.  Units 14 and 18 are scheduled for winter 07/08.  U17 VBS monitoring equipment will be installed at this time as well.
6 HAZMAT, SPILLS AND CLEANUP.   Nothing to report this week.
7 TELETYPES AND OTHER NOTEWORTHY ITEMS
7.1 A teletype sent on 21 May requests that the Washington shore adult ladders be reduced to 0.5 feet head from 2200 until 0400 hours for a lamprey test running from 2 June until 1 October.  
7.2 A teletype sent on 07 June requests that all Lower Snake and Columbia River FCRPS projects operate all turbine units within the 1% best efficiency standard during the best efficiency operating period (April 1 through October 31).  Operation outside of 1% is allowed for maintenance, start up/shut down and other instances as outlined in the Fish Passage Plan.  Operation outside of 1% is NOT allowed for any other reason unless a power emergency has been declared.
7.3 A teletype sent on 20 June details the start of summer spill for juvenile fish passage.  

7.3.1 From 0001 on 21 June until 2400 on 15 July, Bonneville will spill 85K during daytime hours using the spill pattern provided by Dennis Schwartz, labeled “Bonneville Spill Patterns- 50-100kcfs”.  Bonneville will spill to the gas cap at night.  Refer to the 2007 FPP, page BON-14 Table BON-6 for daytime hours.

7.3.2 From 0001 on 16 July until 2400 on 31 August, Bonneville will spill 75K during daytime hours, using the spill pattern provided by Dennis Schwartz, and to the gas cap at night.

7.3.3 If it is necessary to spill above 110kcfs, the Control Room will use the spill pattern found in the 1007 FPP page Bon-51, Table Bon-15.  
7.3.4 At no time will the Project voluntarily spill above the 120% gas cap.  The 120% spill cap level for Bonneville can be found in the teletype “SPILL PRIORITY LIST”. 

7.3.5 At all times, maintain a minimum generation of 30kcfs and minimum spill of 50kcfs.
      7.4   A teletype sent on 14 July provides guidance on project operation to provide voluntary and involuntary  

              spill.  The teletype provided spill order by project, as well as spill caps for various TDG levels.   

JAMES R. MAHAR, P.E.

Operations Manager
1

