CENWP-OP-B                                                                                                                         August 04, 2008
MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist
SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 31 of 2008, which covers the period from 27 July through 02 August 2008.
1 OPERATION SUMMARY

1.1 Daily average river flows ranged from 138.0 to 177.2 kcfs.  Daily average project tailwater ranged from 12.8’ to 15.2’ msl.  Daily average powerhouse forebay elevation ranged from 74.7’ to 75.8’ msl.  Daily average river temperature ranged from 68°F to 69°F.  Secchi disk measurements ranged from 6.0’ to 7.0’.
1.2 Spill Bays 1 and 18, which supply attraction flow for Cascades Island and B-branch, respectively, are included in the spill pattern.  Daily average spill ranged from 90.3 to 92.1 kcfs.
1.2.1 Bay 15 was placed in auto at 1338 on 24 July.  Bay 18 is the only bay that remains dogged off at 2.9’, where it will remain while the hoist is being repaired.
1.3 Unit Operation.  Powerhouse 2 is the priority powerhouse.
1.3.1 Unit 1 went out of service at 0804 on 07 July for five year overhaul.

1.3.2 Unit 7 has remained out of service since 04 June 2007 for turbine rehab.

1.3.3 Unit 8 was run periodically for testing from 1010 until 1456 on 28 July.  A 72 hour heat run was conducted from 0810 on 29 July until 0820 on 01 August.  Returned to service at 1015 on 01 August.  
1.3.4 Unit 11 has remained out of service since 0238 on 12 June due to rotor cracks.
1.3.5 Unit 13 was out of service from 1638 until 1749 on 31 July due to excessive shaft packing leakage.

1.3.6 Unit 15 was out of service from 0559 until 1046 on 31 July for STS repairs.
1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris.
1.5 Gatewell drawdowns are measured on Wednesdays.
2 ACTIVITIES

2.1 AWS Closures.  The following AWS closures occurred this reporting week.
2.1.1 FV 6-9 was closed for cleaning from 1620 until 1627 on 30 July.
2.2 STS/VBS Inspections.  The following STS/VBS video inspections occurred this week:  

Table 1.  STS and VBS inspections.  

	Unit
	Date
	Run Hours
	Comments

	11
	7/31
	95
	Out of service

	12
	7/31
	970
	OK

	13
	7/31
	663
	OK

	14
	7/31
	919
	OK

	15
	7/31
	964
	15-C - Shaft on south side bolts broken, south belts not rotating.  Replaced STS with spare.

	16
	7/31
	658
	OK

	17
	7/31
	856
	OK

	18
	7/31
	982
	OK


2.3 Dewatering and Fish Salvage. No dewatering or fish salvage activities occurred this reporting week.
3 RESEARCH  
3.1 Battelle.  BGS hydrophones are installed and are now functional.  
3.2 Battelle.  Spillway survival evaluation continues.
3.3 CRITFC.  Annual chinook, coho, sockeye and steelhead sampling in the AFF continues.  One summer chinook sample mortality on 29 July.  For more information, please contact project fisheries for MFR sent on 01 Aug.  
3.4 CTUIR.  Received 222 lamprey to date collected by NOAA at the Adult Fish Facility (AFF) for restoring lamprey to the Umatilla River.  
3.5 NOAA – Gilbreath.  Saltwater tag effects study continues.
3.6 NOAA – Jacobson.  Collected 25 subyearling chinook from PSMFC for pathogen testing.   
3.7 NOAA – Moser.  Trapping and PIT tagging at the AFF continues.  
3.8 PSMFC Smolt Monitoring.   Condition, index, and gas bubble trauma monitoring at the SMF.  Please contact deanb@gorge.net for results.  Zero adult salmonids were recorded crossing the separator bars.  25 subyearling chinook were collected for NOAA and 27 juvenile shad for USGS.
3.9 University of Idaho.  Combining daytime, estimated nighttime, LPS counts and detections from radio and HD PIT sites to determine the best methods to estimate system wide passage of adult Pacific lamprey.  Working in the experimental flume at the AFF on lamprey passage improvements.
3.10 USDA Wildlife Services. Avian hazing continues.
3.11 USGS.  47 juvenile shad were collected this week for the Impacts of American Shad on the Columbia River study.  
3.12 WDFW Fish Counting.  Visual counts at Bradford Island and WA Shore continue.
4 FISHWAYS 

4.1 Project biologists inspected on 27 July -02 August.  
4.2  Fish Passage Plan Observations.
Table 2.  FPP Out of Criteria items.

	DATE
	LOCATion
	FPP itEMS
	CAUSE
	RESPONSE
	TR#

	BRADFORD ISLAND

	7/31-8/2
	A branch
	Staff gauge <1.3
	Please see 4.5.4
	
	

	7/31-8/2
	B branch 
	Staff gauge <1.3
	Please see 4.5.4
	
	

	7/31-8/2  
	BI weir
	BI weir <1.2
	Please see 4.5.4
	
	

	CASCADES ISLAND

	7/31-8/2
	Cascades Is. weir
	UMT staff gauge <1.3
	Please see 4.5.4
	
	

	WASHINGTON SHORE-nothing to report

	
	
	
	
	
	


4.2.1 FG6-10 closed.  The motor was repaired on 15 August 2007.  The shaft and gate have separated and the diffuser will not be operational until repaired in winter 09/10.  It is OOC when TW is above 19.0’ msl.
4.2.2 PH1 elec. velocity meter is in an eddy.  Readings for FPP criteria are taken at the south end mechanical meter.

4.2.3 PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Repairs scheduled for 09/10.  

4.2.4 FG3-5 closed.  The shaft and gate have separated.  Repairs scheduled for winter 09/10.  It’s OOC at TW 16.7’ msl and higher.

4.2.5 PH2 velocity meter.  The velocity meter is believed to be broken.  Repairs will not be made until winter 08/09.

4.3 Fishway Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore. Temperatures are in Fahrenheit.

4.3.1 Bradford Island temperatures are not available at this time.  
Table 3.  Bonneville Fishway Temperatures

	Date
	WS average
	AFF
	SMF

	7/27
	68.5
	68.5
	67.6

	7/28
	68.9
	69.0
	68.0

	7/29
	68.8
	68.9
	67.9

	7/30
	68.6
	68.8
	67.7

	7/31
	68.6
	68.6
	67.5

	8/1
	68.1
	68.2
	67.3

	8/2
	68.0
	68.2
	67.3


4.4 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.  

4.5 Adult Fishways. 
4.5.1 Calibration was not checked this week.
4.5.2 B-Branch and Cascades Island fishway PLC are in manual.
4.5.3 Removal of the FOG sea lion exclusion bars has not yet begun due to a broken lifting beam.  The four large orifices have bars (two at each end of the powerhouse). The stab plates remain installed in gates without exclusion bars.  
4.5.4 Shad passage mode concluded early, on 31 July, due to low numbers of shad passing BON.  Due to an oversight, this action had not been coordinated with FPOM and is thus recorded as out of criteria.
4.5.5 Cascades Island exit is open as of 0840 on 11 July, to provide an exit for sockeye trapped upstream of the UMT picket leads.
4.6 Juvenile Fishways.

4.6.1
   B2CC is scheduled to close on 02 September, as agreed to at the June FPOM meeting.

4.7 Lamprey Fishways.  
4.7.1 WS and BI AWS lamprey ramps remain in service.
4.7.2 The WS NDE lamprey ramp remains in service.
5 CONSTRUCTION
5.1 PH1 ITS Wall Removal.  Three automated gates to be installed.  May slope the floor and remove the wall.
5.2 TIE crane.  The TIE crane boom repair is on hold until funding is secured.

6 HAZMAT, SPILLS AND CLEANUP.   Nothing to report this week.
7 TELETYPES AND OTHER NOTEWORTHY ITEMS
7.1 A teletype sent on 19 June details the start of summer spill for juvenile fish passage.  It supercedes the teletype sent on 10 April. 

7.1.1 From 0001 on 21 June until 2400 on 20 July, Bonneville will spill 85K during daytime hours and to the gas cap at night.  
7.1.2 From 0001 on 21 July until 2400 on 31 August, Bonneville will spill 75K during daytime hours and to the 120% gas cap at night.

7.1.3 The spill pattern is found in section 2, Table BON-15 of the 2008 FPP, and daytime hours are defined in section 2, Table BON-5 of the 2008 FPP.  www.nwd-wc.usace.army.mil/tmt/documents/fpp/
7.1.4 At no time will the Project voluntarily spill above the 120% gas cap.  The 120% spill cap level for Bonneville can be found in the teletype “SPILL PRIORITY LIST”. 

7.1.5 At all times, maintain a minimum generation of 30kcfs and minimum spill of 50kcfs.

7.2 A teletype sent on 23 June authorizes the Project to dog open Spillbays 11, 14, and 18 at 2 stops (approx. 2.9’).  Spill shall be maintained as specified in 7.1.  This is to facilitate repairs to the Spillbay 15 hoist.  
7.3 A teletype sent on 17 July authorizes the Project to deviate from unit priority, as stated in the Table 11 of the 2008 FPP, and operate the Unit 8 outside the 1% range as necessary, beginning 18 July to approximately 31 July.  It is requested that operation outside of the 1% be kept to a minimum.  This is necessary to facilitate startup and commissioning testing for Unit 8 with the new minimum gap runner.

7.4 A teletype sent on 24 July requests the Project operate Bonneville Pool with soft constraint of 1.0’ operating range, and a hard constraint operating range of 1.5’ from 0600 on 28 July until 1800 hours on 31 July.  This operation is requested for treaty gillnet fishing.  A 29 July teletype provides the order of precedence in the event treaty fishing and spill constraints are not achievable.  
7.5 Several Memos for the Record regarding fish mortalities were sent out this reporting week.  If you need copies of the 080714 CI, 080714 WS AWS sockeye, 080722 AFF lamprey mort, 080723 AFF steelhead mort, 080801 AFF CRITFC mort.  Please contact BON Project Fisheries for additional copies.

JAMES R. MAHAR, P.E.

Operations Manager
1

